3GPP TSG-RAN4 Meeting #89 	R4-1816409

# Spokane, USA, 12 - 16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0048 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Intra-frequency Relative SS RSRP Accuracy for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-10-31 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The FR1 and FR2 Intra-frequency Relative SS RSRP accuracy requirements only apply to measurements of another cell, and leave the accuracy of relative measurements on the same cell unspecified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include measurements between any two SS RSRP levels measured on the same cell in the FR1, FR2 Intra-frequency Relative SS RSRP accuracy requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of UE measurement accuracy is unspecified, making it difficult to design some test cases. Related RAN5 test cases cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.1.2, 10.1.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The baseline TS 38.133 v15.3.0 used for this CR does not include any changes agreed at RAN4#88bis, but the changes here do not create any conflict. |  |  |  |  |  |  |  |  |

### << Unchanged sections omitted >>

### 10.1.2	Intra-frequency RSRP accuracy requirements for FR1

#### 10.1.2.1	Intra-frequency SS RSRP accuracy requirements

##### 10.1.2.1.1	Absolute SS RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS RSRP in this clause apply to a cell on the same frequency as that of the serving cell.

The accuracy requirements in Table 10.1.2.1.1-1 are valid under the following conditions:

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB,

-	Other conditions are TBD.

Table 10.1.2.1.1-1: SS RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [4.5] | [9] | [-6] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -70 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -70 |
| [8] | [11] | [-6] dB | All | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. |  |  |  |  |  |  |  |

Editor’s note: the minimum Io conditions in this table are subjected to the SSB SCS used and they are calculated according to system noise PN and received minimum RSRP requirement.

##### 10.1.2.1.2	Relative SS RSRP Accuracy

The relative accuracy of SS RSRP is defined as the SS RSRP measured from one cell compared to the SS RSRP measured from another cell on the same frequency, or between any two SS RSRP levels measured on the same cell.

The accuracy requirements in Table 10.1.2.1.2-1 are valid under the following conditions:

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB,

-	Other conditions are TBD.

Table 10.1.2.1.2-1: SS RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [2] | [3] | [-3] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -50 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -50 |
| [3] | [3] | [-6] dB | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

Editor’s note: the minimum Io conditions in this table are subjected to the SSB SCS used and they are calculated according to system noise PN and received minimum RSRP requirement.

### << Unchanged sections omitted >>

### 10.1.3	Intra-frequency RSRP accuracy requirements for FR2

#### 10.1.3.1	Intra-frequency SS RSRP accuracy requirements

##### 10.1.3.1.1	Absolute SS RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS RSRP in this clause apply to a cell on the same frequency as that of the serving cell.

The accuracy requirements in Table 10.1.3.1.1-1 are valid under the following conditions:

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB,

-	Other conditions are TBD.

Table 10.1.3.1.1-1: SS RSRP Intra frequency absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm/120kHz SSB SCS | dBm/240kHz SSB SCS | dBm/BWChannel | dBm/BWChannel |
| [6] | [9] | TBD | TBD | TBD | TBD | TBD | TBD |
|  |  |  | TBD | TBD | TBD | TBD | TBD |
|  |  |  | TBD | TBD | TBD | TBD | TBD |
|  |  |  | TBD | TBD | TBD | TBD | TBD |
| [8] | [11] | TBD | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. |  |  |  |  |  |  |  |

##### 10.1.3.1.2	Relative SS RSRP Accuracy

The relative accuracy of SS RSRP is defined as the SS RSRP measured from one cell compared to the SS RSRP measured from another cell on the same frequency, or between any two SS RSRP levels measured on the same cell.

The accuracy requirements in Table 10.1.3.1.2-1 are valid under the following conditions:

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB,

-	Other conditions are TBD.

Table 10.1.3.1.2-1: SS RSRP Intra frequency relative accuracy

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm/120kHz SSB SCS | dBm/240kHz SSB SCS | dBm/BWChannel | dBm/BWChannel |
| [6] | [9] | TBD | TBD | TBD | TBD | TBD | TBD |
|  |  |  | TBD | TBD | TBD | TBD | TBD |
|  |  |  | TBD | TBD | TBD | TBD | TBD |
|  |  |  | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |  |  |

#### 10.1.3.2	Intra-frequency [CSI-RS RSRP] accuracy requirements

### << End of changes >>