3GPP TSG RAN WG4 Meeting #89	R4-1816420

Spokane, US, 12th – 16th Nov, 2018

3GPP TSG-RAN WG2 Meeting #104	R2-1818901

Spokane, USA, 12 - 16 November 2018

Title:	LS on RRC processing delay for BWP switching

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG2

To:	TSG RAN WG4

Cc:

Contact Persons:

#### Name:	Tero Henttonen

E-mail Address:	tero.henttonen@nokia.com

#### Name:	Cong Shi

E-mail Address:	shicong@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has discussed the BWP switching and RRC processing delays for BWP switching and would like to inform RAN4 of the following:

- Concerning the RRC-based BWP switching, RAN2 will only work in RRC processing part of the procedure delay and leave to RAN4 to define BWP switching part of the procedure delay. However, since the RRC processing delay is defined as the time it takes for UE to process the RRC message and be ready to send the corresponding RRC acknowledgment (e.g. RRCReconfigurationComplete), RAN2 will also need to reflect the BWP switching delay in RRC but can do this based on the requirements defined in RAN4 (e.g. via referring to RAN4 specifications).

- RAN2 would note that also the RRC-based BWP switching and MAC-based BWP switching (upon initiation of random access procedure in certain cases) may occur and should be considered in RAN4 requirements. RAN2 would like to understand whether the BWP switching delays defined for DCI-based BWP switching also apply to these cases.

RAN2 respectfully asks RAN4 feedback on the questions above.

2. Actions:

To RAN4 group.

- RAN2 respectfully asks RAN4 to take the RAN2 agreements on RRC processing delay for RRC-based BWP switching into account in their specifications.

- RAN2 respectfully asks RAN4 whether the BWP switching delays defined for DCI-based BWP switching also apply to the cases of RRC signalling based BWP switch and BWP switching by MAC entity upon initiation of Random Access procedure.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#105		25 Feb - 01 Mar 2018		Athens

3GPP RAN2#105bis		08 - 12 Apr 2018			China