3GPP TSG-RAN WG4 Meeting # 89	R4-1816456

# Spokane, WA, USA,  12 - 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: Removal of duplicating subsections about “Summation error” (Section 10). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In subsection 10.5 (Measurement uncertainty for out-of-band TRP requirements), the following subsections 10.5.1, 10.5.2.3, 10.5.3.3.3 and 10.5.4.3 contain similar text. Further, such text is missing in subsection 10.4 (Measurement uncertainty for in-band TRP requirements). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This is a revisions of R4-1815976. Subsections 10.5.1, 10.5.2.3, 10.5.3.3.3 and 10.5.4.3 are removed and the text is moved to subsection 10.8 which already contains a short description of TRP summation error. Reference is made from subsections 10.4 and 10.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current structure can be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4, 10.5 and 10.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

###### 10.4.1.2.4.2	MU Value

Table 10.4.1.2.4.2-1: Indoor Anechoic Chamber uncertainty assessment for EIRP measurement of OTA base station output power

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.44 | 0.54 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.87 | 1.06 |

The agreed summation error (SE) of 0,75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,1 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,3dB

<End of modified section>

<Start of modified section>

###### 10.4.1.3.4.2	MU Value

Table 10.4.1.3.4.2-1: Compact antenna test range uncertainty assessment for OTA Base station output power measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.594 | 0.669 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.16 | 1.31 |

The agreed summation error (SE) of 0,75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,4 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,5dB

<End of modified section>

<Start of modified section>

###### 10.4.2.2.4.2	MU Value

Table 10.4.2.2.4.2-1: In-door anechoic chamber uncertainty assessment for OTA BS ACLR EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | [0.14] | [0.26] | [Gaussian] | [1] | [1] | [0.14] | [0.26] |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.444 | 0.538 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.871 | 1.055 |

The agreed summation error (SE) of 0,75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,1 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,3dB

<End of modified section>

<Start of modified section>

###### 10.4.2.3.4.2	MU Value

Table 10.4.2.3.4.2-1: Compact antenna test range uncertainty assessment for absolute ACLR measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | [0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.069 | 1.349 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.10 | 2.64 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 2.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 2.7dB

<End of modified section>

<Start of modified section>

###### 10.4.3.2.4.2	MU Value

Table 10.4.3.2.4.2-1: In-door anechoic chamber uncertainty assessment for OTA OBUE EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | [0.14] | [0.26] | [Gaussian] | [1] | [1] | [0.14] | [0.26] |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.444 | 0.538 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.871 | 1.055 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1.1 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1.3dB

<End of modified section>

<Start of modified section>

###### 10.4.3.3.4.2	MU Value

Table 10.4.3.3.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA operating band unwanted emissions measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.83 | 0.93 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.63 | 1.83 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 2.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 2.7dB

<End of modified section>

<Start of modified section>

###### 10.4.4.2.4.2	MU Value

Table 10.4.4.2.4.2-1: In-door anechoic chamber uncertainty assessment for OTA SEM EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | [0.14] | [0.26] | [Gaussian] | [1] | [1] | [0.14] | [0.26] |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.493 | 0.560 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.966 | 1.098 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1.3dB

<End of modified section>

<Start of modified section>

###### 10.4.4.3.4.2	MU Value

Table 10.4.4.3.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA SEM

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0,83 | 0,93 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1,63 | 1,83 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 2.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 2.7dB

<End of modified section>

<Start of modified section>

## 10.5	Measurement uncertainty for Out-of-band TRP requirements

### 10.5.1	General

The TRP MU consists of an MU per point and a Summation Error (SE) which allows for errors in the calculation of the TRP from multiple directional power measurements and allows for a sparse grid to be used to reduce measurement time. The total MU is calculated as follows:

Refer to subclause 10.8 for the SE value.

### 10.5.2	Transmitter mandatory spurious emissions

#### 10.5.2.1	General

The conducted spurious emission requirement MU is split up into a number of frequency ranges

| Transmitter spurious emissions, Mandatory Requirements | dB | MU |
| --- | --- | --- |
| 30 MHz ≤ f ≤ 4 GHz |  | 2.0 |
| 4 GHz < f ≤ 19 GHz |  | 4.0 |

The conducted analysis based on UTRA/E-UTRA frequencies which were all below 4.2GHz (at the time), the break point in the MU is hence somewhat related to the in-band and out of band MU analysis. As in-band MU analysis is now being done up to 6GHz (for the LAA and NR bands) it is sensible to change the frequency break point to 6GHz.

The conducted and OTA test set up can be compared as follows:

Figure 10.5.2.1-1:	Conducted and OTA spurious emissions test set up

As the spurious emissions requirements cover a  large frequency span from 30MHz to 12.75GHz it is difficult to use the same measurement chambers as the in band measurements. The CATR for example is not suitable for low frequency measurements.

The chamber analysis is hence done on a general OTA chamber which is capable of all frequency ranges.

Other chambers may be used if their MU is within the final value and they are suitable for the frequencies being tested.

#### 10.5.2.2	General chamber

##### 10.5.2.2.1	General

As the AAS BS antenna radiating dimensions are fixed then the far field distance increases (FF≈2d2/λ). At 12.75 GHz the far field distance for a 1.5m AAS BS antenna array is almost 200m, this is clearly impractical in an indoor chamber (and the path loss would also make measurement difficult), so spurious emission testing will not always be in the far field. This is acceptable as the requirement is TRP and hence it is not necessary to measure in the far field however it needs to be considered when looking at MU.

Considerations of the large frequency range must also be considered, including the chamber performance (quiet zone), the calibration effectiveness and the available reference and test antennas over the frequency range.

##### 10.5.2.2.2	Calibration

Each frequency point can be calibrated using the EIRP calibration approach in sub-clause 10.2.3.2.2 .

The calibration should be repeated for each frequency being tested and each test antenna.

##### 10.5.2.2.3	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 6.7.6.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the AAS BS to transmit according to the applicable test configuration

6)	Align the BS and the test antenna such that measurements to determine TRP can be performed (see subclause 10.8.3)

7)	Measure the emission at the specified frequencies with specified measurement bandwidth

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see subclause 10.8.3) and for frequency points to be tested.

Note 1: the TRP measurement grid may not be the same for all measurement frequencies.

Note 2: the frequency sweep or the TRP measurement grid sweep may be done in any order

9)	Calculate TRP at each specified frequency using the directional measurements.

##### 10.5.2.2.4	MU assessment

###### 10.5.2.2.4.1	MU Budget

Table 10.5.2.2.4.1-1: General  Chamber uncertainty contributions
Tx spurious emissions

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E4-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | E4-2 |
| 3 | Quality of quiet zone | E4-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E4-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E4-5 |
| 6 | Phase curvature | E4-6 |
| 7 | Conducted measurement uncertainty (minus mismatch) | E4-22, F.2 |
| 8 | Impedance mismatch in the receiving chain | E4-8 |
| 9 | Random uncertainty | E4-9 |
| 23 | Measurement antenna frequency variation | E4-20 |
| 24 | FSPL estimation error | E4-21 |
| 25 | Test system frequency flatness | E4-19 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | E4-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E4-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | E4-12 |
| 13 | Quality of quiet zone | E4-3 |
| 14 | Polarization mismatch for reference antenna | E4-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E4-5 |
| 16 | Phase curvature | E4-6 |
| 17 | Uncertainty of the network analyzer | E4-13, F.1 |
| 18 | Influence of the reference antenna feed cable | E4-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E4-15 |
| 20 | Influence of the receiving antenna feed cable | E4-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E4-17 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E4-18 |

###### 10.5.2.2.4.2	MU Value

Table 10.5.2.2.4.2-1: General  Chamber uncertainty assessment
Tx spurious emissions

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |  |  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 7 | Conducted measurement uncertainty (minus mismatch | 1.02 | 2.04 | Gaussian | 1 | 1 | 1.02 | 2.04 |
| 8 | Impedance mismatch in the receiving chain | 0.2 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.32 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 23 | Measurement antenna frequency variation | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 24 | FSPL estimation error | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 |
| 25 | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.2 | Gaussian | 1 | 1 | 0.13 | 0.2 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.5 | Rectangular | √3 | 1 | 0.29 | 0.29 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.12 | 2.11 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.20 | 4.15 |

#### 10.5.2.3	Void

.

#### 10.5.2.4	Test Tolerance

The conduced test tolerance for the mandatory spurious emissions requirements is zero. As the requirements are set by regulatory limits the same test tolerance is used for OTA.

TT=0.

#### 10.5.2.5	Summary

Adding the MUperpoint and the SE (see subclause 10.8) we have:

,	30MHz<f≤6 GHz

,	6GHz<f≤19 GHz

By adding the MU per point and the SE values together we have the following total MU:MUtotal = 2.3dB,	30MHz<f≤6GHz

MUtotal = 4.2dB,	6GHz<f≤19GHz

The test tolerance for mandatory spurious emissions is zero.

### 10.5.3	Receiver spurious emissions

#### 10.5.3.1	General

The conducted receiver spurious emission requirement MU is the same as for the TX spurious emissions, the measurement technique is the same and the power level is >-60dBm (where there is a break point for conducted power measurement accuracy), so this is reasonable.

For the OTA receiver emissions requirements however the lower power level of the requirement reduces the dynamic range of the TRP measurement and reduces measurement accuracy.

Considering that the loss in the chamber is based on the wanted signal being in the far field the per point noise floor is assumed to be approx -100dBm and the receiver emissions level translated to the test equipment is approx -90dBm. Hence the TRP calculation has only a 10dB dynamic range.

An uncertainty of 1dB is added to the TRP uncertainty budget to account for this additional uncertainty.

#### 10.5.3.2	General chamber

##### 10.5.3.2.1	General

As the AAS BS antenna radiating dimensions are fixed then the far field distance increases (FF≈2d2/λ). At 12.75 GHz the far field distance for a 1.5m AAS BS antenna array is almost 200m, this is clearly impractical in an indoor chamber (and the path loss would also make measurement difficult), so spurious emission testing will not always be in the far field. This is acceptable as the requirement is TRP and hence it is not necessary to measure in the far field however it needs to be considered when looking at MU.

Considerations of the large frequency range must also be considered, including the chamber performance (quiet zone), the calibration effectiveness and the available reference and test antennas over the frequency range.

##### 10.5.3.2.2	Calibration

The calibration is the same as described in sub-clause 10.5.2.2.2.

##### 10.5.3.2.3	Procedure

The measurement procedure is the same as described in sub-clause 10.5.2.2.3.

##### 10.5.3.2.4	MU assessment

###### 10.5.3.2.4.1	MU Budget

Table 10.5.3.2.4.1-1: General  Chamber uncertainty contributions
Rx spurious emissions

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E4-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | E4-2 |
| 3 | Quality of quiet zone | E4-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E4-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E4-5 |
| 6 | Phase curvature | E4-6 |
| 7 | Conducted measurement uncertainty | E4-22, F.2 |
| 8 | Impedance mismatch in the receiving chain | E4-8 |
| 9 | Random uncertainty | E4-9 |
| 23 | Measurement antenna frequency variation | E4-20 |
| 24 | FSPL estimation error | E4-21 |
| 25 | Test system frequency flatness | E4-19 |
| 26 | Measurement system dynamic range uncertainty | E4-21 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer |  |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E4-10 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | E4-11 |
| 13 | Quality of quiet zone | E4-12 |
| 14 | Polarization mismatch for reference antenna | E4-3 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E4-4 |
| 16 | Phase curvature | E4-5 |
| 17 | Uncertainty of the network analyzer | E4-6 |
| 18 | Influence of the reference antenna feed cable | E4-13, F.1 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E4-14 |
| 20 | Influence of the receiving antenna feed cable | E4-15 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E4-16 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E4-17 |

###### 10.5.3.2.4.2	MU Value

Table 10.5.3.2.4.2-1: General Chamber uncertainty assessment
Rx spurious emissions

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |  |  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 7 | Conducted measurement uncertainty (minus mismatch) | 1 | 1.99 | Gaussian | 1 | 1 | 1 | 1.99 |
| 8 | Impedance mismatch in the receiving chain | 0.2 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.32 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 23 | Measurement antenna frequency variation | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 24 | FSPL estimation error | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 |
| 25 | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| 26 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | Gaussian | 1 | 1 | 0.51 | 0.51 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.2 | Gaussian | 1 | 1 | 0.13 | 0.2 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.5 | Rectangular | √3 | 1 | 0.29 | 0.29 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.22 | 2.13 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.39 | 4.18 |

#### 10.5.3.3	Void

#### 10.5.3.4	Test Tolerance

The conduced test tolerance for the receiver spurious emissions requirements is zero. However for OTA AAS BS the receiver spurious emissions requirements only apply to TTD in OFF mode. As such the limit is set by RAN4 to be considerably lower than the equivalent regulatory requirement.

In addition due to the difficulty in measuring low levels of TRP close to the measurement system noise floor the risk of false failures is high. As the risk is due to the noise floor of the measurement system it cannot be mitigated by BS design

Hence it has been agreed that for receiver spurious emissions the TT=MU.

#### 10.5.3.5	Summary

Adding the MUperpoint and the SE (see subclause 10.8) we have:

,	30MHz<f≤6 GHz

,	6GHz<f≤19 GHz

By adding the MU per point and the SE values together we have the following total MU:MUtotal = 2.5dB,	30MHz<f≤6GHz

MUtotal = 4.2dB,	6GHz<f≤19GHz

The TT values are as follows:

TT = 2.5dB,	30MHz<f≤6GHz

TT = 4.2dB,	6GHz<f≤19GHz

### 10.5.4	Additional (co-existence) spurious emissions

#### 10.5.4.1	General

The additional spurious emissions requirements consist of the co-existence emissions requirements, and some additional regional requirements such as the protection of PHS and 700 and 800 public safety.

The conducted MU are consistent with the mandatory spurious emissions MU but are split into different frequency sub-divisions

Table 10.5.4.1-1: Conducted additional spurious emissions requirements

| Name | Unit | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| --- | --- | --- | --- | --- |
|  |  | AAS | AAS | E-UTRA |
| 6.7.6.4 Transmitter spurious emissions, Additional spurious emission requirements | dB |  |  |  |
| > -60dBm, f ≤ 3.0GHz |  | 2.0 |  |  |
| > -60dBm, 3.0GHz < f ≤ 4.2GHz |  | 2.5 |  |  |
| > -60dBm, 4.2GHz < f ≤ 6.0GHz |  | 3.0 |  |  |
| > -60dBm, 6.0GHz < f ≤ 26.0GHz |  | x | x | x |
| ≤ -60dBm, f ≤ 3.0GHz |  | 3.0 |  |  |
| ≤ -60dBm, 3.0GHz < f ≤ 4.2GHz |  | 3.5 |  |  |
| ≤ -60dBm, 4.2GHz < f ≤ 6.0GHz |  | 4.0 |  |  |
| ≤ -60dBm, 6.0GHz < f ≤ 26.0GHz |  | x | x | x |

The lower power levels (≤ -60dBm ) are only needed for home BS, as we have no home BS in AAS these can be ignored.

The test set up for the OTA additional emissions requirements is the same as that for the mandatory spurious emissions in Figure 10.5.2.1-1. However, the additional spurious emissions are at a much lower level than the mandatory requirements so the additional effect of the test system dynamic range must be considered in the same way as the receiver emissions requirements.

Unlike the other spurious emissions requirements the additional (co-existence) requirements are specified for other 3GPP bands and as such can be measured in the same chambers as the in-band measurements.

As the CATR MU budget results is the largest MU and is used for setting the in-band MU values only the CATR MU is analysed below. However any suitable IAC or near field) chamber can be used.

#### 10.5.4.2	CATR

##### 10.5.4.2.1	General

The CATR method only is described as it provides the worst MU budget for the additional requirements MU analysis.

##### 10.5.4.2.2	Calibration

The same calibration is carried out as the EIRP calibration described in TR 37.82 [4]

Note: This stage may be omitted provided calibration stage has been performed already during output power measurement

##### 10.5.4.2.3	Procedure

The reference procedure can be found in subclause 10.4.1.3.3.

The appropriate parameters in step 5 is the mean power of additional spurious emissions test over the measurement BW described in the test requirement

##### 10.5.4.2.4	MU assessment

###### 10.5.4.2.4.1	MU Budget

Table 10.5.4.2.4.1-1: Compact antenna test range uncertainty contributions for AAS BS OTA additional spurious emissions measurement

| UID | Description of uncertainty contribution | Details in annex of [1] |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E2-1 |
| 2 | Conducted measurement uncertainty (minus mismatch) | E4-22, F.2 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 5 | QZ ripple DUT | E2-5 |
| 19 | Miscellaneous uncertainty | E2-14 |
| 25 | Measurement system dynamic range uncertainty | E4-22 |
| 26 | Test system frequency flatness | E2-16 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | F.1 |
| 7 | Mismatch of receiver chain | E2-7 |
| 8 | Insertion loss variation of receiver chain | E2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | E2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | F.1 |
| 12 | Misalignment positioning system | E2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | E2-1 |
| 14 | Rotary Joints | E2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | E2-3 |
| 16 | Quality of quiet zone | E2-5 |
| 20 | Switching uncertainty | E2-15 |

###### 10.5.4.2.4.2	MU Value

Table 10.5.4.2.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA additional spurious emissions measurement

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f≤3 GHz | 3GHz<f ≤4.2GHz | 4.2GHz<f ≤6GHz |  |  |  | f≤3 GHz | 3GHz<f ≤4.2GHz | 4.2GHz<f ≤6GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 | 0 |
| 2 | Conducted measurement uncertainty (minus mismatch) | 1.02 | 1.28 | 1.53 | Gaussian | 1 | 1 | 1.02 | 1.28 | 1.53 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 | 0.0928 |
| 25 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | 0.51 | Normal | 1 | 1 | 0.51 | 0.51 | 0.51 |
| 26 | Test system frequency flatness | 0.25 | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.2 | 0.2 | Normal | 1 | 1 | 0.13 | 0.2 | 0.2 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.1 | 0.1 | 0.1 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.5 | 0.433 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1.27 | 1.50 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 2.48 | 2.93 | 3.37 |

#### 10.5.4.3	Void

#### 10.5.4.4	Test Tolerance

The conduced test tolerance for the additional spurious emissions requirements is zero.

However for OTA AAS BS, the difficulty in measuring TRP of the additional spurious emissions requirements at low levels close to the measurement system noise floor means the high risk of false failures. As the risk is due to the noise floor of the measurement system it cannot be mitigated by BS design.

As the 3GPP to 3GPP co-existence requirements are not regulatory but set by RAN4 to assist with co-existence of 3GPP systems in the same geographical area it is acceptable for RAN4 to set the TT value to be non-zero.

Hence it has been agreed that for 3GPP to 3GPP co-existence spurious emissions the TT=MU.

Some additional requirements such as the protection of PHS and the 700 and 800MHz  public safety bands, are regulatory so it is not possible to have anon zero TT, hence for these requirements the TT is zero.

#### 10.5.4.4	Summary

Adding the MUperpoint and the SE (see subclause 10.8) we have:

,	f≤3 GHz

,	3GHz<f≤4.2 GHz

,	4.2GHz<f≤6 GHz

By adding the MU per point and the SE values together we have the following total MU:

MUtotal = 2.6dB,								f≤3 GHz

MUtotal = 3.0dB,	3GHz<f≤4.2 GHz

MUtotal = 3.5dB,	4.2GHz<f≤6 GHz

For additional requirements for co-existence between 3GPP bands the TT values are as follows:

TT = 2.6dB,	f≤3 GHz

TT = 3.0dB,	3GHz<f≤4.2 GHz

TT = 3.5dB,	4.2GHz<f≤6 GHz

For PHS, and public safety additional requirements the TT=0dB.

## 10.8	TRP measurement

### 10.8.1	General

The TRP or the radiated power is simply the total power radiated by an EUT, as in clause 5.1.1. In theory, due to energy conservation, TRP is independent of choice of test distance..

In theory, TRPReference is defined as the integral of the EUT’s far field radiation pattern over a spherical surface, that is

TRPReference= θ=0πϕ=02πUθ,ϕ sinθdθdϕ

where U(θ,ϕ) is the radiation intensity at each angle in watts per Steradian. In the far field, the radiation intensity can be defined as

Uθ,ϕ=EIRP(θ,ϕ)4π

Thus the definite integral for TRPReference becomes

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

Since EIRP is defined only in the far field,  some occasions the power measurements can be performed at distances less than 2d2/λ (the traditional approximate far-field distance ).,  It may be possible to measure power density in near field considering only the magnitude of the tangential part of E field, Etr,θ,ϕ, with an acceptable level of accuracy assuming far field conditions. Thus equation TRPReference can be expressed as

TRPReference= θ=0πϕ=02πr2PDr,θ,ϕ sinθdθdϕ

where  PDr,θ,ϕ=Etr,θ,ϕ2η and η=μ0ε0≈377 Ω    (intrinsic impedance of vacuum).

The minimum distance between the measurement antenna and the smallest sphere enclosing the EUT must be at least 2λ.

In practice, discrete samples of EIRP or power density are measured at different directions (θ,ϕ) over the entire sphere, which are used to numerically approximate the surface TRPReference integral.  The obtained value, TRPEstimate , is an approximation of TRPReference and the difference between them is defined as the TRP summation error (SE) which is

SE = | TRPEstimate – TRPreference |   (in dB)

The SE is the error caused by the numerical integration of measured values on the grid to obtain TRPEstimate, given that the number of test points correspond to the reference angular steps, which are in turn dependent on antenna size and frequency, as described in subclause 10.8.2. A reasonable tradeoff between accuracy and sampling time is achieved when the SE = 0,75 dB.

When measuring radiated power, at each measurement point, two partial results for two orthogonal polarizations needs to be added. These can be the θ and φ polarizations or any pair of orthogonal polarizations.

The distribution of sampling points on the spherical surface depends the type of sampling grids applied. In following subclauses we describe several spherical sampling grids which can be applied to EIRP or power density measurements.

<End of modified section>