3GPP TSG-RAN WG4 Meeting #89	R4-1816485

Spokane, USA, 12th - 16th Nov, 2018

Agenda item:	7.9.4.4.3.1

Source: 	CATT.

Title: 	TP to TS 38.141-2: Removal of test procedure for Tx OFF power and transient period

Document for:	Approval

Introduction

This contribution presents a text proposal to draft TS 38.141-2 [1] to remove the test procedure for Tx OFF power and transient period according to the WF [2].

Reference

- Draft TS 38.141-2 v1.1.0_after Chengdu, Huawei.

- R4-1816314, WF on OTA OFF power and transient time for FR2, CATT

--------------Start of text proposal-------------

6.5	OTA transmit ON/OFF power

6.5.1	OTA transmitter OFF power

6.5.1.1	Definition and applicability

OTA transmitter OFF power requirements apply only to TDD operation of NR BS.

OTA transmitter OFF power is defined as the mean power measured over 70/Nµs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

For BS supporting intra-band contiguous CA, the transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the Aggregated BS Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2 during the transmitter OFF period.

For BS type 1-O, the transmitter OFF power is defined as the output power at the co-location test antenna conducted output(s). For BS type 2-O the transmitter OFF power is defined as TRP.

For multi-band co-location test antenna conducted output(s), the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band co-location test antenna conducted output(s) supporting multiple operating bands, the requirement is applicable per supported operating band.

For multi-band RIBs, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band RIBs supporting transmission in multiple operating bands, the requirement is applicable per supported operating band.

6.5.1.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.2.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.2.3.

6.5.1.3	Test purpose

The purpose of this test is to verify the OTA transmitter OFF power is within the limits of the minimum requirements.

6.5.1.4	Method of test

6.5.1.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for mult-carrier and/or CA:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

Directions to be tested:

- The requirement for FR1 is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

- The requirement for FR2 is specified as TRP requirement. The beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP output power level is achieved.

6.5.1.4.2	Procedure

6.5.1.4.2.1	General procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing the specified measurements.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

6.5.1.4.2.2	BS type 1-O

3)	Place the co-location test antenna as specified in subclause 4.12.

4)	Set the BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test models or set of physical channels in subclause 4.9.2.

5)	Measure the mean power spectrum density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

6)	For an NR BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs.

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.1.4.2.3	BS type 2-O

Editor’s note: Text be added when a suitable measurement procedure is agreed

6.5.1.5	Test requirements

6.5.1.5.1	BS type 1-O

The mean power spectral density measured according to subclause 6.5.1.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.1.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

6.5.1.5.2	BS type 2-O

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency f ≤ XX GHz.

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

The measured mean power spectral density according to subclause 9.5.1.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

For multi-band RIBs, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

6.5.2	OTA transmitter transient period

6.5.2.1	Definition and applicability

The OTA transmitter transient period requirements apply only to TDD operation of BS.

The OTA transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The OTA transmitter transient period is illustrated in figure 6.5.2.1-1.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.5.2.1-1: Illustration of the relations of transmitter ON period,
transmitter OFF period and transmitter transient period

This requirement applies at each co-location test antenna conducted output supporting reception in the operating band for BS type 1-O, and at each RIB supporting transmission in the operating band for BS type 2-O.

6.5.2.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.3.3.

6.5.2.3	Test purpose

The purpose of this test is to verify the OTA transmitter transient periods are within the limits of the minimum requirements.

6.5.2.4	Method of test

6.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

Directions to be tested: The requirement is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

6.5.2.4.2	Procedure

6.5.2.4.2.1	General procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing the specified measurements.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

6.5.2.4.2.2	BS type 1-O

3)	Place the co-location test antenna as specified in subclause 4.12.

4)	Set the BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test models or set of physical channels in subclause 4.9.

5)	Measure the mean power spectrum density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

6)	For an BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs.

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.2.4.2.3	BS type 2-O

Editor’s note: Text be added when a suitable measurement procedure is agreed

6.5.2.5	Test requirements

6.5.2.5.1	BS type 1-O

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

6.5.2.5.2	BS type 2-O

[The estimated TRP according to subclause 9.5.2.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency f ≤ XX GHz.

The estimated TRP according to subclause 9.5.2.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

The estimated TRP according to subclause 9.5.2.4.2 shall be less than [-36+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

For multi-band RIBs, the requirement is only applicable during the transmitter OFF period in all supported operating bands.]