3GPP TSG-RAN WG4 Meeting #89                 	R4-1816536

Spokane, USA, Nov 12th – 16th 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on SA NR UE Transmit Timing Accuracy Tests in FR1 (Section A.6.4.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Adding for NR UE Transmit timing for FR1 for SA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding for NR UE Transmit timing for FR1 for SA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR1 SA transmit timing test desciptions will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section A.6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR…CR... TS38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------START of CHANGES-------

A.6.4.1 UE Transmit Timing

A.6.4.1.1 NR UE Transmit Timing Test for FR1

A.6.4.1.1.1 Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected

gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment,

minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in

clause 7.1.2.

Supported test configurations are shown in Table 6.4.1.1.1-1

Table A.6.4.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, FDD |
| 2 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, TDD |
| 3 | LTE FDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| 4 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, FDD |
| 5 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, TDD |
| 6 | LTE FDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

For this test a single NR cell is used. Table A.6.4.1.1.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.6.4.1.1.1-3.

Table A.6.4.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter |  | Unit | Config | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Channel Number |  |  | 1,2,3,4,5,6 | 1 |  | 1 |  |
| NR Channel Number |  |  | 1,2,3,4,5,6 | 2 |  | 2 |  |
| TDD configuration |  |  | 1,4 | Not Applicable |  |  |  |
|  |  |  | 2,5 | TDDConf.1.1 |  |  |  |
|  |  |  | 3,6 | TDDConf.1.2 |  |  |  |
| BWchannel |  | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |
|  |  |  | 2,5 | 10: NRB,c = 52 |  |  |  |
|  |  |  | 3,6 | 40: NRB,c = 106 |  |  |  |
| BWP | DL BWP |  | 1,2,3,4,5,6 | DLBWP.1.1 |  |  |  |
|  | UL BWP |  | 1,2,3,4,5,6 | ULBWP.1.1 |  |  |  |
| DRx Cycle |  | ms | 1,2,3,4,5,6 | N/A |  |  | 320Note5 |
| PDSCH Reference measurement channel |  |  | 1,4 | SR.1.1 FDD |  |  |  |
|  |  |  | 2,5 | SR.1.1 TDD |  |  |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | 1,4 | CR.1.1 FDD |  |  |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |  |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | 1,4 | CCR.1.1 FDD |  |  |  |
|  |  |  | 2,5 | CCR.1.1 TDD |  |  |  |
|  |  |  | 3,6 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  | 1,2,3,4,5,6 | OP.1 |  |  |  |
| SSB configuration |  |  | 1,2,4,5 | SSB.1 FR1 |  |  |  |
|  |  |  | 3,6 | SSB.2 FR1 |  |  |  |
| SMTC configuration |  |  | 1,2,4,5 | SMTC.1 |  |  |  |
|  |  |  | 3,6 | SMTC.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1,2,3,4,5,6 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | 1,2,3,4,5,6 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | 1,2,4,5 | -98 |  | -98 |  |
|  |  |  | 3,6 | -95 |  | -95 |  |
|  |  |  | 1,2,3,4,5,6 | 3 |  | 3 |  |
|  |  |  | 1,2,3,4,5,6 | 3 |  | 3 |  |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5 | -95 |  | -95 |  |
|  |  |  | 3,6 | -92 |  | -92 |  |
| IoNote3 |  | dBm/9.36MHz | 1,2,4,5 | -65.2 |  | -65.2 |  |
|  |  | dBm/38.1MHz | 3,6 | -59.2 |  | -59.2 |  |
| Propagation condition |  |  | 1,2,3,4,5,6 | AWGN |  |  |  |
| SRS Config |  |  | 1,2,3,4,5,6 | Config1Note6 | Config2Note6 |  |  |

Table A.6.4.1.1.1-3 SRS Configuration for Timing Accuracy Test

|  | Field | Config1 | Config 2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
|  | SRS-ResourceSetId | 0 | 0 |  |
| SRS-Resource | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | sl1 | sl1 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1 | sl640 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

Table A.6.4.1.1.1-4: DRX-Configuration for UL Timing Tests.

| Field | Test 2 |
| --- | --- |
|  | Value |
| drx-onDurationTimer | [ms6] |
| drx-InactivityTimer | [ms1] |
| drx-RetransmissionTimerDL | [sl1] |
| drx-RetransmissionTimerUL | [sl1] |
| longDRX-CycleStartOffset | [ms320] |
| shortDRX | disable |

A.6.4.1.1.2 Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

- Setup NR PCell according to parameters given in Table A.6.4.1.1.1-1.

- After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ± Te of the first detected path of DL SSB.

  - The NTA offset value (in Tc units) is 25600 for FR1 and 13792 for FR2

  - The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

- The test system shall adjust the timing of the DL path by values given in Table A.6.4.1.1.2-1

Table A.6.4.1.1.2-1 Adjustment Value for DL Timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |
| 120 | +16*64Tc | +8*64Tc |
| 240 | +8*64Tc | +4*64Tc |

- The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Section 7.1.2 Table 7.1.2-3. This will only be done for Test1.

- The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ± Te of the first detected path of DL SSB. For Test 2 and Test 4 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

--------------------END of CHANGES------------