3GPP TSG-RAN4 Meeting # 89	R4-1816578

# Spokane, USA,  12  - 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | 0006 | rev | 2 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Annex to TR 37.843: Power density measurements close to EUT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, MVG Industries |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2018-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Information about correct selection of the test distance and size of the measurement antenna for power density measurements close to EUT is not included in the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Annex is added as well as a reference to this annex in clause 10.8.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement of power density close to EUT might be performed under errouneous settings. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.8.1, New annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------- start of change ----------

## 10.8	TRP measurement

### 10.8.1	General

The TRP or the radiated power is simply the total power radiated by an EUT, as in clause 5.1.1. In theory, due to energy conservation, TRP is independent of choice of test distance..

In theory, TRPReference is defined as the integral of the EUT’s far field radiation pattern over a spherical surface, that is

TRPReference= θ=0πϕ=02πUθ,ϕ sinθdθdϕ

where U(θ,ϕ) is the radiation intensity at each angle in watts per Steradian. In the far field, the radiation intensity can be defined as

Uθ,ϕ=EIRP(θ,ϕ)4π

Thus the definite integral for TRPReference becomes

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

Since EIRP is defined only in the far field,  some occasions the power measurements can be performed at distances less than 2d2/λ (the traditional approximate far-field distance ).,  It may be possible to measure power density in near field considering only the magnitude of the tangential part of E field, Etr,θ,ϕ, with an acceptable level of accuracy assuming far field conditions. Thus equation TRPReference can be expressed as

TRPReference= θ=0πϕ=02πr2PDr,θ,ϕ sinθdθdϕ

where  PDr,θ,ϕ=Etr,θ,ϕ2η and η=μ0ε0≈377 Ω    (intrinsic impedance of vacuum).

The minimum distance between the measurement antenna and the smallest sphere enclosing the EUT must be at least 2λ. More details about the necessary conditions for accurate power density measurements close to EUT are included in Annex G.

In practice, discrete samples of EIRP or power density are measured at different directions (θ,ϕ) over the entire sphere, which are used to numerically approximate the surface TRPReference integral.  The obtained value, TRPEstimate , is an approximation of TRPReference and the difference between them is defined as the TRP summation error.

TRPsyserr = | TRPEstimate – TRPreference |   (in dB)

When measuring radiated power, at each measurement point, two partial results for two orthogonal polarizations needs to be added. These can be the θ and φ polarizations or any pair of orthogonal polarizations.

The distribution of sampling points on the spherical surface depends the type of sampling grids applied. In following subclauses we describe several spherical sampling grids which can be applied to EIRP or power density measurements.

### 10.8.2	TRP measurement grids

----------end of changes ----------

--------- start of new annex ----------

# ANNEX G:

# Power density measurements close to EUT

This annex contains guidelines for measurement of power density flux on a sphere surrounding the Equipment Under Test (EUT). The rationale for using a small test distance (compared to the far-field of EUT) is that any amount of power that radiates from the EUT also will radiate through any sphere enclosing the EUT, i.e., energy conservation.

Power flux density based on measurement of tangential electric field components can be used for correct TRP assessment [31]. For accurate power density assessment, the following conditions need to be met:

1. The test distance shall be at least the far-field distance of the measurement antenna, i.e., r0≥2DMA2/λ.

2. The measurement antenna shall sample an approximately constant field.

3. The test distance must be at least two wavelengths from the the smallest sphere enclosing the EUT.

The second criterion is quantified as:

r0≥wλ2Rsph+λ,			(G-1)

where Rsph is the radius of the smallest sphere enclosing the EUT. Furthermore, eq (G-1) sets a minimum test distance for a given measurement antenna size w [31] .

Figure G-1: The measurement distance and the dimensions of EUT and MA.

Example: for assessing TRP using power density data close to EUT, consider the patterns in Figure G-2, which are sampled at (a) 0.75 m, (b) 2.5 m and (c) 30 m from the same EUT. The far-field distance of the EUT is 30 m. The angular patterns vary with distance, but the full-sphere integration for TRP deviates less than 0.065 dB. The lower power density levels in the lower figure are balanced by the larger total area of the measurement sphere.

Figure G-2: Angular power density patterns measured at (a) 0.75 m, (b) 2.5 m and (c) 30 m from the EUT [31].

--------- end of new Annex ----------