3GPP TSG-RAN WG4 Meeting # 89	R4-1816581

# Spokane, USA,  12 November, 2018 - 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RX beamforming in interfrequency FR2 requirements for PC4 in section 9.3 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Docomo, Verizon |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  |  |  | Date: |  | 2018/15/11 |
| Category: | F |  |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements for Mpss_sss_sync, MSSB_index_inter and Mmeasurement_period for PC4 in FR2 are FFS. Square brackets remain in other requirements |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify Mpss_sss_sync and Mmeasurement_period=[40] for FR2 PC4 and remove square brackets from other cases. Clarify that the power class is the FR2 power class, since a UE may also have an FR1 power class which is not relevant to Mpss_sss_sync and Mmeasurement_period |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for PC4 are not specified. Requirements for time index reading are not specified. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |

### 9.3.4	Inter frequency cell identification

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within TSSB_time_index_inter.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1 and table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3 and table 9.3.4-4.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.5-1 and table 9.3.5-2.

Mpss/sss_sync_inter : For a UE supporting FR2 power class 1, Mpss/sss_sync_inter=64 samples. For a UE supporting FR2 power class 2 (vehicle mounted), Mpss/sss_sync_inter=40 samples. For a UE supporting FR2 power class 3 (handheld), Mpss/sss_sync_inter=40 samples. For a UE supporting FR2 power class 4, Mpss/sss_sync=[40] samples.

MSSB_index_inter: For a UE supporting power class 1, MSSB_index_inter =[TBD] samples. For a vehicle mounted UE supporting power class 2 or a UE supporting power class 3, MSSB_index_inter =[TBD] samples. For a UE supporting power class 4, Mmeas_period_inter =[TBD] samples.

Mmeas_period_inter: For a UE supporting FR2 power class 1, Mmeas_period_inter =64 samples. For a vehicle mounted UE supporting FR2 power class 2 (vehicle mounted), Mpss/sss_sync_inter=40 samples.For a UE supporting FR2 power class 3 (handheld), Mmeas_period_inter =40 samples. For a UE supporting FR2 power class 4, Mmeas_period_inter =[40] samples.