3GPP TSG-RAN WG4 Meeting #89	R4-1816604

# Spokane, WA, USA,  12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draftCR | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TDD configuration for UE Tx test in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR TDD slot pattern (UL/DL configuration) used for UE UL tests may not be synchronized with LTE. Contiguous UL transmission time for SCS=60kHz is less than 1ms. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the TDD UL/DL configuration for SCS=60kHz so that the their UL/DL allocation is aliged with LTE TDD configuration 2 and special subframe configuration 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | According to the A.2.3, TDD slot pattern for REFSENS is applied for UE UL test. - If this pattern is used for TDD-TDD intra-band LTE-NR EN-DC scenario for example, it may cause the cross link interference and the UE may not satisfy the core requirement. - The core requirements of MOP, Transmit off power, and frequecny error shall be measured with the measurement period of 1ms. Since the current TDD slot patten for SCS=60kHz has only 0.25ms for UL period, RAN5 need to sepcify the measurment procesure with 4 discoutiguous UL slots. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.3, A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

## A.2.3	Reference measurement channels for TDD

TDD slot patterns defined for reference sensitivity tests will be used for UL RMCs defined below.

### A.2.3.1	DFT-s-OFDM Pi/2-BPSK

Table A.2.3.1-1: Reference Channels for DFT-s-OFDM Pi/2-BPSK for 15kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 5 | 15 | 12 | 11 | pi/2 BPSK | 0 | 1/4 | 384 | 16 | 2 | 1 | 1584 | 1584 |
|  | 5 | 15 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 10 | 15 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 10 | 15 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 15 | 15 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 15 | 15 | 75 | 11 | pi/2 BPSK | 0 | 1/4 | 2408 | 16 | 2 | 1 | 9900 | 9900 |
|  | 20 | 15 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 20 | 15 | 100 | 11 | pi/2 BPSK | 0 | 1/4 | 3104 | 16 | 2 | 1 | 13200 | 13200 |
|  | 25 | 15 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 25 | 15 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 30 | 15 | 80 | 11 | pi/2 BPSK | 0 | 1/4 | 2472 | 16 | 2 | 1 | 10560 | 10560 |
|  | 30 | 15 | 160 | 11 | pi/2 BPSK | 0 | 1/4 | 4872 | 24 | 2 | 2 | 21120 | 21120 |
|  | 40 | 15 | 108 | 11 | pi/2 BPSK | 0 | 1/4 | 3368 | 16 | 2 | 1 | 14256 | 14256 |
|  | 40 | 15 | 216 | 11 | pi/2 BPSK | 0 | 1/4 | 6664 | 24 | 2 | 2 | 28512 | 28512 |
|  | 50 | 15 | 135 | 11 | pi/2 BPSK | 0 | 1/4 | 4104 | 24 | 2 | 2 | 17820 | 17820 |
|  | 50 | 15 | 270 | 11 | pi/2 BPSK | 0 | 1/4 | 8448 | 24 | 2 | 3 | 35640 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.1-2: Reference Channels for DFT-s-OFDM Pi/2-BPSK for 30kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 30 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 5 | 30 | 5 | 11 | pi/2 BPSK | 0 | 1/4 | 160 | 16 | 2 | 1 | 660 | 660 |
|  | 5 | 30 | 10 | 11 | pi/2 BPSK | 0 | 1/4 | 320 | 16 | 2 | 1 | 1320 | 1320 |
|  | 10 | 30 | 12 | 11 | pi/2 BPSK | 0 | 1/4 | 384 | 16 | 2 | 1 | 1584 | 1584 |
|  | 10 | 30 | 24 | 11 | pi/2 BPSK | 0 | 1/4 | 768 | 16 | 2 | 1 | 3168 | 3168 |
|  | 15 | 30 | 18 | 11 | pi/2 BPSK | 0 | 1/4 | 576 | 16 | 2 | 1 | 2376 | 2376 |
|  | 15 | 30 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 20 | 30 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 20 | 30 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 25 | 30 | 32 | 11 | pi/2 BPSK | 0 | 1/4 | 1032 | 16 | 2 | 1 | 4224 | 4224 |
|  | 25 | 30 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 30 | 30 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 30 | 30 | 75 | 11 | pi/2 BPSK | 0 | 1/4 | 2408 | 16 | 2 | 1 | 9900 | 9900 |
|  | 40 | 30 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 40 | 30 | 100 | 11 | pi/2 BPSK | 0 | 1/4 | 3104 | 16 | 2 | 1 | 13200 | 13200 |
|  | 50 | 30 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 50 | 30 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 60 | 30 | 81 | 11 | pi/2 BPSK | 0 | 1/4 | 2536 | 16 | 2 | 1 | 10692 | 10692 |
|  | 60 | 30 | 162 | 11 | pi/2 BPSK | 0 | 1/4 | 5000 | 24 | 2 | 2 | 21384 | 21384 |
|  | 80 | 30 | 108 | 11 | pi/2 BPSK | 0 | 1/4 | 3368 | 16 | 2 | 1 | 14256 | 14256 |
|  | 80 | 30 | 216 | 11 | pi/2 BPSK | 0 | 1/4 | 6664 | 24 | 2 | 2 | 28512 | 28512 |
|  | 90 | 30 | 120 | 11 | pi/2 BPSK | 0 | 1/4 | 3752 | 16 | 2 | 1 | 15840 | 15840 |
|  | 90 | 30 | 243 | 11 | pi/2 BPSK | 0 | 1/4 | 7560 | 24 | 2 | 2 | 32076 | 32076 |
|  | 100 | 30 | 135 | 11 | pi/2 BPSK | 0 | 1/4 | 4104 | 24 | 2 | 2 | 17820 | 17820 |
|  | 100 | 30 | 270 | 11 | pi/2 BPSK | 0 | 1/4 | 8448 | 24 | 2 | 3 | 35640 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.1-3: Reference Channels for DFT-s-OFDM Pi/2-BPSK for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 10 | 60 | 5 | 11 | pi/2 BPSK | 0 | 1/4 | 160 | 16 | 2 | 1 | 660 | 660 |
|  | 10 | 60 | 10 | 11 | pi/2 BPSK | 0 | 1/4 | 320 | 16 | 2 | 1 | 1320 | 1320 |
|  | 15 | 60 | 9 | 11 | pi/2 BPSK | 0 | 1/4 | 288 | 16 | 2 | 1 | 1188 | 1188 |
|  | 15 | 60 | 18 | 11 | pi/2 BPSK | 0 | 1/4 | 576 | 16 | 2 | 1 | 2376 | 2376 |
|  | 20 | 60 | 12 | 11 | pi/2 BPSK | 0 | 1/4 | 384 | 16 | 2 | 1 | 1584 | 1584 |
|  | 20 | 60 | 24 | 11 | pi/2 BPSK | 0 | 1/4 | 768 | 16 | 2 | 1 | 3168 | 3168 |
|  | 25 | 60 | 15 | 11 | pi/2 BPSK | 0 | 1/4 | 480 | 16 | 2 | 1 | 1980 | 1980 |
|  | 25 | 60 | 30 | 11 | pi/2 BPSK | 0 | 1/4 | 984 | 16 | 2 | 1 | 3960 | 3960 |
|  | 30 | 60 | 18 | 11 | pi/2 BPSK | 0 | 1/4 | 576 | 16 | 2 | 1 | 2376 | 2376 |
|  | 30 | 60 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 40 | 60 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 40 | 60 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 50 | 60 | 32 | 11 | pi/2 BPSK | 0 | 1/4 | 1032 | 16 | 2 | 1 | 4224 | 4224 |
|  | 50 | 60 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 60 | 60 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 60 | 60 | 75 | 11 | pi/2 BPSK | 0 | 1/4 | 2408 | 16 | 2 | 1 | 9900 | 9900 |
|  | 80 | 60 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 80 | 60 | 100 | 11 | pi/2 BPSK | 0 | 1/4 | 3104 | 16 | 2 | 1 | 13200 | 13200 |
|  | 90 | 60 | 60 | 11 | pi/2 BPSK | 0 | 1/4 | 1864 | 16 | 2 | 1 | 7920 | 7920 |
|  | 90 | 60 | 120 | 11 | pi/2 BPSK | 0 | 1/4 | 3752 | 16 | 2 | 1 | 15840 | 15840 |
|  | 100 | 60 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 100 | 60 | 135 | 11 | pi/2 BPSK | 0 | 1/4 | 4104 | 24 | 2 | 2 | 17820 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.2	DFT-s-OFDM QPSK

Table A.2.3.2-1: Reference channels for DFT-s-OFDM QPSK for 15kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 5 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 10 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 15 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 15 | 15 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 15 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 20 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 20 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 20 | 15 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 25 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 15 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 25 | 15 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 30 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.2-2: Reference channels for DFT-s-OFDM QPSK for 30kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 5 | 30 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 15 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 20 | 30 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 30 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 25 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 30 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 30 | 30 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 40 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 40 | 30 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 50 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 50 | 30 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 80 | 30 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 80 | 30 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 90 | 30 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 90 | 30 | 243 | 11 | QPSK | 2 | 1/6 | 12040 | 24 | 2 | 4 | 64152 | 32076 |
|  | 100 | 30 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 100 | 30 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.2-3: Reference channels for DFT-s-OFDM QPSK for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 10 | 60 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 15 | 11 | QPSK | 2 | 1/6 | 768 | 16 | 2 | 1 | 3960 | 1980 |
|  | 25 | 60 | 30 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 7920 | 3960 |
|  | 30 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 30 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 40 | 60 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 40 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 50 | 60 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 50 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 60 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 60 | 60 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 80 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 80 | 60 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 90 | 60 | 60 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 15840 | 7920 |
|  | 90 | 60 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 100 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.3	DFT-s-OFDM 16QAM

Table A.2.3.3-1: Reference channels for DFT-s-OFDM 16QAM for 15kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 15 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 5 | 15 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 10 | 15 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 10 | 15 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 15 | 15 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 15 | 15 | 75 | 11 | 16QAM | 10 | 1/3 | 13064 | 24 | 1 | 2 | 39600 | 9900 |
|  | 20 | 15 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 20 | 15 | 100 | 11 | 16QAM | 10 | 1/3 | 17424 | 24 | 1 | 3 | 52800 | 13200 |
|  | 25 | 15 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 25 | 15 | 128 | 11 | 16QAM | 10 | 1/3 | 22536 | 24 | 1 | 3 | 67584 | 16896 |
|  | 30 | 15 | 80 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42240 | 10560 |
|  | 30 | 15 | 160 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 84480 | 21120 |
|  | 40 | 15 | 108 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57024 | 14256 |
|  | 40 | 15 | 216 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114048 | 28512 |
|  | 50 | 15 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
|  | 50 | 15 | 270 | 11 | 16QAM | 10 | 1/3 | 47112 | 24 | 1 | 6 | 142560 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.3-2: Reference channels for DFT-s-OFDM 16QAM for 30kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 30 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 30 | 5 | 11 | 16QAM | 10 | 1/3 | 888 | 16 | 2 | 1 | 2640 | 660 |
|  | 5 | 30 | 10 | 11 | 16QAM | 10 | 1/3 | 1800 | 16 | 2 | 1 | 5280 | 1320 |
|  | 10 | 30 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 10 | 30 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 15 | 30 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 15 | 30 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 20 | 30 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 20 | 30 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 25 | 30 | 32 | 11 | 16QAM | 10 | 1/3 | 5632 | 24 | 1 | 1 | 16896 | 4224 |
|  | 25 | 30 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 30 | 30 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 30 | 30 | 75 | 11 | 16QAM | 10 | 1/3 | 13064 | 24 | 1 | 2 | 39600 | 9900 |
|  | 40 | 30 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 40 | 30 | 100 | 11 | 16QAM | 10 | 1/3 | 17424 | 24 | 1 | 3 | 52800 | 13200 |
|  | 50 | 30 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 50 | 30 | 128 | 11 | 16QAM | 10 | 1/3 | 22536 | 24 | 1 | 3 | 67584 | 16896 |
|  | 60 | 30 | 81 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42768 | 10692 |
|  | 60 | 30 | 162 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 85536 | 21384 |
|  | 80 | 30 | 108 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57024 | 14256 |
|  | 80 | 30 | 216 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114048 | 28512 |
|  | 90 | 30 | 120 | 11 | 16QAM | 10 | 1/3 | 21000 | 24 | 1 | 3 | 63360 | 15840 |
|  | 90 | 30 | 243 | 11 | 16QAM | 10 | 1/3 | 43032 | 24 | 1 | 6 | 128304 | 32076 |
|  | 100 | 30 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
|  | 100 | 30 | 270 | 11 | 16QAM | 10 | 1/3 | 47112 | 24 | 1 | 6 | 142560 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.3-3: Reference channels for DFT-s-OFDM 16QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 10 | 60 | 5 | 11 | 16QAM | 10 | 1/3 | 888 | 16 | 2 | 1 | 2640 | 660 |
|  | 10 | 60 | 10 | 11 | 16QAM | 10 | 1/3 | 1800 | 16 | 2 | 1 | 5280 | 1320 |
|  | 15 | 60 | 9 | 11 | 16QAM | 10 | 1/3 | 1608 | 16 | 2 | 1 | 4752 | 1188 |
|  | 15 | 60 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 20 | 60 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 20 | 60 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 25 | 60 | 15 | 11 | 16QAM | 10 | 1/3 | 2664 | 16 | 2 | 1 | 7920 | 1980 |
|  | 25 | 60 | 30 | 11 | 16QAM | 10 | 1/3 | 5248 | 24 | 1 | 1 | 15840 | 3960 |
|  | 30 | 60 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 30 | 60 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 40 | 60 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 40 | 60 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 50 | 60 | 32 | 11 | 16QAM | 10 | 1/3 | 5632 | 24 | 1 | 1 | 16896 | 4224 |
|  | 50 | 60 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 60 | 60 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 60 | 60 | 75 | 11 | 16QAM | 10 | 1/3 | 13064 | 24 | 1 | 2 | 39600 | 9900 |
|  | 80 | 60 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 80 | 60 | 100 | 11 | 16QAM | 10 | 1/3 | 17424 | 24 | 1 | 3 | 52800 | 13200 |
|  | 90 | 60 | 60 | 11 | 16QAM | 10 | 1/3 | 10504 | 24 | 1 | 2 | 31680 | 7920 |
|  | 90 | 60 | 120 | 11 | 16QAM | 10 | 1/3 | 21000 | 24 | 1 | 3 | 63360 | 15840 |
|  | 100 | 60 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 100 | 60 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.4	DFT-s-OFDM 64QAM

Table A.2.3.4-1: Reference channels for DFT-s-OFDM 64QAM for 15kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 64QAM | 18 | 1/2 | 9992 | 24 | 1 | 2 | 19800 | 3300 |
|  | 10 | 15 | 50 | 11 | 64QAM | 18 | 1/2 | 19968 | 24 | 1 | 3 | 39600 | 6600 |
|  | 15 | 15 | 75 | 11 | 64QAM | 18 | 1/2 | 30216 | 24 | 1 | 4 | 59400 | 9900 |
|  | 20 | 15 | 100 | 11 | 64QAM | 18 | 1/2 | 39936 | 24 | 1 | 5 | 79200 | 13200 |
|  | 25 | 15 | 128 | 11 | 64QAM | 18 | 1/2 | 51216 | 24 | 1 | 7 | 101376 | 16896 |
|  | 30 | 15 | 160 | 11 | 64QAM | 18 | 1/2 | 63528 | 24 | 1 | 8 | 126720 | 21120 |
|  | 40 | 15 | 216 | 11 | 64QAM | 18 | 1/2 | 86040 | 24 | 1 | 11 | 171072 | 28512 |
|  | 50 | 15 | 270 | 11 | 64QAM | 18 | 1/2 | 108552 | 24 | 1 | 13 | 213840 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.4-2: Reference channels for DFT-s-OFDM 64QAM for 30kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 10 | 11 | 64QAM | 18 | 1/2 | 3968 | 24 | 1 | 1 | 7920 | 1320 |
|  | 10 | 30 | 24 | 11 | 64QAM | 18 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 15 | 30 | 36 | 11 | 64QAM | 18 | 1/2 | 14344 | 24 | 1 | 2 | 28512 | 4752 |
|  | 20 | 30 | 50 | 11 | 64QAM | 18 | 1/2 | 19968 | 24 | 1 | 3 | 39600 | 6600 |
|  | 25 | 30 | 64 | 11 | 64QAM | 18 | 1/2 | 25608 | 24 | 1 | 4 | 50688 | 8448 |
|  | 30 | 30 | 75 | 11 | 64QAM | 18 | 1/2 | 30216 | 24 | 1 | 4 | 59400 | 9900 |
|  | 40 | 30 | 100 | 11 | 64QAM | 18 | 1/2 | 39936 | 24 | 1 | 5 | 79200 | 13200 |
|  | 50 | 30 | 128 | 11 | 64QAM | 18 | 1/2 | 51216 | 24 | 1 | 7 | 101376 | 16896 |
|  | 60 | 30 | 162 | 11 | 64QAM | 18 | 1/2 | 64552 | 24 | 1 | 8 | 128304 | 21384 |
|  | 80 | 30 | 216 | 11 | 64QAM | 18 | 1/2 | 86040 | 24 | 1 | 11 | 171072 | 28512 |
|  | 90 | 30 | 243 | 11 | 64QAM | 18 | 1/2 | 96264 | 24 | 1 | 12 | 192456 | 32076 |
|  | 100 | 30 | 270 | 11 | 64QAM | 18 | 1/2 | 108552 | 24 | 1 | 13 | 213840 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.4-3: Reference channels for DFT-s-OFDM 64QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 10 | 11 | 64QAM | 18 | 1/2 | 3968 | 24 | 1 | 1 | 7920 | 1320 |
|  | 15 | 60 | 18 | 11 | 64QAM | 18 | 1/2 | 7168 | 24 | 1 | 1 | 14256 | 2376 |
|  | 20 | 60 | 24 | 11 | 64QAM | 18 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 25 | 60 | 30 | 11 | 64QAM | 18 | 1/2 | 12040 | 24 | 1 | 2 | 23760 | 3960 |
|  | 30 | 60 | 36 | 11 | 64QAM | 18 | 1/2 | 14344 | 24 | 1 | 2 | 28512 | 4752 |
|  | 40 | 60 | 50 | 11 | 64QAM | 18 | 1/2 | 19968 | 24 | 1 | 3 | 39600 | 6600 |
|  | 50 | 60 | 64 | 11 | 64QAM | 18 | 1/2 | 25608 | 24 | 1 | 4 | 50688 | 8448 |
|  | 60 | 60 | 75 | 11 | 64QAM | 18 | 1/2 | 30216 | 24 | 1 | 4 | 59400 | 9900 |
|  | 80 | 60 | 100 | 11 | 64QAM | 18 | 1/2 | 39936 | 24 | 1 | 5 | 79200 | 13200 |
|  | 90 | 60 | 120 | 11 | 64QAM | 18 | 1/2 | 48168 | 24 | 1 | 6 | 95040 | 15840 |
|  | 100 | 60 | 135 | 11 | 64QAM | 18 | 1/2 | 54296 | 24 | 1 | 7 | 106920 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.5	DFT-s-OFDM 256QAM

Table A.2.3.5-1: Reference channels for DFT-s-OFDM 256QAM for 15kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 256QAM | 20 | 2/3 | 17424 | 24 | 1 | 3 | 26400 | 3300 |
|  | 10 | 15 | 50 | 11 | 256QAM | 20 | 2/3 | 34816 | 24 | 1 | 5 | 52800 | 6600 |
|  | 15 | 15 | 75 | 11 | 256QAM | 20 | 2/3 | 53288 | 24 | 1 | 7 | 79200 | 9900 |
|  | 20 | 15 | 100 | 11 | 256QAM | 20 | 2/3 | 69672 | 24 | 1 | 9 | 105600 | 13200 |
|  | 25 | 15 | 128 | 11 | 256QAM | 20 | 2/3 | 90176 | 24 | 1 | 11 | 135168 | 16896 |
|  | 30 | 15 | 160 | 11 | 256QAM | 20 | 2/3 | 112648 | 24 | 1 | 14 | 168960 | 21120 |
|  | 40 | 15 | 216 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 228096 | 28512 |
|  | 50 | 15 | 270 | 11 | 256QAM | 20 | 2/3 | 188576 | 24 | 1 | 23 | 285120 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.5-2: Reference channels for DFT-s-OFDM 256QAM for 30kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 10 | 11 | 256QAM | 20 | 2/3 | 7040 | 24 | 1 | 1 | 10560 | 1320 |
|  | 10 | 30 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 15 | 30 | 36 | 11 | 256QAM | 20 | 2/3 | 25104 | 24 | 1 | 3 | 38016 | 4752 |
|  | 20 | 30 | 50 | 11 | 256QAM | 20 | 2/3 | 34816 | 24 | 1 | 5 | 52800 | 6600 |
|  | 25 | 30 | 64 | 11 | 256QAM | 20 | 2/3 | 45096 | 24 | 1 | 6 | 67584 | 8448 |
|  | 30 | 30 | 75 | 11 | 256QAM | 20 | 2/3 | 53288 | 24 | 1 | 7 | 79200 | 9900 |
|  | 40 | 30 | 100 | 11 | 256QAM | 20 | 2/3 | 69672 | 24 | 1 | 9 | 105600 | 13200 |
|  | 50 | 30 | 128 | 11 | 256QAM | 20 | 2/3 | 90176 | 24 | 1 | 11 | 135168 | 16896 |
|  | 60 | 30 | 162 | 11 | 256QAM | 20 | 2/3 | 114776 | 24 | 1 | 14 | 171072 | 21384 |
|  | 80 | 30 | 216 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 228096 | 28512 |
|  | 90 | 30 | 243 | 11 | 256QAM | 20 | 2/3 | 172176 | 24 | 1 | 21 | 256608 | 32076 |
|  | 100 | 30 | 270 | 11 | 256QAM | 20 | 2/3 | 188576 | 24 | 1 | 23 | 285120 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.5-3: Reference channels for DFT-s-OFDM 256QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 10 | 11 | 256QAM | 20 | 2/3 | 7040 | 24 | 1 | 1 | 10560 | 1320 |
|  | 15 | 60 | 18 | 11 | 256QAM | 20 | 2/3 | 12552 | 24 | 1 | 2 | 19008 | 2376 |
|  | 20 | 60 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 25 | 60 | 30 | 11 | 256QAM | 20 | 2/3 | 21000 | 24 | 1 | 3 | 31680 | 3960 |
|  | 30 | 60 | 36 | 11 | 256QAM | 20 | 2/3 | 25104 | 24 | 1 | 3 | 38016 | 4752 |
|  | 40 | 60 | 50 | 11 | 256QAM | 20 | 2/3 | 34816 | 24 | 1 | 5 | 52800 | 6600 |
|  | 50 | 60 | 64 | 11 | 256QAM | 20 | 2/3 | 45096 | 24 | 1 | 6 | 67584 | 8448 |
|  | 60 | 60 | 75 | 11 | 256QAM | 20 | 2/3 | 53288 | 24 | 1 | 7 | 79200 | 9900 |
|  | 80 | 60 | 100 | 11 | 256QAM | 20 | 2/3 | 69672 | 24 | 1 | 9 | 105600 | 13200 |
|  | 90 | 60 | 120 | 11 | 256QAM | 20 | 2/3 | 83976 | 24 | 1 | 10 | 126720 | 15840 |
|  | 100 | 60 | 135 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 142560 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.6	CP-OFDM QPSK

Table A.2.3.6-1: Reference channels for CP-OFDM QPSK for 15kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 13 | 11 | QPSK | 2 | 1/6 | 672 | 16 | 2 | 1 | 3432 | 1716 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 10 | 15 | 52 | 11 | QPSK | 2 | 1/6 | 2600 | 16 | 2 | 1 | 13728 | 6864 |
|  | 15 | 15 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 15 | 15 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 20 | 15 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 20 | 15 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 25 | 15 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 25 | 15 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.6-2: Reference channels for CP-OFDM QPSK for 30kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 5 | 30 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 15 | 30 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 20 | 30 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 20 | 30 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 25 | 30 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 25 | 30 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 30 | 30 | 39 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10296 | 5148 |
|  | 30 | 30 | 78 | 11 | QPSK | 2 | 1/6 | 3848 | 24 | 2 | 2 | 20592 | 10296 |
|  | 40 | 30 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 40 | 30 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 50 | 30 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 50 | 30 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 80 | 30 | 109 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28776 | 14388 |
|  | 80 | 30 | 217 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57288 | 28644 |
|  | 90 | 30 | 123 | 11 | QPSK | 2 | 1/6 | 6152 | 24 | 2 | 2 | 32472 | 16236 |
|  | 90 | 30 | 245 | 11 | QPSK | 2 | 1/6 | 12296 | 24 | 2 | 4 | 64680 | 32340 |
|  | 100 | 30 | 137 | 11 | QPSK | 2 | 1/6 | 6792 | 24 | 2 | 2 | 36168 | 18084 |
|  | 100 | 30 | 273 | 11 | QPSK | 2 | 1/6 | 13576 | 24 | 2 | 4 | 72072 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.6-3: Reference channels for CP-OFDM QPSK for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 10 | 60 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 16 | 11 | QPSK | 2 | 1/6 | 808 | 16 | 2 | 1 | 4224 | 2112 |
|  | 25 | 60 | 31 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 8184 | 4092 |
|  | 30 | 60 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 30 | 60 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 40 | 60 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 40 | 60 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 50 | 60 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 50 | 60 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 60 | 60 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 60 | 60 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 80 | 60 | 54 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 14256 | 7128 |
|  | 80 | 60 | 107 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 28248 | 14124 |
|  | 90 | 60 | 61 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 16104 | 8052 |
|  | 90 | 60 | 121 | 11 | QPSK | 2 | 1/6 | 6024 | 24 | 2 | 2 | 31944 | 15972 |
|  | 100 | 60 | 68 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17952 | 8976 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.7	CP-OFDM 16QAM

Table A.2.3.7-1: Reference channels for CP-OFDM 16QAM for 15kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 15 | 13 | 11 | 16QAM | 10 | 1/3 | 2280 | 16 | 2 | 1 | 6864 | 1716 |
|  | 5 | 15 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 10 | 15 | 26 | 11 | 16QAM | 10 | 1/3 | 4480 | 24 | 1 | 1 | 13728 | 3432 |
|  | 10 | 15 | 52 | 11 | 16QAM | 10 | 1/3 | 9224 | 24 | 1 | 2 | 27456 | 6864 |
|  | 15 | 15 | 40 | 11 | 16QAM | 10 | 1/3 | 7040 | 24 | 1 | 1 | 21120 | 5280 |
|  | 15 | 15 | 79 | 11 | 16QAM | 10 | 1/3 | 13832 | 24 | 1 | 2 | 41712 | 10428 |
|  | 20 | 15 | 53 | 11 | 16QAM | 10 | 1/3 | 9224 | 24 | 1 | 2 | 27984 | 6996 |
|  | 20 | 15 | 106 | 11 | 16QAM | 10 | 1/3 | 18432 | 24 | 1 | 3 | 55968 | 13992 |
|  | 25 | 15 | 67 | 11 | 16QAM | 10 | 1/3 | 11784 | 24 | 1 | 2 | 35376 | 8844 |
|  | 25 | 15 | 133 | 11 | 16QAM | 10 | 1/3 | 23040 | 24 | 1 | 3 | 70224 | 17556 |
|  | 30 | 15 | 80 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42240 | 10560 |
|  | 30 | 15 | 160 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 84480 | 21120 |
|  | 40 | 15 | 108 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57024 | 14256 |
|  | 40 | 15 | 216 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114048 | 28512 |
|  | 50 | 15 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
|  | 50 | 15 | 270 | 11 | 16QAM | 10 | 1/3 | 47112 | 24 | 1 | 6 | 142560 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.7-2: Reference channels for CP-OFDM 16QAM for 30kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 30 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 30 | 6 | 11 | 16QAM | 10 | 1/3 | 1064 | 16 | 2 | 1 | 3168 | 792 |
|  | 5 | 30 | 11 | 11 | 16QAM | 10 | 1/3 | 1928 | 16 | 2 | 1 | 5808 | 1452 |
|  | 10 | 30 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 10 | 30 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 15 | 30 | 19 | 11 | 16QAM | 10 | 1/3 | 3368 | 16 | 2 | 1 | 10032 | 2508 |
|  | 15 | 30 | 38 | 11 | 16QAM | 10 | 1/3 | 6656 | 24 | 1 | 1 | 20064 | 5016 |
|  | 20 | 30 | 26 | 11 | 16QAM | 10 | 1/3 | 4480 | 24 | 1 | 1 | 13728 | 3432 |
|  | 20 | 30 | 51 | 11 | 16QAM | 10 | 1/3 | 8968 | 24 | 1 | 2 | 26928 | 6732 |
|  | 25 | 30 | 33 | 11 | 16QAM | 10 | 1/3 | 5760 | 24 | 1 | 1 | 17424 | 4356 |
|  | 25 | 30 | 65 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 34320 | 8580 |
|  | 30 | 30 | 39 | 11 | 16QAM | 10 | 1/3 | 6784 | 24 | 1 | 1 | 20592 | 5148 |
|  | 30 | 30 | 78 | 11 | 16QAM | 10 | 1/3 | 13576 | 24 | 1 | 2 | 41184 | 10296 |
|  | 40 | 30 | 53 | 11 | 16QAM | 10 | 1/3 | 9224 | 24 | 1 | 2 | 27984 | 6996 |
|  | 40 | 30 | 106 | 11 | 16QAM | 10 | 1/3 | 18432 | 24 | 1 | 3 | 55968 | 13992 |
|  | 50 | 30 | 67 | 11 | 16QAM | 10 | 1/3 | 11784 | 24 | 1 | 2 | 35376 | 8844 |
|  | 50 | 30 | 133 | 11 | 16QAM | 10 | 1/3 | 23040 | 24 | 1 | 3 | 70224 | 17556 |
|  | 60 | 30 | 81 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42768 | 10692 |
|  | 60 | 30 | 162 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 85536 | 21384 |
|  | 80 | 30 | 109 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57552 | 14388 |
|  | 80 | 30 | 217 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114576 | 28644 |
|  | 90 | 30 | 123 | 11 | 16QAM | 10 | 1/3 | 21504 | 24 | 1 | 3 | 64944 | 16236 |
|  | 90 | 30 | 245 | 11 | 16QAM | 10 | 1/3 | 43032 | 24 | 1 | 6 | 129360 | 32340 |
|  | 100 | 30 | 137 | 11 | 16QAM | 10 | 1/3 | 24072 | 24 | 1 | 3 | 72336 | 18084 |
|  | 100 | 30 | 273 | 11 | 16QAM | 10 | 1/3 | 48168 | 24 | 1 | 6 | 144144 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.7-3: Reference channels for CP-OFDM 16QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 10 | 60 | 6 | 11 | 16QAM | 10 | 1/3 | 1064 | 16 | 2 | 1 | 3168 | 792 |
|  | 10 | 60 | 11 | 11 | 16QAM | 10 | 1/3 | 1928 | 16 | 2 | 1 | 5808 | 1452 |
|  | 15 | 60 | 9 | 11 | 16QAM | 10 | 1/3 | 1608 | 16 | 2 | 1 | 4752 | 1188 |
|  | 15 | 60 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 20 | 60 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 20 | 60 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 25 | 60 | 16 | 11 | 16QAM | 10 | 1/3 | 2792 | 16 | 2 | 1 | 8448 | 2112 |
|  | 25 | 60 | 31 | 11 | 16QAM | 10 | 1/3 | 5376 | 24 | 1 | 1 | 16368 | 4092 |
|  | 30 | 60 | 19 | 11 | 16QAM | 10 | 1/3 | 3368 | 16 | 2 | 1 | 10032 | 2508 |
|  | 30 | 60 | 38 | 11 | 16QAM | 10 | 1/3 | 6656 | 24 | 1 | 1 | 20064 | 5016 |
|  | 40 | 60 | 26 | 11 | 16QAM | 10 | 1/3 | 4480 | 24 | 1 | 1 | 13728 | 3432 |
|  | 40 | 60 | 51 | 11 | 16QAM | 10 | 1/3 | 8968 | 24 | 1 | 2 | 26928 | 6732 |
|  | 50 | 60 | 33 | 11 | 16QAM | 10 | 1/3 | 5760 | 24 | 1 | 1 | 17424 | 4356 |
|  | 50 | 60 | 65 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 34320 | 8580 |
|  | 60 | 60 | 40 | 11 | 16QAM | 10 | 1/3 | 7040 | 24 | 1 | 1 | 21120 | 5280 |
|  | 60 | 60 | 79 | 11 | 16QAM | 10 | 1/3 | 13832 | 24 | 1 | 2 | 41712 | 10428 |
|  | 80 | 60 | 54 | 11 | 16QAM | 10 | 1/3 | 9480 | 24 | 1 | 2 | 28512 | 7128 |
|  | 80 | 60 | 107 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 56496 | 14124 |
|  | 90 | 60 | 61 | 11 | 16QAM | 10 | 1/3 | 10760 | 24 | 1 | 2 | 32208 | 8052 |
|  | 90 | 60 | 121 | 11 | 16QAM | 10 | 1/3 | 21000 | 24 | 1 | 3 | 63888 | 15972 |
|  | 100 | 60 | 68 | 11 | 16QAM | 10 | 1/3 | 11784 | 24 | 1 | 2 | 35904 | 8976 |
|  | 100 | 60 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.8	CP-OFDM 64QAM

Table A.2.3.8-1: Reference channels for CP-OFDM 64QAM for 15kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 64QAM | 19 | 1/2 | 9992 | 24 | 1 | 2 | 19800 | 3300 |
|  | 10 | 15 | 52 | 11 | 64QAM | 19 | 1/2 | 21000 | 24 | 1 | 3 | 41184 | 6864 |
|  | 15 | 15 | 79 | 11 | 64QAM | 19 | 1/2 | 31752 | 24 | 1 | 4 | 62568 | 10428 |
|  | 20 | 15 | 106 | 11 | 64QAM | 19 | 1/2 | 42016 | 24 | 1 | 5 | 83952 | 13992 |
|  | 25 | 15 | 133 | 11 | 64QAM | 19 | 1/2 | 53288 | 24 | 1 | 7 | 105336 | 17556 |
|  | 30 | 15 | 160 | 11 | 64QAM | 19 | 1/2 | 63528 | 24 | 1 | 8 | 126720 | 21120 |
|  | 40 | 15 | 216 | 11 | 64QAM | 19 | 1/2 | 86040 | 24 | 1 | 11 | 171072 | 28512 |
|  | 50 | 15 | 270 | 11 | 64QAM | 19 | 1/2 | 108552 | 24 | 1 | 13 | 213840 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.8-2: Reference channels for CP-OFDM 64QAM for 30kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 11 | 11 | 64QAM | 19 | 1/2 | 4352 | 24 | 1 | 1 | 8712 | 1452 |
|  | 10 | 30 | 24 | 11 | 64QAM | 19 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 15 | 30 | 38 | 11 | 64QAM | 19 | 1/2 | 15112 | 24 | 1 | 2 | 30096 | 5016 |
|  | 20 | 30 | 51 | 11 | 64QAM | 19 | 1/2 | 20496 | 24 | 1 | 3 | 40392 | 6732 |
|  | 25 | 30 | 65 | 11 | 64QAM | 19 | 1/2 | 26120 | 24 | 1 | 4 | 51480 | 8580 |
|  | 30 | 30 | 78 | 11 | 64QAM | 19 | 1/2 | 31240 | 24 | 1 | 4 | 61776 | 10296 |
|  | 40 | 30 | 106 | 11 | 64QAM | 19 | 1/2 | 42016 | 24 | 1 | 5 | 83952 | 13992 |
|  | 50 | 30 | 133 | 11 | 64QAM | 19 | 1/2 | 53288 | 24 | 1 | 7 | 105336 | 17556 |
|  | 60 | 30 | 162 | 11 | 64QAM | 19 | 1/2 | 64552 | 24 | 1 | 8 | 128304 | 21384 |
|  | 80 | 30 | 217 | 11 | 64QAM | 19 | 1/2 | 86040 | 24 | 1 | 11 | 171864 | 28644 |
|  | 90 | 30 | 245 | 11 | 64QAM | 19 | 1/2 | 98376 | 24 | 1 | 12 | 194040 | 32340 |
|  | 100 | 30 | 273 | 11 | 64QAM | 19 | 1/2 | 108552 | 24 | 1 | 13 | 216216 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.8-3: Reference channels for CP-OFDM 64QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 11 | 11 | 64QAM | 19 | 1/2 | 4352 | 24 | 1 | 1 | 8712 | 1452 |
|  | 15 | 60 | 18 | 11 | 64QAM | 19 | 1/2 | 7168 | 24 | 1 | 1 | 14256 | 2376 |
|  | 20 | 60 | 24 | 11 | 64QAM | 19 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 25 | 60 | 31 | 11 | 64QAM | 19 | 1/2 | 12296 | 24 | 1 | 2 | 24552 | 4092 |
|  | 30 | 60 | 38 | 11 | 64QAM | 19 | 1/2 | 15112 | 24 | 1 | 2 | 30096 | 5016 |
|  | 40 | 60 | 51 | 11 | 64QAM | 19 | 1/2 | 20496 | 24 | 1 | 3 | 40392 | 6732 |
|  | 50 | 60 | 65 | 11 | 64QAM | 19 | 1/2 | 26120 | 24 | 1 | 4 | 51480 | 8580 |
|  | 60 | 60 | 79 | 11 | 64QAM | 19 | 1/2 | 31752 | 24 | 1 | 4 | 62568 | 10428 |
|  | 80 | 60 | 107 | 11 | 64QAM | 19 | 1/2 | 43032 | 24 | 1 | 6 | 84744 | 14124 |
|  | 90 | 60 | 121 | 11 | 64QAM | 19 | 1/2 | 48168 | 24 | 1 | 6 | 95832 | 15972 |
|  | 100 | 60 | 135 | 11 | 64QAM | 19 | 1/2 | 54296 | 24 | 1 | 7 | 106920 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.9	CP-OFDM 256QAM

Table A.2.3.9-1: Reference channels for CP-OFDM 256QAM for 15kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 256QAM | 20 | 2/3 | 17424 | 24 | 1 | 3 | 26400 | 3300 |
|  | 10 | 15 | 52 | 11 | 256QAM | 20 | 2/3 | 36896 | 24 | 1 | 5 | 54912 | 6864 |
|  | 15 | 15 | 79 | 11 | 256QAM | 20 | 2/3 | 55304 | 24 | 1 | 7 | 83424 | 10428 |
|  | 20 | 15 | 106 | 11 | 256QAM | 20 | 2/3 | 73776 | 24 | 1 | 9 | 111936 | 13992 |
|  | 25 | 15 | 133 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 140448 | 17556 |
|  | 30 | 15 | 160 | 11 | 256QAM | 20 | 2/3 | 112648 | 24 | 1 | 14 | 168960 | 21120 |
|  | 40 | 15 | 216 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 228096 | 28512 |
|  | 50 | 15 | 270 | 11 | 256QAM | 20 | 2/3 | 188576 | 24 | 1 | 23 | 285120 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.9-2: Reference channels for CP-OFDM 256QAM for 30kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 11 | 11 | 256QAM | 20 | 2/3 | 7680 | 24 | 1 | 1 | 11616 | 1452 |
|  | 10 | 30 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 15 | 30 | 38 | 11 | 256QAM | 20 | 2/3 | 26632 | 24 | 1 | 4 | 40128 | 5016 |
|  | 20 | 30 | 51 | 11 | 256QAM | 20 | 2/3 | 35856 | 24 | 1 | 5 | 53856 | 6732 |
|  | 25 | 30 | 65 | 11 | 256QAM | 20 | 2/3 | 46104 | 24 | 1 | 6 | 68640 | 8580 |
|  | 30 | 30 | 78 | 11 | 256QAM | 20 | 2/3 | 55304 | 24 | 1 | 7 | 82368 | 10296 |
|  | 40 | 30 | 106 | 11 | 256QAM | 20 | 2/3 | 73776 | 24 | 1 | 9 | 111936 | 13992 |
|  | 50 | 30 | 133 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 140448 | 17556 |
|  | 60 | 30 | 162 | 11 | 256QAM | 20 | 2/3 | 114776 | 24 | 1 | 14 | 171072 | 21384 |
|  | 80 | 30 | 217 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 229152 | 28644 |
|  | 90 | 30 | 245 | 11 | 256QAM | 20 | 2/3 | 172176 | 24 | 1 | 21 | 258720 | 32340 |
|  | 100 | 30 | 273 | 11 | 256QAM | 20 | 2/3 | 192624 | 24 | 1 | 23 | 288288 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.9-3: Reference channels for CP-OFDM 256QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 11 | 11 | 256QAM | 20 | 2/3 | 7680 | 24 | 1 | 1 | 11616 | 1452 |
|  | 15 | 60 | 18 | 11 | 256QAM | 20 | 2/3 | 12552 | 24 | 1 | 2 | 19008 | 2376 |
|  | 20 | 60 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 25 | 60 | 31 | 11 | 256QAM | 20 | 2/3 | 22032 | 24 | 1 | 3 | 32736 | 4092 |
|  | 30 | 60 | 38 | 11 | 256QAM | 20 | 2/3 | 26632 | 24 | 1 | 4 | 40128 | 5016 |
|  | 40 | 60 | 51 | 11 | 256QAM | 20 | 2/3 | 35856 | 24 | 1 | 5 | 53856 | 6732 |
|  | 50 | 60 | 65 | 11 | 256QAM | 20 | 2/3 | 46104 | 24 | 1 | 6 | 68640 | 8580 |
|  | 60 | 60 | 79 | 11 | 256QAM | 20 | 2/3 | 55304 | 24 | 1 | 7 | 83424 | 10428 |
|  | 80 | 60 | 107 | 11 | 256QAM | 20 | 2/3 | 75792 | 24 | 1 | 9 | 112992 | 14124 |
|  | 90 | 60 | 121 | 11 | 256QAM | 20 | 2/3 | 86040 | 24 | 1 | 11 | 127776 | 15972 |
|  | 100 | 60 | 135 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 142560 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

## A.3.3	DL reference measurement channels for TDD

### A.3.3.1	General

Table A.3.3.1-1 Additional reference channels parameters for TDD

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| UL-DL configuration | referenceSubcarrierSpacing | 15 kHz | 30 kHz | 60 kHz |
|  | dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
|  | nrofDownlinkSlots | 3 | 7 | 14 |
|  | nrofDownlinkSymbols | 10 | 6 | 12 |
|  | nrofUplinkSlot | 1 | 2 | 4 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
| Number of HARQ Processes |  | 8 | 8 | 16 |
| K1 value |  | K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} | K1 =8 if mod(i,10) = 0 K1 =7 if mod(i,10) = 1 K1 =6 if mod(i,10) = 2 K1 =5 if mod(i,10) = 3 K1 =4 if mod(i,10) = 4 K1 =3 if mod(i,10) = 5 K1 =2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} | K1 =15 if mod(i,20) = 0 K1 =14 if mod(i,20) = 1 K1 =13 if mod(i,20) = 2 K1 =12 if mod(i,20) = 3 K1 =11 if mod(i,20) = 4 K1 =10 if mod(i,20) = 5 K1 =9 if mod(i,20) = 6 K1 =8 if mod(i,20) = 7 K1 =7 if mod(i,20) = 8 K1 =6 if mod(i,20) = 9 K1 =5 if mod(i,20) = 10 K1 =4 if mod(i,20) = 11 K1 =3 if mod(i,20) = 12 K1 =2 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} |

### A.3.3.2	FRC for receiver requirements for QPSK

Table A.3.3.2-1 Fixed reference channel for receiver requirements (SCS 15 kHz, TDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | [160] | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 1,2,5,6,7 | Bits | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 1,2,5,6,7 | CBs | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 1,2,5,6,7 | Bits | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| Max. Throughput averaged over 1 frame | Mbps | 0.836 | 1.684 | 2.560 | 3.456 | 4.356 | 5.252 | 7.044 | 8.712 |
| Note 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 3: SS/PBCH block is transmitted in slot 0 of each frame Note 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Table A.3.3.2-2 Fixed reference channel for receiver requirements (SCS 30 kHz, TDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 273 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19} | Bits | 736 | 1608 | 2472 | 3368 | 4224 | 4992 | 6912 | 8712 | 10504 | 14088 | 17928 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19} | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19} | Bits | 2376 | 5184 | 8208 | 11016 | 14040 | 16848 | 22896 | 28728 | 34992 | 46872 | 58968 |
| Max. Throughput averaged over 1 frame | Mbps | 0.957 | 2.090 | 3.214 | 4.378 | 5.491 | 6.490 | 8.986 | 11.326 | 13.655 | 18.314 | 23.306 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.2-3 Fixed reference channel for receiver requirements (SCS 60 kHz, TDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 135 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {2,…,39} | Bits | 736 | 1192 | 1608 | 2024 | 2472 | 3368 | 4224 | 5120 | 6912 | 8712 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {2,…,39} | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {2,…,39} | Bits | 2376 | 3888 | 5184 | 6696 | 8208 | 11016 | 14040 | 17064 | 23112 | 29160 |
| Max. Throughput averaged over 1 frame | Mbps | 1.914 | 3.099 | 4.181 | 5.262 | 6.427 | 8.757 | 10.982 | 13.312 | 17.971 | 22.651 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |

### A.3.3.3	FRC for maximum input level for 64QAM

Table A.3.3.3-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, TDD, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | [160] | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 1,2,5,6,7 | Bits | 12296 | 25608 | 38936 | 52224 | 64552 | 77896 | 106576 | 131176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 1,2,5,6,7 | CBs | 2 | 4 | 5 | 7 | 8 | 10 | 13 | 16 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 1,2,5,6,7 | Bits | 16200 | 33696 | 51192 | 68688 | 86184 | 103680 | 139968 | 174960 |
| Max. Throughput averaged over 1 frame | Mbps | 6.148 | 12.804 | 19.468 | 26.112 | 32.276 | 38.948 | 53.288 | 65.588 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Table A.3.3.3-2 Fixed reference channel for maximum input level receiver requirements (SCS 30 kHz, TDD, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 273 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19} | Bits | 5376 | 11784 | 18432 | 25104 | 31752 | 37896 | 52224 | 64552 | 79896 | 106576 | 135296 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19} | CBs | 1 | 2 | 3 | 3 | 4 | 5 | 7 | 8 | 10 | 13 | 17 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19} | Bits | 7128 | 15552 | 24624 | 33048 | 42120 | 50544 | 68688 | 86184 | 104976 | 140616 | 176904 |
| Max. Throughput averaged over 1 frame | Mbps | 6.989 | 15.319 | 23.962 | 32.635 | 41.278 | 49.265 | 67.891 | 83.918 | 103.865 | 138.549 | 175.885 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.3-3. Fixed reference channel for maximum input level receiver requirements (SCS 60 kHz, TDD, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 135 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {2,…,39} | Bits | 5376 | 8712 | 11784 | 15112 | 18432 | 25104 | 31752 | 38936 | 52224 | 65576 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {2,…,39} | CBs | 1 | 2 | 2 | 2 | 3 | 3 | 4 | 5 | 7 | 8 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {2,…,39} | Bits | 7128 | 11664 | 15552 | 20088 | 24624 | 33048 | 42120 | 51192 | 69336 | 87480 |
| Max. Throughput averaged over 1 frame | Mbps | 13.978 | 22.651 | 30.638 | 39.291 | 47.923 | 65.270 | 82.555 | 101.234 | 135.782 | 170.498 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |

### A.3.3.4	FRC for maximum input level for 256 QAM

Table A.3.3.4-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, TDD, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | [160] | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS table for TBS determination |  | 256QAM |  |  |  |  |  |  |  |
| Modulation |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 1,2,5,6,7 | Bits | 16896 | 34816 | 53288 | 71688 | 90176 | 108552 | 143400 | 180376 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 1,2,5,6,7 | CBs | 3 | 5 | 7 | 9 | 12 | 14 | 18 | 23 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 1,2,5,6,7 | Bits | 21600 | 44928 | 68256 | 91584 | 114912 | 138240 | 186624 | 233280 |
| Max. Throughput averaged over 1 frame | Mbps | 8.448 | 17.408 | 26.644 | 35.844 | 45.088 | 54.276 | 71.700 | 90.188 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Table A.3.3.4-2 Fixed Reference channel for maximum input level receiver requirements (SCS 30 kHz, TDD, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 273 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination |  | 256QAM |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19} | Bits | 7424 | 16136 | 25608 | 33816 | 44040 | 52224 | 71688 | 90176 | 108552 | 147576 | 184424 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19} | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19} | Bits | 9504 | 20736 | 32832 | 44064 | 56160 | 67392 | 91584 | 114912 | 139968 | 187488 | 235872 |
| Max. Throughput averaged over 1 frame | Mbps | 9.651 | 20.977 | 33.290 | 43.961 | 57.252 | 67.891 | 93.194 | 117.229 | 141.118 | 191.849 | 239.751 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.4-3 Fixed reference channel for maximum input level receiver requirements (SCS 60 kHz, TDD, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 135 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination |  | 256QAM |  |  |  |  |  |  |  |  |  |
| Modulation |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {2,…,39} | Bits | 7424 | 12040 | 16136 | 21000 | 25608 | 33816 | 44040 | 53288 | 71688 | 90176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {2,…,39} | CBs | 1 | 2 | 3 | 3 | 4 | 5 | 6 | 7 | 9 | 12 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {2,…,39} | Bits | 9504 | 15552 | 20736 | 26784 | 32832 | 44064 | 56160 | 68256 | 92448 | 116640 |
| Max. Throughput averaged over 1 frame | Mbps | 19.302 | 31.304 | 41.954 | 54.600 | 66.581 | 87.922 | 114.504 | 138.549 | 186.389 | 234.458 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |