3GPP TSG-RAN WG4 Meeting #89	R4-1816645

Spokane, US, 12 - 16 Nov 2018

Agenda Item:	9.11.3

Source:	Dish Network, HNS

Title:	TP to TR38.819: UE aspect issues

Document for:	Approval

# 1	Introduction

This document is a text proposal to TR 38.819 summarizing the UE aspect issues for this band.

# 2	Text proposal

<<< Start of TP>>>

# 7	Study of NR specific issues

## 7.1	UE aspect issues

n65 has to protect PHS in Region 3, which is below n65 uplink. n1 same the same protection requirements, hence the same network signaling value and A-MPR are applied to n65.

n65 has to protect Band34, which is right above n65 uplink as shown in figure 5.1-1. A-MPR study is done using applicable NR trasnimmiter requirements. The following scenarios are simulated:

Figure 7.1-1 B34 protection scenarios

Simulation results provided by Qualcomm (R4-1812118):

Table 1: A-MPR for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rbend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | Rbend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | Rbend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | Fc=1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | Fc=1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | Fc=2002.5 |  | >2.16 | A1 | >3.78 | ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
| 10MHz | Fc=1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | Fc=1995 |  | >4.5 | A1 | >7.56 | ≤4.32 | A2 | ≤7.38 | ≤4.32 | A3 |
| 10MHz | Fc=2000 | >6.84 |  | A5 | <2.88 |  | A5 | ≥3.06 ≤6.66 | >1.44 | A6 |
| 15MHz | Fc=1987.5 |  | >7.02 | A1 | >11.52 | ≤6.84 | A2 | ≤11.34 | ≤6.84 | A3 |
| 15MHz | Fc=1997.5 | >9.36 |  | A5 | <3.6 |  | A5 | ≥3.78 ≤9.18 | >1.44 | A6 |
| 20MHz | Fc=1990 | >13.5 |  | A5 | <4.5 |  | A5 | ≥4.68 ≤13.32 | >2.16 | A6 |
| 20MHz | Fc=1995 | >12.6 |  | A5 | <5.4 |  | A5 | ≥5.58 ≤12.42 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 2. NOTE 3: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 11 | ≤ 5 |  | ≤ 8.5 | ≤ 20 | ≤ 10 |  |
| CP-OFDM QPSK | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 13 | ≤ 6.5 |  | ≤ 8.5 | ≤ 20 | ≤ 12 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |

Simulation results provided by Nokia (R4-1812051):

Table 3: Parameter

| CBW [MHz] | Offset from band edge [MHz] | Fc [MHz] | A-MPR |
| --- | --- | --- | --- |
| 5 | 15 | 1992,5 | A1 |
| 5 | 10 | 1997,5 | A2 |
| 5 | 5 | 2002,5 | A3 |
| 10 | 20 | 1985 | A4 |
| 10 | 10 | 1995 | A5 |
| 10 | 5 | 2000 | A6 |
| 15 | 15 | 1987,5 | A7 |
| 20 | 10 | 1990 | A8 |

Table 4: A-MPR

|  |  | A1 |  | A2 |  | A3 |  | A4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer |
| SC-FDMA | pi/2-BPSK | MPR | MPR | MPR | 2 | MPR | 8 | MPR | 1,5 |
|  | QPSK | MPR | MPR | MPR | 3,5 | 2 | 9 | MPR | 2,5 |
|  | 16QAM | MPR | MPR | MPR | 4 | 2 | 9,5 | MPR | 3 |
|  | 64QAM | MPR | MPR | MPR | 4 | MPR | 10 | MPR | 3 |
|  | 256QAM | MPR | MPR | MPR | 5 | 10 | 11 | MPR | MPR |
| OFDMA | QPSK | MPR | MPR | MPR | 4,5 | 4 | 11 | MPR | 4,5 |
|  | 16QAM | MPR | MPR | MPR | 4,5 | 3,5 | 11 | MPR | 4,5 |
|  | 64QAM | MPR | MPR | MPR | 5 | 4 | 10,5 | MPR | 4,5 |
|  | 256QAM | MPR | MPR | MPR | MPR | 10 | 11,5 | MPR | MPR |
|  |  | A5 |  | A6 |  | A7 |  | A8 |  |
|  |  | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer |
| SC-FDMA | pi/2-BPSK | MPR | 7 | 13 | 14,5 | MPR | 7 | 13 | 13 |
|  | QPSK | 1,5 | 8,5 | 13,5 | 15 | 1 | 8 | 14 | 14 |
|  | 16QAM | 2 | 9 | 15 | 15,5 | 1,5 | 8,5 | 15 | 14,5 |
|  | 64QAM | MPR | 9 | 15,5 | 15,5 | MPR | 8,5 | 15 | 14,5 |
|  | 256QAM | 5,5 | 9,5 | 16 | 16 | MPR | 9 | 16 | 15 |
| OFDMA | QPSK | 3,5 | 10,5 | 18 | 18 | 3,5 | 10 | 18 | 17 |
|  | 16QAM | 3,5 | 10 | 18 | 17,5 | 3,5 | 10 | 18 | 17,5 |
|  | 64QAM | MPR | 10,5 | 17,5 | 17,5 | MPR | 10 | 18,5 | 17,5 |
|  | 256QAM | MPR | 10,5 | 18,5 | 18 | MPR | 10 | 18,5 | 17,5 |

<<< End of TP>>>