3GPP TSG-RAN WG4 Meeting #89	R4-1816735

Spokane, Washington, USA 12 – 16 November 2018

Source:	Ericsson

Title:	TP to TS 38.141-2: Section 4.9.2.3 Data content for PHY channels

Agenda item:	7.9.4.2

Document for:	Approval

# 1	Introduction

Revised TP to TS 38.141-2 based upon agreements made during RAN4 #89.

Note in the text proposal, the notation “[xx211]” is used to allow the editor to insert the actual reference number once the reference is added.

# 2	References

- R4-1815277, “TS 38.141-2 v1.1.0”, Huawei RAN4#88B, Nov. 12-16, 2018

[Start of Text Proposal]

### 4.9.2.3	Data content of Physical channels and Signals for NR-TM

Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS 38.211 [xx211], subclause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of ‘0’ bits shall be generated prior to the scrambling.

Initialization of the scrambler and RE-mappers as defined in TS 38.211 [xx211] use the following additional parameters:

-	 = 0

-  Antenna ports starting with 1000 for PDSCH

- 	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	Rank 1 (single layer)

-   = 0

#### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [xx211], subclause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: 1(# of PDCCH)*1(# of CCE per PDCCH)* 6 (REG per CCE)*9(data RE per REG)*2(bits per RE) with these parameters according to the NR-FR2-TM definitions in subclause 4.9.2.2.

-	Generate this amount of bits according to ‘all 0’ data.

NsymbCORESET= 1

-	1 CCE shall be according to TS38.211 [xx211], clause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH should occupy first two symbols for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [xx211] , subclause 7.3.2.3.

-	NID=NIDcellin DM-RS sequence generation in TS 38.211 [xx211], subclause 7.4.1.3.

-	nRNTI = 0 in scrambling sequence generation in TS 38.211 [xx211], subclause 7.3.2.3.

-	nID=NIDcell

-	Perform mapping to REs according to TS 38.211 [xx211] , subclause 7.3.2.5.

#### 4.9.2.3.2		PDSCH

-	For each slot generate the required amount of bits for all PRBs according to ‘all 0’ data.

-	For  = 0  for 1 user PDSCH transmissions distinguished by .

-	Perform user specific scrambling according to TS 38.211 [xx211], subclause 7.3.1.1.

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [xx211], subclause7.3.1.2.

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [xx211], subclause 7.3.1.3.   Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is equal to 1.

-	Perform PDSCH mapping type A according to TS 38.211 [xx211].

-	DM-RS sequence generation according to TS 38.211 [xx211] , subclause 7.4.1.1.1 where l is the OFDM symbol number within the slot with symbols indicated by table 4.9.2.2-3.

-	DM-RS mapping according to TS 38.211 [xx211], subclause 7.4.1.1.2 with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS sequence generation according to TS 38.211 [xx211], subclause 7.4.1.2.1, with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS mapping according to TS 38.211 [xx211], subclause 7.4.1.2.2, with parameters listed in table 4.9.2.2-3.

[End of Text Proposal]