3GPP TSG-RAN WG4 Meeting #89	R4-1816753

# Spokane, WA, USA, 12 – 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38-101-3 | CR | <CR#> | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configured maximum output power for intra-band non-contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The configured maximum output power (Pcmax) for intra-band non-contiguous EN-DC is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Pcmax for intra-band non-contigous EN-DC is specified in acccordance with the specification for intra-band contiguous EN-DC but with parameters relevant for the intra-band non-contigous band combination verified. 6.2B.3.2: the relation between Pcmax and the A-MPR for non-contiguous intra-band contiguous combinations is clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The power control for intra-band non-contiguous EN-DC is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.1.2, 6.2B.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The proposed changes are based on the draft CR in R4-1814713 (intra-band contiguous EN-DC). |  |  |  |  |  |  |  |  |

< start of changes >

##### 6.2B.4.1.2	Intra-band non-contiguous EN-DC

< equations for Pcmax >

The following requirements apply for one component carrier per CG configured for synchronous DC. The CG(s) are indexed by j = 1 for MCG and j = 2 for SCG.

The configured maximum output power PCMAX,c for for serving cell c = c(1) of CG 1 (MCG) is set in accordance with sub-clause 6.2B.4.1.1 but where

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc determined in accordance with sub-clause 6.2B.3.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

The configured maximum output power PCMAX,c,f for serving cell c = c(2) with frequency f  on CG 2 (SCG) is set in accordance with sub-clause 6.2B.4.1.1 but where

-	for a UE indicating support of dynamicPowerSharing, A-MPRc = A-MPR with A-MPR determined in accordance with sub-clause 6.2B.3.2 and MPRc = 0 dB;

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with sub-clause 6.2B.3.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

For UEs not indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE, the total configured transmission power is determined in accordance with sub-clause 6.2B.4.1.1 but with Ppowerclass,EN-DC the EN-DC power class of the intra-band non-contiguous band combination configured.

For UEs indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE, the UE can configure the total transmission power in accordance with sub-clause 6.2B.4.1.1 but with Ppowerclass,EN-DC the EN-DC power class of the intra-band non-contiguous band combination configured and A-MPR determined in accordance with sub-clause 6.2B.3.2.

The total maximum output power PUMAX over both CGs is specified is measured in accordance with sub-clause 6.2B.4.1.1 and the measured maximum output power PUMAX in slot q on CG 2 shall be within the limits specified in sub-clause 6.2B.4.1.1 but with parameters applicable for the non-contiguous band combination configured.

##### 6.2B.4.1.3	Inter-band EN-DC within FR1

< text omitted >

#### 6.2B.3.2	Intra-band non-contiguous EN-DC

##### 6.2B.3.2.0	General

For intra-band EN-DC band combinations with additional requirements the A-MPR allowed are specified in Table 6.2B.3.1-1 for combinations of network signalling values indicated in E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR allowed below is in addition to the MPR requirements specified in sub-clause 6.2B.2.

Table 6.2B.3.2.0-1: Allowed power reduction for EN-DC

| DC configuration | Requirement (sub-clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (subclause) |
| --- | --- | --- | --- | --- |
| DC_41A_n411 | 6.6.3.3.19 and 6.6.2.2.2 of [4] and 6.5.2.3.2 and 6.5.3.3.1 of [2] | NS_04 | NS_04 | 6.2B.3.2.2 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The A-MPR is applied as MPR if NS_04 is not signalled. |  |  |  |  |

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with [36.101]

-	for the SCG, A-MPRc = A-MPR

-	for the total configured transmission power, A-MPRtot = A-MPR

with A-MPR as defined in this sub-clause.

[If the configured transmission power spectral density between the MCG and SCG differs by more than [6] dB, then A-MPRc is further modified

A-MPRc = [Infinity].]

##### 6.2B.3.2.1	A-MPR for NS_04

< end of changes >