3GPP TSG-RAN WG4 Meeting #90												  	         R4-1900055

Athens, Greece, 25th of February – 1st of March, 2019

Source: 	Nokia

Title: 					TP to TR 38.173: n48 A-MPR

Agenda item:			9.12.1

For:						Approval

# 1	Introduction

This contribution presents A-MPR simulation results for n48 and proposes a TP to capture the study into the TR.

# 2	Discussion

## 2.1	Requirements

FCC has released new CBRS UE requirements as follows.

(ii) Except as otherwise specified in paragraph (e)(2) of this section, for channel and frequency assignments made by a CBSD to End User Devices, the conducted power of any End User Device emission outside the fundamental emission (whether in or outside of the authorized band) shall not exceed -13 dBm/MHz within 0 to B megahertz (where B is the bandwidth in megahertz of the assigned channel or multiple contiguous channels of the End User Device) above the upper CBSD-assigned channel edge and within 0 to B megahertz below the lower CBSD-assigned channel edge. At all frequencies greater than B megahertz above the upper CBSD assigned channel edge and less than B megahertz below the lower CBSD-assigned channel edge, the conducted power of any End User Device emission shall not exceed -25 dBm/MHz. Notwithstanding the emission limits in this paragraph, the Adjacent Channel Leakage Ratio for End User Devices shall be at least 30 dB.

(2) Additional protection levels. Notwithstanding paragraph (e)(1) of this section, for CBSDs and End User Devices, the conducted power of emissions below 3540 MHz or above 3710 MHz shall not exceed -25 dBm/MHz, and the conducted power of emissions below 3530 MHz or above 3720 MHz shall not exceed −40dBm/MHz.

(3) Measurement procedure. (i) Compliance with this provision is based on the use of measurement instrumentation employing a resolution bandwidth of 1 megahertz or greater. However, in the 1 megahertz bands immediately outside and adjacent to the licensee’s authorized frequency channel, a resolution bandwidth of no less than one percent of the fundamental emission bandwidth may be employed. A narrower resolution bandwidth is permitted in all cases to improve measurement accuracy provided the measured power is integrated over the full reference bandwidth (i.e., 1 MHz or 1 percent of emission bandwidth, as specified). The fundamental emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.

Figure 1: CBRS UE requirements

These rules can be presented as follows:

n48 3550 MHz – 3700 MHz 3550 MHz – 3700 MHz TDD

Table 3: SEM

|  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | Measurement bandwidth |
| ± 0 - 1 | -13 |  |  |  |  | 1 % channel bandwidth |
| ± 1 - X | -13 |  |  |  |  | 1 MHz |
| < – X or > X when 3540 MHz < ΔfOOB < 3710 MHz | -25 |  |  |  |  |  |
| NOTE: X is occupied channel bandwidth as defined in Table 6.5.1-1 |  |  |  |  |  |  |

Table 4: Additional Spurious emissions

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 40 MHz |  |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

Table 5: UL CH BWs

| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n48 | 15 | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |

## 2.2	A-MPR concept and results

As can be seen from Figure 1 the emission requirements are tighter outside of the band compared to inside the band with an exception that first 10 MHz on high side of that band has same -13 dBm requirement as within the band. Therefore, it would be beneficial to define two different A-MPRs firstly the band edge A-MPR which has higher -25 dBm...-40 dBm emission requirement on IMD3 region and secondly an inner-band A-MPR which would have lower A-MPR as -13 dBm requirement would apply on IMD3 region. This concept is presented in Figure.

Figure 2: Two different A-MPRs per CH BW

How these two sets of A-MPR could be captured into the specification is presented in table 6 and 7 below.

Table 6: A-MPR regions for n48

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 5 MHz | 3552.5 ≤ FC ≤ 3697.5 | A1 |
| 10 MHz | 3555 ≤ FC ≤ 3695 | A2 |
| 15 MHz | 3557.5 ≤ FC < 3562.5 3687.5 < FC ≤ 3692.5 | A3 |
| 15 MHz | 3562.5 ≤ FC < 3687.5 | A4 |
| 20 MHz | 3560 ≤ FC < 3570 3680 < FC ≤ 3690 | A5 |
| 20 MHz | 3570 ≤ FC < 3680 | A6 |
| 40 MHz | 3570 ≤ FC < 3600 3650 < FC ≤ 3680 | A7 |
| 40 MHz | 3600 ≤ FC < 3650 | A8 |

Total UE output power backoff is max(MPR, A-MPR).

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |
| --- | --- | --- | --- |
|  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  | 0.52 | 0.52 | 02 |
| DFT-s-OFDM QPSK | ≤ 1 |  | 0 |
| DFT-s-OFDM 16 QAM | ≤ 2 |  | ≤ 1 |
| DFT-s-OFDM 64 QAM | ≤ 2.5 |  |  |
| DFT-s-OFDM 256 QAM | 4.5 |  |  |
| CP-OFDM QPSK | ≤ 3 |  | ≤ 1.5 |
| CP-OFDM 16 QAM | ≤ 3 |  | ≤ 2 |
| CP-OFDM 64 QAM | ≤ 3.5 |  |  |
| CP-OFDM 256 QAM | ≤ 6.5 |  |  |

the simulated A-MPR in Table 7.

Table 7: n48 A-MPR

|  |  | 5 MHz Edge |  | 10 MHz Edge |  | 15 MHz Edge |  | 15 MHz Center |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A1 |  | A2 |  | A3 |  | A4 |  |  |  |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |  | IMD3 and IMD5 --> -13 |
| SC-FDMA | pi/2-BPSK | - | - | - | - | 1,5 | 1,5 | - | - |  | IMD3 -> -13 and IMD5 -> -25 |
|  | QPSK | - | - | - | - | 3 | 2 | - | - |  | IMD3 -> -25 and IMD5 -> -40 |
|  | 16QAM | - | - | - | - | 3 | 2,5 | - | - |  | IMD3 -> -13 and IMD5 -> -40 |
|  | 64QAM | - | - | - | - | 3 | - | - | - |  | IMD3 -> -40 |
|  | 256QAM | - | - | - | - | - | - | - | - |  |  |
| OFDMA | QPSK | - | - | - | - | 4 | 3 | - | - |  |  |
|  | 16QAM | - | - | - | - | 4 | 3 | - | - |  |  |
|  | 64QAM | - | - | - | - | 4 | - | - | - |  |  |
|  | 256QAM | - | - | - | - | - | - | - | - |  |  |
|  |  | 20 MHz Edge |  | 20 MHz Center |  | 40 MHz Edge |  | 40 MHz Center |  |  |  |
|  |  | A5 |  | A6 |  | A7 |  | A8 |  |  |  |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |  |  |
| SC-FDMA | pi/2-BPSK | 3 | 1,5 | - | - | 8 | 8 | 1 | - |  |  |
|  | QPSK | 3,5 | 2 | 2 | - | 8 | 8 | 2 | - |  |  |
|  | 16QAM | 4,5 | 2,5 | - | - | 8,5 | 8,5 | 2,5 | - |  |  |
|  | 64QAM | 4,5 | - | - | - | 8 | 8,5 | - | - |  |  |
|  | 256QAM | 5 | - | - | - | 8 | 8,5 | - | - |  |  |
| OFDMA | QPSK | 6 | 3,5 | - | - | 8,5 | 8,5 | - | - |  |  |
|  | 16QAM | 6 | 3,5 | - | - | 8,5 | 9 | - | - |  |  |
|  | 64QAM | 6 | - | - | - | 8,5 | 9 | - | - |  |  |
|  | 256QAM | - | - | - | - | 8,5 | 8 | - | - |  |  |

As can be seen from results presented in Table 7 there is no need to specify A-MPR for A1, A2, A4, A6 inner and A8 inner scenarios. A-MPR need to A6 and A8 outer is limited to only few modulations. For A5 there is an A-MPR need for many modulations but additive backoff compared to MPR is 1.5 – 3 dB. A8 requires always substantial A-MPR.

As an example few A-MPR results are presented in Figures 3 and 4.

Figure 3: 20 MHz A-MPR																	Figure 4: 40 MHz A-MPR

# 3	Conclusion

In this contribution we present our A-MPR simulation results for n48 and have associated TP.

****************************** Start of the TP **********************************

## 7.1.1	A-MPR

#### 7.1.1.1	Vendor A

Simulation assumptions were as follows

IQ-Image and LO leakage = 28 dBc

CIM3 = 60 dBc

PA calibration point was 20 MHz, 15 kHz, QPSK, DFT-S-OFMA, 100 RB at lower channel edge with 0.5 dB MPR

A-MPR concept and results

FCC emission requirements are tighter outside of the band compared to inside the band with an exception that first 10 MHz on high side of that band has same -13 dBm requirement as within the band. Therefore, it would be beneficial to define two different A-MPRs firstly the band edge A-MPR which has higher -25 dBm...-40 dBm emission requirement on IMD3 region and secondly an inner-band A-MPR which would have lower A-MPR as -13 dBm requirement would apply on IMD3 region. This concept is presented in Figure.

Figure 7.1.1.1-1: Two different A-MPRs per CH BW

Based on concept presented in Figure 7.1.1.1-1 we provide simulation results in Tables 7.1.1.1-1 and 7.1.1.1-2 below. Total UE output power backoff is max(MPR, A-MPR).

Table 7.1.1.1-1: A-MPR regions for n48

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 5 MHz | 3552.5 ≤ FC ≤ 3697.5 | A1 |
| 10 MHz | 3555 ≤ FC ≤ 3695 | A2 |
| 15 MHz | 3557.5 ≤ FC < 3562.5 3687.5 < FC ≤ 3692.5 | A3 |
| 15 MHz | 3562.5 ≤ FC < 3687.5 | A4 |
| 20 MHz | 3560 ≤ FC < 3570 3680 < FC ≤ 3690 | A5 |
| 20 MHz | 3570 ≤ FC < 3680 | A6 |
| 40 MHz | 3570 ≤ FC < 3600 3650 < FC ≤ 3680 | A7 |
| 40 MHz | 3600 ≤ FC < 3650 | A8 |

Table 7.1.1.1-2: n48 A-MPR

|  |  | 5 MHz Edge |  | 10 MHz Edge |  | 15 MHz Edge |  | 15 MHz Center |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A1 |  | A2 |  | A3 |  | A4 |  |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |
| SC-FDMA | pi/2-BPSK | - | - | - | - | 1,5 | 1,5 | - | - |
|  | QPSK | - | - | - | - | 3 | 2 | - | - |
|  | 16QAM | - | - | - | - | 3 | 2,5 | - | - |
|  | 64QAM | - | - | - | - | 3 | - | - | - |
|  | 256QAM | - | - | - | - | - | - | - | - |
| OFDMA | QPSK | - | - | - | - | 4 | 3 | - | - |
|  | 16QAM | - | - | - | - | 4 | 3 | - | - |
|  | 64QAM | - | - | - | - | 4 | - | - | - |
|  | 256QAM | - | - | - | - | - | - | - | - |
|  |  | 20 MHz Edge |  | 20 MHz Center |  | 40 MHz Edge |  | 40 MHz Center |  |
|  |  | A5 |  | A6 |  | A7 |  | A8 |  |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |
| SC-FDMA | pi/2-BPSK | 3 | 1,5 | - | - | 8 | 8 | 1 | - |
|  | QPSK | 3,5 | 2 | 2 | - | 8 | 8 | 2 | - |
|  | 16QAM | 4,5 | 2,5 | - | - | 8,5 | 8,5 | 2,5 | - |
|  | 64QAM | 4,5 | - | - | - | 8 | 8,5 | - | - |
|  | 256QAM | 5 | - | - | - | 8 | 8,5 | - | - |
| OFDMA | QPSK | 6 | 3,5 | - | - | 8,5 | 8,5 | - | - |
|  | 16QAM | 6 | 3,5 | - | - | 8,5 | 9 | - | - |
|  | 64QAM | 6 | - | - | - | 8,5 | 9 | - | - |
|  | 256QAM | - | - | - | - | 8,5 | 8 | - | - |

As can be seen from results presented in Table 7.1.1.1-2 our results indicate there is no need to specify A-MPR for A1, A2, A4, A6 inner and A8 inner scenarios. A-MPR needed for A6 and A8 outer allocations is limited to only few modulations. For A5 there is an A-MPR need for many modulations but additive backoff compared to MPR is 1.5 – 3 dB. A8 requires always substantial A-MPR.

****************************** Start of the TP **********************************