3GPP TSG-RAN WG4 Meeting #90												  	         R4-1900090

Athens, Greece, 25th of February – 1st of March, 2019

Source: 	Nokia, TMO US

Title: 					TP to TR 38.716-02-00 CA_n71-n261

Agenda Item: 		9.2.3

Document for:	Approval

# 1	Introduction

This TP introduces the following NR Carrier Aggregation CA_n71-n261 into the TR.

| Combination | # CC for DL | CA configurations | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes and intra contiguous/non-contiguous CA (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71-n261 | 2 | DL_n71A-n261A_ UL_n71A_BCS0 | Rel-15 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | None |
| n71-n261 | 2 | DL_n71A-n261A_ UL_n261A_BCS0 | Rel-15 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | None |
| n71-n261 | 3 | DL_n71A-n261(2A)_ UL_n71A_BCS0 | Rel-15 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | 2B_2CC_DL_n71A-n261A_1CC_UL_n71A_BCS0 (new) |
| n71-n261 | 3 | DL_n71A-n261(2A)_ UL_n261A_BCS0 | Rel-15 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom | New | 2B_2CC_DL_n71A-n261A_1CC_UL_n261A_BCS0 (new) |

*********************** Start of the TP ***************************************************

## 6.X	CA_n71_n261

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: CA band combination of band n71 + n261

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_ n71-n261 | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

Table 6.x.1-2: Supported bandwidths per CA band combination of CA_n71A-n261A

| CA operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| CA_n71A-n261A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| CA_n71A-n261(2A) | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n261 | See CA_n261(2A) BCS 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.2	UE co-existence studies

Table 6.x.2-1 gives the UL 2nd, 3rd, 4th, 5th, 6th, 7th harmonic for CA_n71A-n261A.

Table 6.x.2-1: Band n71 and Band n261 UL harmonics products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 27500 | 28350 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 55000 | 56700 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 82500 | 85050 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 110000 | 113400 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 137500 | 141750 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 3978 | 4188 | 165000 | 170100 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4641 | 4886 | 192500 | 198450 |

From table 6.X.2-1 it can be seen that UL harmonic frequencies of Band n71 and Band n261 do not locate within the UE’s own receive bands, therefore it can be concluded that there is no issue on harmonic interference.

### 6.x.3	∆TIB and ∆RIB values

For CA_n71A-n261A, the TIB,c and RIB,c values are given in the tables below.

Table 6.x.3-1: ΔTIB,c

| NR inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n71A-n261A | n71 | 0 |
|  | n261 | 0 |

Table 6.x.3-2: ΔRIB,c

| NR inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n71A-n261A | n71 | 0 |
|  | n261 | 0 |

### 6.x.4	REFSENs requirements

There is no specific REFSENs requirement.

*********************** End of the TP ***************************************************