3GPP TSG-RAN WG4 Meeting #90	R4- 1900417

Athens, Greece, 25th Feb – 1st March, 2019

Agenda item:	5.6.6.2

Source: 	Qualcomm Incorporated

Title: 	On sTTI CQI Fading Tests

Document for:	Discussion

# 1	Introduction

RAN4#89 agreed with the simulation assumptions and test metrics of CQI reporting under fading condition test [1]. This contribution discusses the SNR test points and requirements of sTTI CQI reporting test for both CRS-based and DMRS-based PDSCH.

# 2	CRS-based PDSCH

## 2.1	Test points

The test metrics of the existing wideband CQI reporting requirements are specified as follows:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to δ

In [2], Ericsson suggested testing points of {1,2} and {9,10}. We present in Table 1 to 3 the test metrics of our simulation results in these test points.

Table 1	Simulation results of wideband CQI with FDD slot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| 1 | 5 | 0.38 | 0.31 | 1.34 |
| 2 | 6 | 0.37 | 0.35 | 1.35 |
| 9 | 9 | 0.37 | 0.25 | 1.2 |
| 10 | 10 | 0.38 | 0.38 | 1.31 |

Table 2	Simulation results of wideband CQI with FDD subslot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| 1 | 7 | 0.40 | 0.42 | 1.32 |
| 2 | 7 | 0.41 | 0.32 | 1.28 |
| 9 | 11 | 0.38 | 0.39 | 1.25 |
| 10 | 12 | 0.38 | 0.51 | 1.43 |

Table 3	Simulation results of wideband CQI with TDD slot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| 1 | 5 | 0.38 | 0.33 | 1.36 |
| 2 | 6 | 0.37 | 0.38 | 1.39 |
| 9 | 9 | 0.37 | 0.27 | 1.22 |
| 10 | 10 | 0.38 | 0.41 | 1.34 |

Observation 1: For CRS-based sTTI PDSCH, CQI requirements are satisfied for two sets of test points {1, 2} and {9, 10}.

Proposal 1: For CRS-based PDSCH, set two sets of test points {1, 2} and {9, 10} for both slot-based and subslot-based wideband CQI reporting test (same as Ericsson proposal in [2]).

# 3	DMRS-based PDSCH

## 2.1	Test points

The test metrics of the existing wideband CQI reporting requirements are specified as follows:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to δ

As discussed in [2], we should avoid setting SNR test points whose CQI indexes result in large coding difference between target and actual one. For subslot-PDSCH lower SNR test point set, we suggest selecting the SNR points corresponding to CQI index 5, since the corresponding SNR points {4,5} still lie in reasonable SNR range.

Table 4 shows our simulation results of wideband CQI reporting with subslot-PDSCH.

Proposal 2: For DMRS-based PDSCH, set one set of test points {4, 5} for subslot-based wideband CQI reporting test.

Table 6	Simulation results of wideband CQI with subslot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| 4 | 5 | 0.37 | 0.34 | 1.33 |
| 5 | 5 | 0.38 | 0.24 | 1.34 |

# 4	Conclusion

Proposal 1: For CRS-based PDSCH, set two sets of test points {1, 2} and {9, 10} for both slot-based and subslot-based wideband CQI reporting test (same as Ericsson proposal in [2]).

Proposal 2: For DMRS-based PDSCH, set one set of test points {4, 5} for subslot-based wideband CQI reporting test.

# 5	References

- R4-1816325, “Introduction of CQI tests for sTTI”, Ericsson.

- R4-1815224, “sTTI CQI reporting tests”