3GPP TSG-RAN WG4 RAN4#90 Meeting	R4-1900574

Athens, GR, 25th Feb – 1st March, 2019

Source:	KDDI

Title:	Work Plan for NR band n18

Agenda item:	9.13.4

Document for:	Discussion

Introduction

This paper is submitted to share common understanding in RAN4 of work plan regarding “Introduction of NR band n18” which was approved in RAN#82 [1].  Target complete meeting was set to RAN#82 (December, 2019) in WID [1] but we will try to complete it as soon as possible for avoiding RAN4 work congestion.

Work Plan and required work

Impacted sub-clause for each Technical Specifications

Following tables summarize what shall be specified. Comments and off-line discussions are welcome.

Table 2.1-1: Affected section in TS38.101-1

| Section | Title | Work |
| --- | --- | --- |
| 5.2 | Operating bands | Add n18 in Table 5.2-1 |
| 5.3.5 | UE channel bandwidth per operating band | Add channel bandwidth of n18 in Table 5.3.5-1 |
| 5.4.2.3 | Channel raster entries for each operating band | Add channel raster of n18 in Table 5.4.2.3-1 |
| 5.4.3.3 | Synchronization raster entries for each operating band | Add ss raster entries of n18 in Table 5.4.3.3-1 |
| 5.4.4 | TX–RX frequency separation | Add default UE TX-RX frequency separation of n18 in Table 5.4.4-1 |
| 6.2.1 | UE maximum output power | Add UE power class in Table 6.2.1-1 |
| 6.2.3 | UE additional maximum output power reduction | Add n18 with NS_01 in Table 6.2.3.1-1A: Mapping of Network Signaling label |
| 6.5.3.2 | Spurious emissions for UE co-existence | Add spurious emission requirements for n18 in Table 6.5.3.2-1 |
| 7.3.2 | Reference sensitivity power level | Add reference sensitivity and uplink configuration for reference sensitivity of n18 in Table 7.3.2-1 and Table 7.3.2-3 respectively |
| 7.6.4 | Narrow band blocking | Add n18 in Table 7.6.4-1: Narrow Band Blocking |

Table 2.1-2: Affected section in TS38.104

| Section | Title | Work |
| --- | --- | --- |
| 5.2 | Operating bands | Add n18 in Table 5.2-1 |
| 5.3.5 | BS channel bandwidth per operating band | Add channel bandwidth of n18 in Table 5.3.5-1 |
| 5.4.2.3 | Channel raster entries for each operating band | Add channel raster of n18 in Table 5.4.2.3-1 |
| 5.4.3.3 | Synchronization raster entries for each operating band | Add ss raster entries of n18 in Table 5.4.3.3-1 |
| 6.6.5.2.3 | Additional spurious emissions requirements | Add “NR band n18” in Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands |
| 6.6.5.2.4 | Co-location with other base stations | Add “NR band n18” in Table 6.6.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS |

Table 2.1-3: Affected section in TS38.141-1

| Section | Title | Work |
| --- | --- | --- |
| 6.6.4.5.2 | Basic limits for Wide Area BS (Category A) | Add n18. |
| 6.6.4.5.3.1 | Category B requirements (Option 1) | Add n18. |
| 6.6 | Unwanted emissions | Add “NR band n18” in Table 6.6.5.5.1.3-1: BS spurious emissions limits for BS for co-existence with systems operating in other frequency bands and Table 6.6.5.5.1.4-1: BS spurious emissions basic limits for BS co-located with another BS. |

Table 2.1-4: Affected section in TS38.141-2

| Section | Title | Work |
| --- | --- | --- |
| 6.7.4.5.1.1 | Wide Area BS (Category A) | Add n18. |
| 6.7.4.5.1.2 | Wide Area BS Category B (Option 1) | Add n18. |
| 6.7.5 | OTA transmitter spurious emissions | Add “NR band n18” in Table 6.7.5.4.5-1: BS spurious emissions test limits for BS for co-existence with systems operating in other frequency bands and Table 6.7.5.5.5.1-1: BS type 1-O OTA spurious emissions limits for BS co-located with another BS, |

Table 2.1-5: Affected section in TS38.133

| Section | Title | Work |
| --- | --- | --- |
| 3.5.2 | NR operating bands in FR1 | Add n18 in Table 3.5.2-1: NR frequency band groups for FR1. |

Summary

In this contribution, we provide our work plan for“Introduction of NR band n18”.  As indicated above, most of impacted requirements in each TS are not so contentious; namely just describe “n18” in each TS.  Therefore, we will try to complete this WI hopefully by RAN#84 (June 2019) or by RAN#85 (September 2019) at the latest.

References

[1] RP-182826, “Introduction of NR band n18”, KDDI, et al