3GPP TSG-RAN4 Meeting #90	R4-1900684

# Athens, Greece, 25th Feb – 1st Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on SRS Antenna Switching Interruption requirements (section 8.2.1.2.9, section 8.2.2.2.7) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No SRS antenna switching interruption in R4 spec.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new interruption section for SRS antenna switching interruption. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will be no requirements for SRS antenna switching interruption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.9, 8.2.2.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

8.2.1.2.9	Interruptions at SRS antenna switching

When a UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE needs to transmit periodic, semi-persistent or aperiodic SRS configured by higher layer parameter resourceType on the antenna ports. Interruption on other serving cells is allowed depending on the indicated UE capability supportedSRS-TxPortSwitch, provided that:

-	the SRS antenna switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or indicated by MAC-CE for semi-persistent SRS transmission or configured via RRC [2] for periodic SRS transmission;

-	the serving cell(s) whose DL reception or UL transmission may therefore be interrupted, are indicated by UE capability reported txSwitchImpactToRx and txSwitchWithAnotherBand [2];

-	the SRS antenna switching is not colliding with any other transmission with higher priority defined in TS38.214.

-	for UE, which does not support simultaneous reception and transmission for inter-band CA specified in TS 38.331 [2], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101 [5], the SRS transmission are not simultaneously scheduled with DL SSB transmission on other carriers.

The UE shall not perform SRS antenna switching if the above conditions cannot be met.

The interruption of up to the duration X6 shown in table 8.2.1.2.9-1 during the antenna switching.

Table 8.2.1.2.9-1: Interruption length X6 at SRS antenna switching

|  | Victim cell SCS(KHz) | Aggressor Cell SCS (KHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15 | 30 | 60 | 120 |
| 0 | 15 | 2 | 2 | 2 | 2 |
| 1 | 30 | 2 | 2 | 2 | 2 |
| 2 | 60 | 3 | 2 | 2 | 2 |
| 3 | 120 | 5 | 3 | 3 | 2 |

<< End of Change #1>>

<< Start of Change #2>>

8.2.2.2.7	Interruptions at SRS antenna switching

When a UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE needs to transmit periodic, semi-persistent or aperiodic SRS configured by higher layer parameter resourceType on the antenna ports. Interruption on other serving cells is allowed depending on the indicated UE capability supportedSRS-TxPortSwitch, provided that:

-	the SRS antenna switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission or indicated by MAC-CE for semi-persistent SRS transmission or configured via RRC [2] for periodic SRS transmission;

-	the serving cell(s) whose DL reception or UL transmission may therefore be interrupted, are indicated by UE capability reported txSwitchImpactToRx and txSwitchWithAnotherBand [2];-	the SRS antenna switching is not colliding with any other transmission with higher priority defined in TS38.214.

-	for UE, which does not support simultaneous reception and transmission for inter-band CA specified in TS 38.331 [2], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101 [5], the SRS transmission are not simultaneously scheduled with DL SSB transmission on other carriers.

The UE shall not perform SRS antenna switching if the above conditions cannot be met.

The interruption of up to the duration is shown in table 8.2.2.2.7-1 during the antenna switching.

Table 8.2.2.2.7-1: Interruption duration at SRS antenna switching

|  | Victim cell SCS(KHz) | Aggressor Cell SCS (KHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15 | 30 | 60 | 120 |
| 0 | 15 | 2 | 2 | 2 | 2 |
| 1 | 30 | 2 | 2 | 2 | 2 |
| 2 | 60 | 3 | 2 | 2 | 2 |
| 3 | 120 | 5 | 3 | 3 | 2 |

<< End of Change #2>>