3GPP TSG-RAN WG4 Meeting #90	R4-1900757

Athens, GR, 25 Feb - 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability rule for BS demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The applicability rule for BS demodulation requirements was not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the applicability rule for BS demodulation requirements based on RAN4 agreements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No need to introduce applicability rule for BS demodulation requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2, 11.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-1, TS 38.141-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Start of first change >

### 8.1.2	Applicability rule

8.1.2.1 	Applicability of PUSCH performance requirements

8.1.2.1.1	Applicability of requirements for different subcarrier spacings

The tests for a specific subcarrier spacing are only applicable if the BS supports it.

For a BS supporting multiple subcarrier spacings, the tests for all the subcarrier spacings supported by the BS are applicable.

8.1.2.1.2	Applicability of requirements for different channel bandwidths

For each subcarrier spacing supported by the BS, the tests for a specific channel bandwidth are only applicable if the BS supports it.

For a BS supporting multiple channel bandwidths for one subcarrier spacing, only the tests for the highest channel bandwidth supported by the BS are applicable. If the performance requirements for the highest channel bandwidth is not defined in this document, the tests for the nearest lower channel bandwidth (i.e., test bandwidth) w.r.t. the highest channel bandwidth are appliabiliable, and the test bandwidth are placed in the middle of the channel bandwidth during the test.

8.1.2.1.3	Applicability of requirements for carrier aggregation

Editor’s note: The applicability of CA requirements will be added.

8.1.2.1.4	Applicability of requirements for different configurations

Editor’s note: For the requirements with different configurations on additional DMRS and mapping type, the applicability will be added.

8.1.2.2 	Applicability of PUCCH performance requirements

8.1.2.2.1	Applicability of requirements for different formats

The tests for a specific PUCCH format are only applicable if the BS supports it.

For a BS supporting multiple PUCCH formats, the tests for all the PUCCH formats supported by the BS are applicable.

8.1.2.2.2	Applicability of requirements for different subcarrier spacings

The tests for a specific subcarrier spacing are only applicable if the BS supports it.

For a BS supporting multiple subcarrier spacings, the tests for all the subcarrier spacings supported by the BS are applicable.

8.1.2.2.3	Applicability of requirements for different channel bandwidths

For each subcarrier spacing supported by the BS, the tests for a specific channel bandwidth are only applicable if the BS supports it.

For a BS supporting multiple channel bandwidths for one subcarrier spacing, only the tests for the highest channel bandwidth supported by the BS are applicable. If the performance requirements for the highest channel bandwidth is not defined in this document, the tests for the nearest lower channel bandwidth (i.e., test bandwidth) w.r.t. the highest channel bandwidth are appliabiliable, and the test bandwidth are placed in the middle of the channel bandwidth during the test.

8.1.2.2.4	Applicability of requirements for different configurations

Editor’s note: For the requirements with different configurations on additional DMRS, the applicability will be added.

8.1.2.3 	Applicability of PRACH performance requirements

8.1.2.3.1	Applicability of requirements for different formats

The tests for a specific PRACH format are only applicable if the BS supports it.

For a BS supporting multiple PRACH formats, the tests for all the PRACH formats supported by the BS are applicable.

8.1.2.3.2	Applicability of requirements for different subcarrier spacings

For PRACH with short sequence, the tests for a specfic subcarrier spacing are only applicable if the BS supports it.

For a BS supporting multiple subcarrier spacings for a PRACH format with short sequence, for each FR, the tests for [smallest] subcarrier spacing supported by the BS are applicable.

8.1.2.3.3	Applicability of requirements for different channel bandwidths

PRACH performance requirements are defined a channel bandwidth agnostic way, and the tests for any one channel bandwidth supported by the BS are applicable.

< End of first change >

< Start of second change >

11.1.3	Applicability rule

11.1.3.1 	Applicability of PUSCH performance requirements

11.1.3.1.1	Applicability of requirements for different subcarrier spacings

The tests for a specific subcarrier spacing are only applicable if the BS supports it.

For a BS supporting multiple subcarrier spacings, the tests for all the subcarrier spacings supported by the BS are applicable.

11.1.3.1.2	Applicability of requirements for different channel bandwidths

For each subcarrier spacing supported by the BS, the tests for a specific channel bandwidth are only applicable if the BS supports it.

For a BS supporting multiple channel bandwidths for one subcarrier spacing, only the tests for the highest channel bandwidth supported by the BS are applicable. If the performance requirements for the highest channel bandwidth is not defined in this document, the tests for the nearest lower channel bandwidth (i.e., test bandwidth) w.r.t. the highest channel bandwidth are appliabiliable, and the test bandwidth are placed in the middle of the channel bandwidth during the test.

11.1.3.1.3	Applicability of requirements for carrier aggregation

Editor’s note: The applicability of CA requirements will be added.

11.1.3.1.4	Applicability of requirements for different configurations

Editor’s note: For the requirements with different configurations on additional DMRS and mapping type, the applicability will be added.

11.1.3.2 	Applicability of PUCCH performance requirements

11.1.3.2.1	Applicability of requirements for different formats

The tests for a specific PUCCH format are only applicable if the BS supports it.

For a BS supporting multiple PUCCH formats, the tests for all the PUCCH formats supported by the BS are applicable.

11.1.3.2.2	Applicability of requirements for different subcarrier spacings

The tests for a specific subcarrier spacing are only applicable if the BS supports it.

For a BS supporting multiple subcarrier spacings, the tests for all the subcarrier spacings supported by the BS are applicable.

11.1.3.2.3	Applicability of requirements for different channel bandwidths

For each subcarrier spacing supported by the BS, the tests for a specific channel bandwidth are only applicable if the BS supports it.

For a BS supporting multiple channel bandwidths for one subcarrier spacing, only the tests for the highest channel bandwidth supported by the BS are applicable. If the performance requirements for the highest channel bandwidth is not defined in this document, the tests for the nearest lower channel bandwidth (i.e., test bandwidth) w.r.t. the highest channel bandwidth are appliabiliable, and the test bandwidth are placed in the middle of the channel bandwidth during the test.

11.1.3.2.4	Applicability of requirements for different configurations

Editor’s note: For the requirements with different configurations on additional DMRS, the applicability will be added.

11.1.3.3 	Applicability of PRACH performance requirements

11.1.3.3.1	Applicability of requirements for different formats

The tests for a specific PRACH format are only applicable if the BS supports it.

For a BS supporting multiple PRACH formats, the tests for all the PRACH formats supported by the BS are applicable.

11.1.3.3.2	Applicability of requirements for different subcarrier spacings

For PRACH with short sequence, the tests for a specfic subcarrier spacing are only applicable if the BS supports it.

For a BS supporting multiple subcarrier spacings for a PRACH format with short sequence, for each FR, the tests for [smallest] subcarrier spacing supported by the BS are applicable.

11.1.3.3.3	Applicability of requirements for different channel bandwidths

PRACH performance requirements are defined a channel bandwidth agnostic way, and the tests for any one channel bandwidth supported by the BS are applicable.

< End of second change >