3GPP TSG-RAN WG4 Meeting #90                                                                 R4-1900778

Athens, Greece, 25 Feb - 1 Mar 2019

Source:	Huawei, Hisilicon

Title:	TP to TR 36.716.03-02: Addition of CA configuration for CA_1A-3A-8A-38A_2UL_1A-8A

Agenda item:	7.6.3

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_1A-3A-8A-38A_2UL_1A-8A as defined in Revised WID on Rel-16 LTE inter-band Carrier Aggregation for x bands DL (x= 3, 4, 5) with 2 bands UL RP-182301 [1].

Text Proposal

---Start of changes---

7.x LTE-A inter-band CA: Band 1 and Band 3 and Band 8 and Band 38 DL with UL CA_1A-8A

7.x.1	List of specific combination issues

7.x.1.1	Channel bandwidth per operating band for DL CA_1A-3A-8A-38A and UL CA_1A-8A

Table 7.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-8A-38A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

7.x.1.2	Co-existence studies for LTE-A inter-band CA 4 bands DL CA_1A-3A-8A-38A and 
UL CA_1A-8A

For 4 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 7.x.1.2-1.

Table 7.x.1.2-1: Co-existence study for DL CA_1A-3A-8A-38A with UL CA_1A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

In Table 7.x.1.2-1, 2nd harmonic from the band 8 can possibly impact low frequency edge of the band 3 and there is 4th IMD product by the band 1 and band 8 falls into the band 1.

7.x.1.3	MSD

When uplink CA_1A_8A is paired with downlink CA_1A-3A-8A-38A, the 2nd order harmonic product by band 8 falls into the own Rx frequency band 3. It has been solved by fallback combination CA_3A_8A.

When uplink CA_1A_8A is paired with downlink CA_1A-3A-8A-38A, the 4rd order IMD product by band 1 and band 8 falls into the own Rx frequency band 1. It has been solved by fallback combination CA_1A_8A.

7.x.1.4	∆TIB and ∆RIB values

The requirements of ∆TIB values in table 6.2.5-4 from TS36.101 [2] can be applied for CA_1-3-8-38.

The requirements of ∆RIB values in table 7.3.1-1C from TS36.101 [2] can be applied for CA_1-3-8-38.

---End of changes---

Reference

[1]	RP-182301, “Revised WID: Rel-16 LTE inter-band Carrier Aggregation for x bands DL (x= 3, 4, 5) with 2 band UL”, LG Electronics

[2]	TS 36.101 v15.5.0