3GPP TSG-RAN4 Meeting #90	R4-1900821

# Athens, EU, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0028 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: Addition of RC MU tables for unwanted emission in subclause 12.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR adds MU evaluation table for RC for unwanted emission in TR 38.817-02, sub-clause 12.7. This is an updated version of R4-1815327 presented at RAN4#89. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new subclause 12.7.1.2.2A have been created to capture the measurement evaluation for RC measureing spurious emission for FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, TR 38.817-02 will not described the capabilites of the reverberation chamber methodology to measure unwanted emission. The reverberation chamber test methodology brings some interesting aspects related to test time to consider for spurious emission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sub-clause 12.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 12.7	Conformance testing for OTA out of band TRP requirements

### 12.7.1 	Transmitter Spurious Emissions

#### 12.7.1.1	FR1

The measurement uncertainty for BS type 1-O out of band TRP requirements is based on the AAS BS MU analysis in TR 37.843 [9].

The TRP MU consists of an MU per point and a Summation Error (SE) which allows for errors in the calculation of the TRP from multiple directional power measurements and allows for a sparse grid to be used to reduce measurement time. The total MU is calculated as follows:

Refer to subclause 12.10 for the SE value.

The spurious emission requirements cover a large frequency range from 30MHz to 26GHz, many of the in-band chambers chosen for analysis cannot cover this entire range. The MU analysis is therefore based on a general chamber analysis rather than any specific method.

Other chambers may of course be used as long as the MU is within the specified value (or the test requirement is offset appropriately) and they are suitable for the frequencies being tested.

It is not necessary to measure TRP in the far field as the range may be impractical considering the frequency range being considered.

The MUperpoint  value in TR 37.843 [9] is:

MUperpoint = 2.20 dB,	30MHz < f ≤ 6 GHz

MUperpoint = 4.15 dB,	6GHz < f ≤ 26 GHz

By adding the MU per point and the SE values together we have the following total MU:

MUtotal = 2.3 dB,		30 MHz < f ≤ 6 GHz

MUtotal = 4.2 dB,		6 GHz < f ≤ 26 GHz

The test tolerance for mandatory spurious emissions is zero.

#### 12.7.1.2	FR2

##### 12.7.1.2.1	General

The spurious emissions requirements of the BS type 2-O are between 30MHz to the 2nd harmonic of the DL operating band. Currently the upper frequency limit calculated MU is 60GHz.

This range can be split into a number of regions:

30 MHz < f ≤ 6 GHz

This region also exists in FR1, the same MU is assumed for FR2 and for FR1

6 GHz < f ≤ 18 GHz

This is also an FR1 region however the MU values assumed in FR1 is larger than the in-band MU for FR2 which is at a higher frequency. A FR2 BS will likely be smaller than an FR1 BS and hence the chamber can be smaller and the requirements on the quiet zone are not so great. In addition, the test equipment is suitable for much higher frequencies (FR2 in band is above the frequency range) so the uncertainty not as great. The MU in the region therefore is assumed to be the same as the FR2 in-band MU.

18 GHz < f ≤ 40 GHz

This frequency range covers the FR2 in-band region. The in-band MU budget is in sub-clause 12.x.x.

40 GHz < f ≤ 60 GHz

This frequency range is above the in-band region, the measured frequencies are above the measurement frequency of the test equipment and hence a mixer is used to down convert the measured frequency to within the range of the test equipment.

##### 12.7.1.2.2	MU per point

###### 12.7.1.2.2.1	Indoor anechoic chamber MU Assessment

Descriptions of uncertainty contributors are given in TR 37.843 [26] Annex C1.

Table 12.7.1.2.2.1-1: Indoor anechoic chamber test range uncertainty assessment for spurious emissions

| UID | Uncertainty source | 40 GHz < f ≤ 60 GHz | Distribution of the probability | Divisor based on distribution shape | ci | 40 GHz < f ≤ 60 GHz |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| E1-1 (Note 1) | Positioning misalignment between the AAS BS and the reference antenna | 0.018 | Rect | √3 | 1 | 0.010 |
| E1-2 (Note 1) | Pointing misalignment between the AAS BS and the receiving antenna | 0 | Rect | √3 | 1 | 0.000 |
| E1-3 (Note 1) | Quality of quiet zone | 0.1 | Gaus | 1 | 1 | 0.100 |
| E1-4 (Note 1) | Polarization mismatch between the AAS BS and the receiving antenna | 0.018 | Rect | √3 | 1 | 0.010 |
| E1-5 (Note 1) | Mutual coupling between the AAS BS and the receiving antenna | 0 | Rect | √3 | 1 | 0.000 |
| E1-6 (Note 1) | Phase curvature | 0.05 | Gaus | 1 | 1 | 0.050 |
| E1-7 (Note 1) | Uncertainty of the RF Power Measurement Equipment | 0.6 | Gaus | 1 | 1 | 0.600 |
| E1-19 (Note 1) | test system frequency flatness | 0.25 | Gaus | 1 | 1 | 0.25 |
| (Note 2) | Uncertainty of the LNA | 0 | Gaus | 1 | 1 | 0.000 |
| (Note 3) | Uncertainty of the Mixer | 2.25 | Gaus | 1 | 1 | 2.250 |
| E1-8 (Note 1) | Impedance mismatch in the receiving chain | 0.42 | U | √2 | 1 | 0.297 |
| E1-9 (Note 1) | Random uncertainty | 0.1 | Rect | √3 | 1 | 0.058 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| E1-10 (Note 1) | Impedance mismatch between the receiving antenna and the network analyzer | 0.57 | U | √2 | 1 | 0.403 |
| E1-11 (Note 1) | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.43 | Rect | √3 | 1 | 0.248 |
| E1-12 (Note 1) | Impedance mismatch between the reference antenna and the network analyzer. | 0.57 | U | √2 | 1 | 0.403 |
| E1-3 (Note 1) | Quality of quiet zone | 0.1 | Gaus | 1 | 1 | 0.100 |
| E1-4 (Note 1) | Polarization mismatch for reference antenna | 0.018 | Rect | √3 | 1 | 0.010 |
| E1-5 (Note 1) | Mutual coupling between the reference antenna and the receiving antenna | 0 | Rect | √3 | 1 | 0.000 |
| E1-6 (Note 1) | Phase curvature | 0.07 | Gaus | 1 | 1 | 0.070 |
| E1-13 (Note 1) | Uncertainty of the network analyzer | 0.3 | Gaus | 1 | 1 | 0.300 |
| E1-14 (Note 1) | Influence of the reference antenna feed cable | 0.18 | Rect | √3 | 1 | 0.104 |
| E1-15 (Note 1) | Reference antenna feed cable loss measurement uncertainty | 0.1 | Gaus | 1 | 1 | 0.100 |
| E1-16 (Note 1) | Influence of the receiving antenna feed cable | 0.18 | Rect | √3 | 1 | 0.104 |
| E1-17 (Note 1) | Uncertainty of the absolute gain of the reference antenna | 0.52 | Rect | √3 | 1 | 0.300 |
| E1-18 (Note 1) | Uncertainty of the absolute gain of the receiving antenna | 0 | Rect | √3 | 1 | 0.000 |
| Combined standard uncertainty (1σ) (dB)(dB) |  |  |  |  |  | 2.49 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB)(dB) |  |  |  |  |  | 4.88 |
| Note 1: UID are referenced to TR 37.843 [26]. Note 2: Uncertainty of the LNA - To maintain a low noise figure for the measurement system (possibly considering the addition of a down conversion mixer for high frequencies) and LNA may be required. The variation in the gain of the LNA after the calibration procedure is accounted for in this uncertainty contribution Note 3: Uncertainty of the Mixer - Higher frequency emissions beyond the upper frequency range of the measurement equipment require down converting prior to measurement. The uncertainty introduced by the down conversion is accounted for in this uncertainty contribution. |  |  |  |  |  |  |

###### 12.7.1.2.2.2	CATR MU Assessment

Descriptions of uncertainty contributors are given in TR 37.843 [26] Annex C1.

Table 12.7.1.2.2.2-1: Compact antenna test range uncertainty assessment for spurious emissions

| UID (Note) | Uncertainty source | 40 GHz < f ≤ 60 GHz | Distribution of the probability | Divisor based on distribution shape | ci | 40 GHz < f ≤ 60 GHz |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| E2-1 (Note 1) | Misalignment DUT & pointing error | 0.3 | Rect | √3 | 1 | 0.173 |
| E2-2 (Note 1) | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.7 | Gaus | 1 | 1 | 0.7 |
| (Note 2) | Uncertainty of the LNA | 0 | Gaus | 1 | 1 | 0 |
| (Note 3) | Uncertainty of the Mixer | 2.25 | Gaus | 1 | 1 | 2.25 |
| E2-3 (Note 1) | Standing wave between DUT and test range antenna | 0.21 | U | √2 | 1 | 0.148 |
| E2-4 (Note 1) | RF leakage, test range antenna cable connector terminated. | 0.001 | Normal | 1 | 1 | 0.001 |
| E2-5 (Note 1) | QZ ripple with DUT | 0.0928 | Normal | 1 | 1 | 0.0928 |
| E2-16 (Note 1) | test system frequency flatness | 0.25 | Gaus | 1 | 1 | 0.25 |
| E2-14 (Note 1) | Miscellaneous uncertainty | 0 | Rect | √3 | 1 | 0.000 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| E2-6 (Note 1) | Network Analyzer | 0.3 | Normal | 1 | 1 | 0.3 |
| E2-7 (Note 1) | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | 0.57 | U | √2 | 1 | 0.403 |
| E2-8 (Note 1) | Insertion loss variation in receiver chain | 0.18 | Rect | √3 | 1 | 0.104 |
| E2-4 (Note 1) | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.001 | Normal | 1 | 1 | 0.001 |
| E2-9 (Note 1) | Influence of the calibration antenna feed cable | 0.29 | U | √2 | 1 | 0.205 |
| E2-10 (Note 1) | Uncertainty of the absolute gain of the calibration antenna | 0.52 | Rect | √3 | 1 | 0.300 |
| E2-11 (Note 1) | Misalignment positioning system | 0 | Exp. normal | 2 | 1 | 0 |
| E2-1 (Note 1) | Misalignment SGH and pointing error | 0.5 | Exp. normal | 2 | 1 | 0.25 |
| E2-12 (Note 1) | Rotary joints | 0 | U | √2 | 1 | 0.000 |
| E2-3 (Note 1) | Standing wave between SGH and test range antenna | 0.09 | U | √2 | 1 | 0.064 |
| E2-5 (Note 1) | QZ ripple with SGH | 0.009 | Normal | 1 | 1 | 0.009 |
| E2-15 (Note 1) | Switching uncertainty | 0.43 | Rect | √3 | 1 | 0.248 |
| Combined standard uncertainty (1σ) (dB)(dB) |  |  |  |  |  | 2.5 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB)(dB) |  |  |  |  |  | 4.9 |
| Note 1: UID are referenced to TR 37.843 [26]. Note 2: Uncertainty of the LNA - To maintain a low noise figure for the measurement system (possibly considering the addition of a down conversion mixer for high frequencies) and LNA may be required. The variation in the gain of the LNA after the calibration procedure is accounted for in this uncertainty contribution Note 3: Uncertainty of the Mixer - Higher frequency emissions beyond the upper frequency range of the measurement equipment require down converting prior to measurement. The uncertainty introduced by the down conversion is accounted for in this uncertainty contribution. |  |  |  |  |  |  |

###### 12.7.1.2.2.3	MU per point Summary

Table 12.7.1.2.2.3-1: Test system specific measurement uncertainty values

|  | Expanded uncertainty ue (dB) |
| --- | --- |
|  | 40 GHz < f ≤ 60 GHz |
| Indoor Anechoic Chamber | 4.88 |
| Compact Antenna Test Range | 4.88 |
| … |  |
| Common maximum accepted test system uncertainty | 4.9 |

##### 12.7.1.2.2A	Reverberation Chamber MU Assessment

The detailed descriptions of uncertainty sources relevant for the reverberation chamber test method can be found in [26].

Table 12.7.1.2.2A-1: Reverberation Chamber uncertainty assessment spurious emissions 380 MHz-18 GHz

| UID | Uncertainty source | Uncertainty value 380 MHz < f ≦ 3 GHz | Uncertainty value 3 GHz < f ≦ 3.8 GHz | Uncertainty value 3.8 GHz < f ≦ 12.75 GHz | Uncertainty value 12.75 GHz < f ≦ 18 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] 380 MHz < f ≦ 3 GHz | Standard uncertainty ui [dB] 3 GHz < f ≦ 3.8 GHz | Standard uncertainty ui [dB] 3.8 GHz < f ≦ 12.75 GHz | Standard uncertainty ui [dB] 12.75 GHz < f ≦ 18 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | 0.26 | 0.37 | Gaussian | 1 | 1 | 0.14 | 0.26 | 0.26 | 0.37 |
| 2 | Impedance mismatch in the receiving chain | 0.2 | 0.2 | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0.1 | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna mismatch efficiency | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.13 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 | 0.20 |
| 8 | Mean value estimation of power transfer function | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 | 1.5 | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.66 | 1.68 | 1.70 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.25 | 3.29 | 3.33 | 3.37 |

Table 12.7.1.2.2A-2: Reverberation Chamber uncertainty assessment spurious emissions 18 GHz – 60 GHz

| UID | Uncertainty source | Uncertainty value 18 GHz < f ≦ 26.5 GHz | Uncertainty value 26.5 GHz < f ≦ 40 GHz |  | Uncertainty value 40 GHz < f ≦ 60 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Uncertainty value 18 GHz < f ≦ 26.5 GHz | Uncertainty value 26.5 GHz < f ≦ 40 GHz | Uncertainty value 40 GHz < f ≦ 60 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.52 | 0.84 |  | 1.15 | Gaussian | 1 | 1 | 0.52 | 0.84 | 1.15 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 |  | 0.45 | U-shaped | √2 | 1 | 0.32 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0.1 | 0.1 |  | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 |  | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | 0.27 |  | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.30 | 0.30 |  | Gaussian | 1 | 1 | 0.20 | 0.30 | 0.30 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 |  | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | 0.27 |  | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 |  | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.76 | 1.89 | 2.05 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.45 | 3.71 | 4.02 |

##### 12.7.1.2.3	Test Tolerance

The conduced test tolerance for the mandatory spurious emissions requirements is zero. As the requirements are set by regulatory limits the same test tolerance is used for OTA.

TT =  0.

##### 12.7.1.2.4	Summary

For the frequency range 40 GHz < f ≦ 60 GHz  adding the MUperpoint and the SE (see subclause 12.10) we have:

Hence, we have the following MU values for the whole spurious emissions range:

Table 12.7.1.2.4-1: FR2 Spurious emissions MU and TT values

| Frequency range | MU (dB) | TT (dB) | Comment |
| --- | --- | --- | --- |
| 30 MHz < f ≦ 6 GHz | 2.3 | 0 | Same as FR1 |
| 6 GHz < f ≦ 18 GHz | 2.7 | 0 | same as FR2 in-band |
| 18 GHz < f ≦ 40 GHz | 2.7 | 0 | FR2 in-band, subclause 12.x.x |
| 40 GHz < f ≦ 60 GHz | 5 | 0 |  |

### 12.7.2	Receiver Spurious Emissions

#### 12.7.2.1	FR1

The measurement uncertainty for BS type 1-O out of band TRP requirements is based on the AAS BS MU analysis in TR 37.843 [9].

The TRP MU is very similar to that for the transmitter mandatory spurious emissions. However the receiver requirements are at a much lower power level so TRP calculation may be affected by the noise floor of the measurement system. The following uncertainty contribution is added to take into account this effect:

Measurement system dynamic range uncertainty: uncertainty due to the test requirement being close to the measurement equipment noise floor and the noise floor contributing significantly to the TRP total.

The MUperpoint value in TR 37.843 [9] is:

MUperpoint = 2.39 dB,		30 MHz < f ≤6 GHz

MUperpoint = 4.18 dB,	,	6 GHz < f ≤ 26 GHz

By adding the MU per point and the SE values (see subclause 12.10) together we have the following total MU:

MUtotal = 2.5 dB,		30 MHz < f ≤ 6 GHz

MUtotal = 4.2 dB,		6 GHz < f ≤ 26 GHz

The test tolerance for receiver spurious emissions is equal to the MU so the TT values are as follows:

TT = 2.5 dB,		30 MHz < f ≤ 6 GHz

TT = 4.2 dB,		6 GHz < f ≤ 19 GHz

#### 12.7.2.2	FR2

The receiver spurious emissions requirements of the BS type 2-O are between 30MHz to the 2nd harmonic of the DL operating band. Currently the upper frequency limit calculated MU is 60GHz.

This range can be split into a number of regions:

30 MHz < f ≤ 6 GHz

The same value is assumed for receiver spurious emissions as for transmitter spurious emissions. This is the same as the in band FR2 MU value.

6 GHz < f ≤ 18 GHz

The same value is assumed for receiver spurious emissions as for transmitter spurious emissions. This is the same as the in band FR2 MU value.

18 GHz < f ≤ 40 GHz

The same value is assumed for receiver spurious emissions as for transmitter spurious emissions. This is calculated in subclause 12.7.1.2.

40 GHz < f ≤ 60 GHz

The same value is assumed for receiver spurious emissions as for transmitter spurious emissions. This is calculated in subclause 12.7.1.2.

The test tolerance for receiver spurious emissions is equal to the MU so the TT values are as follows:

The receiver spurious emissions MU and TT values for FR2 are given in table 12.7.2.2-1:

Table 12.7.2.2-1: FR2 receiver Spurious emissions MU and TT values

| Frequency range | MU (dB) | TT (dB) | Comment |
| --- | --- | --- | --- |
| 30 MHz < f ≦ 6 GHz | 2.5 | 2.5 | Same as FR1 |
| 6 GHz < f ≦ 18 GHz | 2.7 | 2.7 | Same as FR2 in-band |
| 18 GHz < f ≦ 40 GHz | 2.7 | 2.7 | FR2 in-band, subclause 12.x.x |
| 40 GHz < f ≦ 60 GHz | 5 | 5 |  |

### 12.7.3	Additional Spurious Emissions requirements

#### 12.7.3.1	FR1

The measurement uncertainty for BS type 1-O out of band TRP requirements is based on the AAS BS MU analysis in TR 37.843 [9].

The additional spurious emission requirements including the co-existence with other BS in the same geographical area are based on existing co-existence with other 3GPP bands so the frequency ranges for the uncertainty assessment are different from the mandatory spurious emissions. In this case the uncertainty budgets for the in-band chambers are considered in the MU analysis. As the TRP level is at a low power level the measurement system dynamic range uncertainty is also considered as with the receiver spurious emissions.

Adding the MUperpoint and the SE we have:

,		f ≤ 3 GHz

,		3 GHz < f ≤ 4.2 GHz

,		4.2 GHz < f ≤ 6 GHz

By adding the MU per point and the SE values (see subclause 12.10) together we have the following total MU:

MUtotal = 2.6 dB,		f ≤ 3 GHz

MUtotal = 2.6 dB,		3 GHz < f ≤ 4.2 GHz

MUtotal = 3.0 dB,		4.2 GHz < f ≤ 6 GHz

For additional requirements for co-existence between 3GPP bands the TT values are as follows:

MUtotal = 2.6 dB,		f ≤ 3 GHz

MUtotal = 2.6 dB,		3 GHz < f ≤ 4.2 GHz

MUtotal = 3.0 dB,		4.2 GHz < f ≤ 6 GHz

For PHS, and public safety additional requirements the TT=0dB.

The MUperpoint  value in TR 37.843 [9] is:

MUperpoint = 2.48 dB,		f ≤3 GHz

MUperpoint = 2.52 dB,		3 GHz < f ≤ 4.2 GHz

MUperpoint = 2.93 dB,	,	4.2 GHz < f ≤ 6 GHz

By adding the MU per point and the SE values (see subclause 12.10) together we have the following total MU:

MUtotal = 2.6 dB,		f ≤ 3 GHz

MUtotal = 2.6 dB,		3 GHz < f ≤ 4.2 GHz

MUtotal = 3.0 dB,		4.2 GHz < f ≤ 6 GHz

The test tolerance for co-existence between 3GPP systems is equal to the MU so the TT values are as follows:

TT = 2.6 dB,		f ≤ 3 GHz

TT = 2.6 dB,		3 GHz < f ≤ 4.2 GHz

TT = 3.0 dB,		4.2 GHz < f ≤ 6 GHz

The test tolerance for additional requirement which are based on regulation (for example co-existence with PHS systems) is zero.

#### 12.7.3.2	FR2

There are currently no additional spurious emissions requirements or co-existence requirements for FR2.