3GPP TSG-RAN WG4 Meeting #90                     	R4-1900870

Athens, Greece, Feb 25th – Mar 1th, 2019

Agenda item:            6.14.1.3.2

Source:	MediaTek Inc.

Title:	NR PBCH simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation results of PBCH.

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation result of PBCH test cases. Table A.1 (in Appendix) is the test case list. Table A.2 contains other detailed parameter setting in the simulation.

Table 2.1 is the 1% BLER SNR for PBCH detection with knowledge of SSB index and Table 2.2 is the 1% BLER SNR values for PBCH detection without knowledge of SSB index

Table 2.1 1% BLER SNR (with knowledge of SSB index)

| Case Number | 2RX | 4RX |
| --- | --- | --- |
| 1 | -10.8 | -12.3 |
| 2 | -9.4 | -12.4 |
| 3 | -10.5 | N/A |
| 4 | -9.8 | N/A |

Table 2.1 1% BLER SNR (without knowledge of SSB index)

| Case Number | 2RX | 4RX |
| --- | --- | --- |
| 1 | -9.3 | -11.8 |
| 2 | -8.1 | -11.33 |
| 3 |  | N/A |
| 4 |  | N/A |

Observation 1: The operating SNR for 4RX test is very low and it is not suitable to be included in the requirement considering the stability at such low SNR

# 3	Conclusions

In the contribution, we provide our simulation results of PBCH tests. We have the following observations:

Observation 1: The operating SNR for 4RX test is very low and it is not suitable to be included in the requirement considering the stability at such low SNR

# 4	Reference

[1] R4-1816490, “Way forward on NR PBCH demodulation requirements”, CMCC.

Appendix

Table A.1 PBCH test cases

| Parameters | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Frequency range | FR1 | FR1 | FR2 | FR2 |
| Channel BW | 10MHz | 40MHz | 100MHz | 100MHz |
| SS/PBCH SCS | 15kHz | 30kHz | 120kHz | 240kHz |
| SS Block pattern | Case A | Case B/Case C | Case D | Case E |
| Antenna configuration | 1x2 1x4 | 1x2 1x4 | 1x2 | 1x2 |
| Propagation channel | TDL-C RMS DS=300ns Doppler=100Hz | TDL-A RMS DS=30ns, Doppler=10Hz | TDL-A RMS DS=30ns, Doppler=300Hz | TDL-A RMS DS=30ns, Doppler=75Hz |

Table A.2 PBCH simulation detailed parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| SS Block pattern | - Case A - 15 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes of {2, 8} + 14*n. For carrier frequencies smaller than or equal to 3 GHz, n=0, 1. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1, 2, 3. - Case B - 30 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8, 16, 20} + 28*n. For carrier frequencies smaller than or equal to 3 GHz, n=0. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1. - Case C - 30 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {2, 8} + 14*n. For carrier frequencies smaller than or equal to 3 GHz, n=0, 1. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1, 2, 3. - Case D - 120 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8, 16, 20} + 28*n. For carrier frequencies larger than 6 GHz, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18. - Case E - 240 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {8, 12, 16, 20, 32, 36, 40, 44} + 56*n. For carrier frequencies larger than 6 GHz, n=0, 1, 2, 3, 5, 6, 7, 8. | N=0 for all simulation |
| Cell ID | 0 |  |
| Number of SS/PBCH blocks within an SS burst set periodicity | 1 |  |
| SS/PBCH block index | 0 |  |
| SS burst periodicity | 20ms |  |
| MIB size | 24 bits |  |
| PBCH payload size (including CRC=24bits and 8 bits of SS/PBCH location information) | 56 bits |  |