3GPP TSG-RAN WG4 Meeting #90	R4-1900937

Athens, Greece, 25 February-1 March, 2019

Title: 					Introduction of enhanced measurements for high speed train for SCells

Source: 	Ericsson

Agenda item:	7.14.2

Document for:	Discussion

# Introduction

In RAN#80 a work item was agreed [1] for release 16 for further enhancement for high speed train in LTE. In this contribution we address the objective for carrier aggregation from an RRM perspective

- Extend the RRM/demodulation enhancement to CA in release 14 HST scenario [RAN4/RAN2] - At least, extend Rel-14 RRM/demodulation enhancement to CA case

# Discussion

## Overview of the release 14 solution

In release 14, enhanced DRX requirements for both idle and RRC connected state were introduced, under control of the single flag highSpeedEnhancedMeasFlag. Since carrier aggregation is not used in idle mode, there is no need to consider idle mode enhancements under the new work item. For RRC connected state, when highSpeedEnhancedMeasFlag is enabled, there are enhanced requirements for the PCC cell identification and measurement period. It is also explicitly stated that these requirements do not apply to SCells/SCC

Table 8.1.2.2.1.2-1B: Requirement to identify a newly detectable FDD intrafrequency cell for UE configured with highSpeedEnhancedMeasFlag DRX cycle length (s); Tidentify_intra (s) (DRX cycles). ≤0.04; 0.8 (Note1). 0.04<DRX-cycle≤0.08; Note2(15). 0.08<DRX-cycle≤1.28; Note2(10). 1.28<DRX-cycle≤2.56; Note2(20). Note1: Number of DRX cycle depends upon the DRX cycle in use. Note2: Time depends upon the DRX cycle in use.. Table 8.1.2.2.1.2-4: Requirement to measure FDD intrafrequency cells for UE configured with highSpeedEnhancedMeasFlag DRX cycle length (s); Tmeasure_intra (s) (DRX cycles). ≤0.04; 0.2 (Note1). 0.04<DRX-cycle≤0.08; Note2 (4). 0.08<DRX-cycle≤1.28; Note2 (3). 1.28<DRX-cycle≤2.56; Note2 (5). Note1: Number of DRX cycle depends upon the DRX cycle in use. Note2: Time depends upon the DRX cycle in use.. #### 8.3.3.1 Measurements of a secondary component carrier with active SCell When the SCell is activated, measurement performance requirements for the frequency are those given in clause 8.1.2.2(E-UTRAN intra frequency measurements and E-UTRAN intra frequency measurements with autonomous gaps). If common DRX is in use, then the requirements for that secondary component carrier are given by the applicable DRX requirements (FDD or TDD) in clause 8.1.2.2, otherwise the non DRX requirements are applicable. When highSpeedEnhancedMeasFlag is configured, the enhanced measurement requirements apply only to measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. The applicable measurement accuracy requirements are in clause 9.1.11 (Carrier aggregation measurement accuracy)

Table 1 : Relevant extracts from TS36.133 (release 14 solution).

During the work for release 14, UE power consumption was extensively discussed. Configuring enhanced measurement performance may have a small power penalty, but nevertheless the requirements have been defined assuming the UE does not need to wake up during mid DRX off period to perform intrafrequency cell search or neighbor measurement. Hence, the release 14 requirements should not impose a major burden on a well-designed UE implementation.

## Release 16 extension

Considering the objective to “At least, extend Rel-14 RRM/demodulation enhancement to CA case”, it is necessary to define enhanced measurement requirements also for SCC. Considering that a relatively power efficient solution was chosen for release 14, it should be straightforward to apply the same requirements to SCC for the purposes of SCell management in high speed train operation. Naturally, there may be some UE power penalty, however it is typically impossible to reduce measurement delays without increased UE measurement activity, which automatically implies additional power consumption, and the enhancements are not requiring additional wakeup. Since SCell DRX in carrier aggregation is aligned with PCell DRX, there should be no significant concern on power consumption.

Proposal 1: Enhanced intrafrequency cell identification and measurement period are defined for SCC in release 16 with the same values as for PCC in release 14 under the highSpeedEnhancedMeasFlag

Next, we consider whether it is necessary for the network to independently configure high speed measurements for the SCC(s) and PCC. Given that the UE is either operating in high speed condition or it is not, we do not see a compelling reason for independent configuration, although there is also no significant problem with an independent configuration. However, we propose:

Proposal 2: Enhanced intrafrequency cell identification and measurement period for SCC are configured with the existing highSpeedEnhancedMeasFlag

Our final considerations are on UE capabilities. The existing release 14 enhancements are supported from release 13 onwards in a release independent manner in 36.307 according to the capability “measurementEnhancements-r14”. Since there are already UE in the field which support enhancement on the PCell and not on the SCell, and this support cannot be determined from the UE release, we support a new capability indicator such as

Proposal 3: UE capability for enhanced intrafrequency cell identification and measurement period for SCC is indicated using a new high speed enhancement capability such as SCCmeasurementEnhancements-r16

Note that this is not a discussion about whether the new capability is optional or mandatory, and from which release it applies. We anticipate this discussion will take place around the completion of release 16.

## 36.133 text proposal

Based on proposals 1-3 the additional CA enhancement could be captured in 36.133 as shown below

#### 8.3.3.1 Measurements of a secondary component carrier with active SCell When the SCell is activated, measurement performance requirements for the frequency are those given in clause 8.1.2.2(E-UTRAN intra frequency measurements and E-UTRAN intra frequency measurements with autonomous gaps). If common DRX is in use, then the requirements for that secondary component carrier are given by the applicable DRX requirements (FDD or TDD) in clause 8.1.2.2, otherwise the non DRX requirements are applicable. For UE which do not support SCCmeasurementEnhancements-r16. when highSpeedEnhancedMeasFlag is configured, the enhanced measurement requirements apply only to measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. For UE which support SCCmeasurementEnhancements-r16. when highSpeedEnhancedMeasFlag is configured, the enhanced measurement requirements apply to measurements of the primary component carrier and all secondary component carriers with active SCell. The applicable measurement accuracy requirements are in clause 9.1.11 (Carrier aggregation measurement accuracy)

# Conclusion

In this contribution we address the objective for carrier aggregation from an RRM perspective

- Extend the RRM/demodulation enhancement to CA in release 14 HST scenario [RAN4/RAN2] - At least, extend Rel-14 RRM/demodulation enhancement to CA case

We provide an overview of the release 14 enhancements, and for release 16 propose:

Proposal 1: Enhanced intrafrequency cell identification and measurement period are defined for SCC in release 16 with the same values as for PCC in release 14 under the highSpeedEnhancedMeasFlag

Proposal 2: Enhanced intrafrequency cell identification and measurement period for SCC are configured with the existing highSpeedEnhancedMeasFlag

Proposal 3: UE capability for enhanced intrafrequency cell identification and measurement period for SCC is indicated using a new high speed enhancement capability such as SCCmeasurementEnhancements-r16

Based on these proposals, we have developed the following text proposal for 36.133:

#### 8.3.3.1 Measurements of a secondary component carrier with active SCell When the SCell is activated, measurement performance requirements for the frequency are those given in clause 8.1.2.2(E-UTRAN intra frequency measurements and E-UTRAN intra frequency measurements with autonomous gaps). If common DRX is in use, then the requirements for that secondary component carrier are given by the applicable DRX requirements (FDD or TDD) in clause 8.1.2.2, otherwise the non DRX requirements are applicable. For UE which do not support SCCmeasurementEnhancements-r16. when highSpeedEnhancedMeasFlag is configured, the enhanced measurement requirements apply only to measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. For UE which support SCCmeasurementEnhancements-r16. when highSpeedEnhancedMeasFlag is configured, the enhanced measurement requirements apply to measurements of the primary component carrier and all secondary component carriers with active SCell. The applicable measurement accuracy requirements are in clause 9.1.11 (Carrier aggregation measurement accuracy)

# References

- RP-181482, “New Work Item on Further performance enhancement for LTE in high speed scenario”, Softbank(moderator)