3GPP TSG-RAN WG4 Meeting #90	R4-1901456

Athens, Greece, 25 February – 1 March 2019

Source:	Ericsson, Telstra

Title:	TP to TR 37.716-21-21: Addition of EN-DC configuration DC_28A_n8A-n258A

Agenda item:	9.8.3

Document for:	Approval

# Background

This contribution provides text proposal on DC_28A_n8A-n258A as defined in the revised WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_28_n8-n258

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n8-n258 | 28 | 703 MHz |  | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n258 | 24250 MHz |  | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_28_n8-n258 | DC_28_n8 DC_28_n258 | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |

### 6.x.3	Co-existence studies

Co-existence studies of Band 28 + Band n8 + band n258 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

### 6.x.4	∆TIB and ∆RIB values

For DC_28_n8-n258, the TIB,c and RIB,c values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n8-n258 | 28 | 0.5 |
|  | n8 | 0.6 |
|  | n258 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n8-n258 | 28 | 0.1 |
|  | n8 | 0.2 |
|  | n258 | 0 |

### 6.x.5	MSD

Based on the co-existence findings in TR [2] MSD does not need to be defined for DC_28_n8-n258.

##### ---End of changes---

# Reference

[1] 		RP-182302, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]		R4-1814769,	”TR 37.716-21-21 v0.2.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16”, LG Electronics