3GPP TSG-RAN WG4 Meeting #90	R4-1901520

# Athens, Greece, February 25 – March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.133 10.3 Introduction of NE-DC SFTD performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-perf |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NE-DC has been introduced as late drop in Rel.15. Hence SFTD reporting between NR PCell and E-UTRA PSCell needs to be supported by the UE. Such requirements are currently missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing section 10.3 with measurement performance requirements for NR – E-UTRA dual connectivity. Introducing sub-clauses with SFTD measurement performance requirements for NE-DC with PCell in FR1 and FR2, respectively. The justification for splitting the text into separate sub-clauses for FR1 and FR2 is to follow the specification structure of 38.133. The same performance requirements as for EN-DC are proposed as the scenario is similar except that the roles of PCell and PSCell are swapped. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SFTD performance requirements for NE-DC will be missing, by which the network node will not be able to configure the UE optimally e.g. with respect to DRX cycles. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3, 10.3.1, 10.3.2, 10.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<UNCHANGED SECTIONS OMITTED>

<FIRST CHANGE>

## 10.3	Measurements for NR – E-UTRAN dual connectivity

### 10.3.1	Introduction

This clause contains performance requirements for UE supporting dual connectivity with NR PCell and E-UTRA PSCell.

### 10.3.2	SFN and frame timing difference (SFTD) in FR1

The SFN and frame timing difference (SFTD) is measured between NR PCell in FR1 and E-UTRA PSCell under NE-DC operation.

The accuracy requirements in Table 10.3.2-3 are applicable under the following conditions:

For PCell:

- [TBD]

- Io range defined in Table 10.3.2-1

For E-UTRA PSCell:

- Cell specific reference signals are transmitted either from one, two or four antenna ports

- Conditions defined in TS 36.101 [25] Clause 7.3 for reference sensitivity are fulfilled

- No changes to the uplink transmission timing are applied during the measurement period

- RSRP|dBm according to TS 36.133 [15] Annex B.3.5 for a corresponding band

- Io range defined in Table 10.3.2-2

Table 10.3.2-1: PCell Io range conditions for SFTD measurement accuracy in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 4, 5 | Minimum Io Note 2, 3 |  | Maximum Io |
| Conditions |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in TBD, depending on NR – E-UTRA band combination. NOTE 3: The condition level is increased by MSD as defined in TBD, if applicable depending on NR – E-UTRA band combination. NOTE 4: NR operating band groups are as defined in Section 3.5.2. NOTE 5: Only NR bands within NE-DC band combinations as specified in TBD are applicable. |  |  |  |  |

Table 10.3.2-2: PSCell Io range conditions for SFTD measurement accuracy

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | E-UTRA operating band groups Note 4, 5 | Minimum Io | Maximum Io |
| Conditions |  | dBm/15kHz Note 2, 3 | dBm/BWChannel |
|  | FDD_A, TDD_A | -121 | -50 |
|  | FDD_C, TDD_C | -120 | -50 |
|  | FDD_D | -119.5 | -50 |
|  | FDD_E, TDD_E | -119 | -50 |
|  | FDD_F | -118.5 | -50 |
|  | FDD_G | -118 | -50 |
|  | FDD_H | -117.5 | -50 |
|  | FDD_N | -114.5 | -50 |
| NOTE 1: When in dBm/15kHz, the minimum Io condition is expressed as the average Io per RE over all REs in that symbol. Io may be different in different symbols within a subframe. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in TBD. NOTE 3: The condition level is increased by MSD as defined in TBD, if applicable depending on NR – E-UTRA band combination. NOTE 4: E-UTRA operating band groups are as defined in TS 36.133 [15] Section 3.5. NOTE 5: Only E-UTRA bands within NE-DC band combinations as specified in TBD are applicable. |  |  |  |

Table 10.3.2-3: SFTD measurement accuracy in FR1

| Accuracy [Tc] | Conditions |
| --- | --- |
|  | Ês/Iot [dB] |
| ±[2560] | [-3] |

### 10.3.3	SFN and frame timing difference (SFTD) in FR2

The SFN and frame timing difference (SFTD) is measured between NR PCell in FR2 and E-UTRA PSCell under NE-DC operation.

The accuracy requirements in Table 10.3.3-3 are applicable under the following conditions:

For PCell:

- [TBD]

- Io range defined in Table 10.3.3-1

For E-UTRA PSCell:

- Cell specific reference signals are transmitted either from one, two or four antenna ports

- Conditions defined in TS 36.101 [25] Clause 7.3 for reference sensitivity are fulfilled

- No changes to the uplink transmission timing are applied during the measurement period

- RSRP|dBm according to TS 36.133 [15] Annex B.3.5 for a corresponding band

- Io range defined in Table 10.3.2-2 in sub-clause 10.3.2.

Table 10.3.3-1: PCell Io range conditions for SFTD measurement accuracy in FR2

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 3, 4 | Minimum Io Note 2 |  | Maximum Io |
| Conditions |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |
|  | NR_TDD_FR2_A | TBD | TBD | -50 |
|  | NR_TDD_FR2_B | TBD | TBD | -50 |
|  | NR_TDD_FR2_F | TBD | TBD | -50 |
|  | NR_TDD_FR2_G | TBD | TBD | -50 |
|  | NR_TDD_FR2_T | TBD | TBD | -50 |
|  | NR_TDD_FR2_Y | TBD | TBD | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in TBD, if applicable depending on NR – E-UTRA band combination. NOTE 3: NR operating band groups are as defined in Section 3.5.3. NOTE 4: Only NR bands within NE-DC band combinations as specified in TBD are applicable. |  |  |  |  |

Table 10.3.3-3: SFTD measurement accuracy in FR2

| Accuracy [Tc] | Conditions |
| --- | --- |
|  | Ês/Iot [dB] |
| ±[2560] | [-3] |

<END OF FIRST CHANGE>

<UNCHANGED SECTIONS OMITTED>