3GPP TSG-RAN4 Meeting #90		R4-1901670

Athens, Greece, 25 Feb– 1 March 2019

Title:	Reply LS on on transient period in sidelink channels

Response to:	R1-1814165

Release:	Rel-16

Work Item:	FS_NR_V2X

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

#### Name:	Chunhui ZHANG

E-mail Address:	Chunhui.zhang@ericsson.com

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on transient period in side link channels (R1-1814165) and provide following answer in RAN4 LS:

Answer for [Question 1]

The transient period will be needed if there is a power change between the symbol containing PSCCH and the following symbol. The magnitude should be similar with NR UE stated in 38.101-1 for FR1 which is 10us. There is no transient period needed if the power does not change between these symbols.

Answer for [Question 2]

There is no transient period needed for option 1B and 3 if the average power in the last symbol containing PSCCH and the first symbol containing PSSCH is the same.

Answer for [Question 3]

The conclusion will be the same for different type of the power amplifier.

Answer for [Question 4]

No other case is foreseen.

2. Actions:

To RAN1 WG:

RAN4 kindly asks RAN1 to take the above information into consideration.

3. Date of Next RAN4 Meetings:

TSG-RAN4 Meeting #90b 	Apri  8   – Apri 12, 2019		Xi’an, China

TSG-RAN4 Meeting #91	May  13   – Ma 17 , 2019		 USA