3GPP TSG-RAN WG4 Meeting RAN4#90                                                                                                                    R4-1901678

Athens, Greece, 25 February – 1 March 2019

Agenda Item:      9.12.1

Source:                 Ericsson

Title:                     A-MPR simulation results for band n48

Document for:    Discussion

# Introduction

In this document, A-MPR simulation results for UE power class 3 in band n48 are presented.

# Discussion

## CBRS band requirements

The requirement of CBRS band is shown in Figure 1, Table 1 and Table 2.

Figure 1

Table 2: SEM

|  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | Measurement bandwidth |
| ± 0 – 1 | -13 |  |  |  |  | 1 % channel bandwidth |
| ± 1 – X | -13 |  |  |  |  | 1 MHz |
| < – X or > X when 3540 MHz < ΔfOOB < 3710 MHz | -25 |  |  |  |  |  |
| NOTE: X is occupied channel bandwidth as defined in Table 6.5.1-1 |  |  |  |  |  |  |

Table 3: Additional Spurious emissions

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 40 MHz |  |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

## A-MPR Simulations

The A-MPR simulations are performed for UL bandwidths of up to 40MHz for PC3 power class only, according to Table 4.

Table 4: Uplink channel BW

| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| n48 | 15 | Yes | Yes | Yes | Yes | Yes |
|  | 30 |  | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes |

Needless to say that location and length of PRBs have impact on the A-MPR values. To address this dependency two different region for A-MPR calculations are considered: region close to/on the edge and regions towards the centre of the bandwidth. Table 5 shows different A_MPR regions for different channel bandwidth. .

Table 5: A-MPR regions for n48

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 5 MHz | 3552.5 ≤ FC ≤ 3697.5 | A1 |
| 10 MHz | 3555 ≤ FC ≤ 3695 | A2 |
| 15 MHz | 3557.5 ≤ FC < 3562.5 3687.5 < FC ≤ 3692.5 | A3 |
| 15 MHz | 3562.5 ≤ FC < 3687.5 | A4 |
| 20 MHz | 3560 ≤ FC < 3570 3680 < FC ≤ 3690 | A5 |
| 20 MHz | 3570 ≤ FC < 3680 | A6 |
| 40 MHz | 3570 ≤ FC < 3600 3650 < FC ≤ 3680 | A7 |
| 40 MHz | 3600 ≤ FC < 3650 | A8 |

The result of A_MPR simulations for regions A3, A5 and A7 (edge cases) are shown in table 6. As the results show and was expected, at the band edge, significant A-MPR are needed, especially for larger channel BW’s since the -40dBm/MHz region is within the 3rd order IMD region.

Table 6: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK |  |  |  |  |  |  |  |  |
|  | QPSK |  |  | 3/0.5 |  | 4/0.5 |  | 9/8 |  |
|  | 16 QAM |  |  | 3/1 |  | 5/1 |  | 9/9 |  |
|  | 64 QAM |  |  | 3/1 |  | 5/1 |  | 9/9 |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK |  |  | 4/3 |  | 6/3.5 |  | 11.5/10 |  |
|  | 16 QAM |  |  | 4/3 |  | 6/4 |  | 11.5//10 |  |
|  | 64 QAM |  |  | 4/4 |  | 6/4 |  | 11.5//10 |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |  |

Definition of inner and outer RBs are defined in TS 38.101-1, section 6.2.

# Conclusion

The result of A-MPR simulations are presented in this contribution. It is shown, at the band edge, significant A-MPR are needed, especially for larger channel BW’s since the -40dBm/MHz region is within the 3rd order IMD region.

# Appendix:

Simulations Plots.