3GPP TSG-RAN4 Meeting #90	R4-1901684

# Athens, Greece, 25 Feb– 1 March 2019

Source: 	Qualcomm Incorporated

Title: 	AMPR sims for CAT M1 and CAT M2

Agenda item:	5.4.2

Document for:	Approval

# Introduction

There are a few cases of AMPR that was left to be simulated in the last meeting for CAT M2 and CAT M1. In this paper we present the same.

# Discussion

The following tables show the AMPR required for the cases stated above:

Proposals: Below are the AMPR tables for the remaining NS values for CAT M1 and CAT M2.

NS_38, CAT M2

| Channel bandwidth [MHz] | Parameters | Region |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | (NBindex,SCstart ) | (0, 0-27) | (0, 30-42) | (0, 45-51) | (0, 96-120) | (0, 123-168) |  |  |
|  | Lcsc | 2,3 ,6 | 2,3 ,6 | 2,3 ,6 | 2,3 ,6 | 2,3 ,6 |  |  |
|  | A-MPR [dB] | ≤[16] | ≤[13] | ≤[10] | ≤[7] | ≤[3] |  |  |
| 5 | (WBindex,SCstart ) | (0, <96) | (0, >219) |  |  |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |  |  |
|  | A-MPR [dB] | ≤[7] | ≤[1] |  |  |  |  |  |
| 10 | (WBindex,SCstart ) | (0, <93) | (0, >219) |  |  |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |  |  |
|  | A-MPR [dB] | ≤[7] | ≤[1] |  |  |  |  |  |
| 15 | (WBindex,SCstart ) | (0, <87) | (0, >234) |  |  |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |  |  |
|  | A-MPR [dB] | ≤[7] | ≤[1] |  |  |  |  |  |
| 20 | (WBindex,SCstart ) | (0, <81) | (0, >234) |  |  |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |  |  |
|  | A-MPR [dB] | ≤[7] | ≤[1] |  |  |  |  |  |

CAT M1:

| NS_06 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BW [MHz] | 1.4 | 1.4 | 3 | 5 |
| (NBindex,SCstart) | (0, <15) | (0, >54) | (0, <15) | (0, <15 ) |
| Lcsc | 2 | 2 | 2 | 2 |
| AMPR [dB] | ≤0.5 | ≤0.5 | ≤0.5 | ≤0.5 |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. |  |  |  |  |
| NOTE2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |

| NS_07 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BW [MHz] | 10 |  |  |  |
| (NBindex,SCstart) | (0, <9) |  |  |  |
| Lcsc | 2 |  |  |  |
| AMPR [dB] | ≤0.5 |  |  |  |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. |  |  |  |  |
| NOTE2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |

| NS_12 |  |  |
| --- | --- | --- |
| Channel bandwidth [MHz] | Parameters | Region |
| 1.4 | (NBindex,SCstart ) | (0, ≤3) |
|  | Lcsc | 2,3,6 |
|  | A-MPR [dB] | ≤ 5 |
| 3 | (NBindex,SCstart) | (0, ≤3) |
|  | Lcsc | 2,3,6 |
|  | A-MPR [dB] | ≤ 5 |
| 5 | (NBindex,SCstart) | (0, ≤3) |
|  | Lcsc | 2,3,6 |
|  | A-MPR [dB] | ≤ 5 |

| NS_38 |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth [MHz] | Parameters | Region |  |
| 1.4 | (NBindex,SCstart ) | (0, <27) | (0, >51) |
|  | Lcsc | 2,3,6 | 2,3,6 |
|  | A-MPR [dB] | ≤[7] | ≤[1] |
| 3 | (NBindex,SCstart ) | (0, <21) | (0, >60) |
|  | Lcsc | 2,3,6 | 2,3,6 |
|  | A-MPR [dB] | ≤[7] | ≤[1] |
| 5 | (NBindex,SCstart ) | (0, <21) | (0, >60) |
|  | Lcsc | 2,3,6 | 2,3,6 |
|  | A-MPR [dB] | ≤[7] | ≤[1] |
| 10 | (NBindex,SCstart ) | (0, <15) |  |
|  | Lcsc | 2,3,6 |  |
|  | A-MPR [dB] | ≤[7] |  |
| 15 | (NBindex,SCstart ) | (0, <12) |  |
|  | Lcsc | 2,3,6 |  |
|  | A-MPR [dB] | ≤[7] |  |

# Conclusion

In this paper, the AMPR for CAT M1 and CAT M2 for the remaining NS values have been presented. The proposal in the section above captures these AMPR values.

# References