3GPP TSG RAN WG4 #90	R4-1901911

# Athens, Greece,  -  Feb, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-2: On FR2 PC1 0dB Reference MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-2-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 0dB MPR reference is not optimum |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove 0dB reference MPR statement and relocate 0dB MPR reference into MPR table Relocate narrow allocation MPR statement into MPR table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 38.101-2 is incomplete without correct PC1 0dB reference MPR clarification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.2.2.1	UE maximum output power reduction for power class 1

For power class 1, MPR for contiguous allocations is defined as follows:

MPR is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in table 5.3.2-1, and waveform types. MPR is defined in Table 6.2.2.1-1.

Table 6.2.2.1-1 MPR for power class 1

| Modulation | MPR (dB) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inner RB allocations, ≤ 200M |  |  | Inner RB allocations, 400M |  | Outer RB allocations, 50M, 100M, 200M, 400M |  | Outer/Inner RB allocations, 50M, 100M, 200M, 400M |  |  |  |
|  | BWalloc,RB > 10.8MHz, RBstart ≥ Ceil(1/3 NRB), RBend ≤ Ceil(2/3 NRB) | BWalloc,RB > 10.8MHz, RBstart < Ceil(1/3 NRB), RBend > Ceil(2/3 NRB) |  | BWalloc,RB > 10.8MHz |  | BWalloc,RB > 10.8MHz |  | 1.44MHz < BWalloc,RB ≤ 10.8MHz |  | BWalloc,RB ≤ 1.44MHz |  |
| DFT-s-OFDM PI/2 BPSK | 0 | ≤ [2.5] | ≤ 3.0 |  | ≤ [5.5] |  | ≤ [10] |  | ≤ [14.4] |  |
| DFT-s-OFDM QPSK | 0 | ≤ [3.0] | ≤ 3.5 |  | ≤ [6.5] |  | ≤ [10] |  | ≤ [14.4] |  |
| DFT-s-OFDM 16 QAM | ≤ [4.0] | ≤ [4.0] | ≤ 4.5 |  | ≤ [6.5] |  | ≤ [10] |  | ≤ [14.4] |  |
| DFT-s-OFDM 64 QAM | ≤ [4.5] | ≤ [4.5] | ≤ 6.5 |  | ≤ [6.5] |  | ≤ [10] |  | ≤ [14.4] |  |
| CP-OFDM QPSK | ≤ [4.5] | ≤ [4.5] | ≤ 5.0 |  | ≤ [6.5] |  | ≤ [10] |  | ≤ [14.4] |  |
| CP-OFDM 16 QAM | ≤ [5.5] | ≤ [5.5] | ≤ 6.5 |  | ≤ [6.5] |  | ≤ [10] |  | ≤ [14.4] |  |
| CP-OFDM 64 QAM | ≤ [7.5] | ≤ [7.5] | ≤ 9 |  | ≤ [7.5] |  | ≤ [10] |  | ≤ [14.4] |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High,

and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

< end of changes >