3GPP TSG RAN WG4 #90	R4-1901912

# Athens, Greece,  -  Feb, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-2: On FR2 PC3 0dB Reference MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-2-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Specify 0dB MPR in PC3 MPR table |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove 0dB MPR reference statement - Insert column and define region of 0dB MPR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 38.101-2 is incomplete without PC3 MPR clarification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.2.2.3	UE maximum output power reduction for power class 3

For power class 3, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 2.5 dB, when the allocated RB size is less than or equal to 1.44 MHz, and 0 ≤ RBstart ≤ Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤NRB-LCRB

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.3-1.

Table 6.2.2.3-1 MPRWT for power class 3

|  |  |  |  | Channel Bandwidth / MPRWT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 / 100 / 200 MHz |  |  |  | 400 MHz |  |  |
|  |  |  |  | RBstart ≥ Ceil(1/3 NRB), RBend ≤ Ceil(2/3 NRB) | RBstart < Ceil(1/3 NRB) or RBend > Ceil(2/3 NRB) |  |  |  |  |  |
| DFT-s-OFDM |  | Pi/2 BPSK |  | 0 |  | 1.5 |  |  | 3.0 |  |  |
|  |  | QPSK |  | 0 |  | 1.5 |  |  | 3.0 |  |  |
|  | 16QAM | 3 |  | 4.5 |
|  | 64QAM | 5 |  | 6.5 |
| CP-OFDM | QPSK | 3.5 |  | 5.0 |
|  | 16QAM | 5 |  | 6.5 |
|  | 64QAM | 7.5 |  | 9.0 |

< end of changes >