3GPP TSG RAN WG4 Meeting #89	R4-1902035

Athens, GR, 25th  Feb – 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [draft] Correction CR on UE measurement capability in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-2-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 specified that UE shall be capable of monitoring at least 24 SSB(s) with different SSB index and/or PCI on a single serving carrier which is PCC or PSCC or 1 SCC if PCC/PSCC is in a band different from SCC. However, when there are only SCCs in FR2 band, how UE selects one SCC to measure 24 SSB(s) is up to UE implementation according to the latest agreement. Even in such case, operators would configure RSRP measurement to 1 SCC in FR2 for mobility purpose and of course operators expect that such measurements are performed by using 24 SSB(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify how to select 1 SCC when there are only SCCs in FR2 band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may monitor 24 SSB(s) on 1 SCC which RSRP measurement is not configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This draft CR was made based on R4-1901952, which has already endorsed in RAN4#90 before treating this draft CR. |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

9.2.3	Number of cells and number of SSB

#### 9.2.3.1	Requirements for FR1

For each intra-frequency layer, during each Layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

- 8 identified cells, and

- [14] SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is no smaller than the number of configured RLM-RS SSB resources.

#### 9.2.3.2	Requirements for FR2

For each intra-frequency layer, during each Layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements on a single serving carrier of all the serving carriers configured in the same FR2 band for at least:

- 6 identified cells, and

- 24 SSBs with different SSB index and/or PCI,

where the single serving carrier shall be:

- PCC when UE is configured with SA NR operation mode with PCC in the band; or

- PSCC when UE is configured with EN-DC with PSCC in the band; or

- One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is up to the UE.

UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other serving carrier(s) in the same band.

---END OF CHANGE 1-