3GPP TSG-RAN WG4 Meeting #90              	R4-1902076

Athens, GR, 25th Feb – 1st March, 2018

Agenda item:	6.14.2.2.1

Source:	Samsung

Title:	Simulation results for NR PUSCH

Document for:	Discussion

# 1	Introduction

During RAN#82 meeting, the status report of Rel-15 NR WI is presented. As mentioned in the report, for NR BS performance requirements, due to lack of simulation results, some requirements, test parameters and configurations are still remaining TBD. Additional work is needed to further alignment simulation results with aiming to specify the requirement, and correct the requirement due to unachievable performance measures.

In this contribution, both ideal and impairment simulation results are provided.

# 2	Simulation Results

In this section, based on the agreed simulation assumption for NR PUSCH as table 1, table 2. The ideal and impairment simulation results of PUSCH are presented as table 3, to table 7.

Table 1: Simulation assumption for NR PUSCH for CP-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1,2 | 1,2 |
| Number of Rx | 2, 4,8 | 2 |
| Transmission scheme | Identity matrix (TMPI index 0) | n.a. |
| Number of layers | 1, 2 | 1,2 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1(2), 1+1(2,11) | 1 |
| symbols length | 14 | 10 |
| Time domain resource allocation type | type A | type B |
| MCS index | 2, 16, 20 (20 only for 1Tx) | 2, 16, 20 (20 only for 1 Tx) |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz MCS 20: TDL-A 30ns, 10Hz | MCS 2 : TDL-A 30 ns, 300Hz MCS 16: TDL-A 30 ns,300Hz MCS 20: TDL-A 30 ns, 75HZ |
| SCS and BW | 15kHz: 5,10, 20 MHz 30kHz: 10, 20, 40, 100 MHz | 60kHz: 50, 100 MHz 120kHz: 50,100,200 MHz |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Configured with KPTRS =2,LPTRS=1 |

Table 2: Simulation assumption for NR PUSCH for DFT-s-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1,2 | 1 |
| Number of Rx | 2, 4,8 | 1,2 |
| Transmission scheme | Identity matrix (TMPI index 0) | n.a. |
| Number of layers | 1, 2 | 1 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1(2), 1+1(2,11) | 1 |
| symbols length | 14 | 10 |
| Time domain resource allocation type | type A | type B |
| MCS index | 2, 16, 20 | 2, 16, 20 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | MCS 2: TDL-B 100ns, 400 Hz | MCS 2 : TDL-A 30 ns, 300Hz |
| SCS and BW | 15kHz: 5MHz 30kHz: 10MHz 15kHz: 25 PRB (contig., middle of test BW) 30kHz: 24 PRB (contig., middle of test BW) | 60kHz: 50MHz 120kHz: 50MHz 60kHz: 30 PRB (contig., middle of test BW) 120kHz: 30 PRB (contig., middle of test BW) |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS=1 |

Table3: The target SNR performance for NR PUSCH for FR1 with 1 Tx with CP-OFDM

| Tx/Rx | SCS (KHz) | Number of DMRS symbol | BW (MHz) | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS16 (dB) (ideal) | Target SNR for MCS20 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) | Target SNR for MCS16 (dB) (impairment) | Target SNR for MCS20 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2Rx | 15 | 1 | 5 | 1.57 | 11.69 | 10.94 | 3.57 | 13.69 | 12.94 |
|  |  | 1+1 | 5 | -3.01 | 8.46 | 10.81 | -1.01 | 10.46 | 12.81 |
|  |  | 1 | 10 | 1.54 | 14.82 | 11.09 | 3.54 | 16.82 | 13.09 |
|  |  | 1+1 | 10 | -2.90 | 8.53 | 10.89 | -0.9 | 10.53 | 12.89 |
|  |  | 1 | 20 | N.A | N.A | 10.92 | N.A | N.A | 12.92 |
|  |  | 1+1 | 20 | -3.56 | 8.40 | 10.78 | -1.56 | 10.4 | 12.78 |
|  | 30 | 1 | 10 | -3.13 | 8.88 | 10.42 | -1.13 | 10.88 | 12.42 |
|  |  | 1+1 | 10 | -3.58 | 8.64 | 10.45 | -1.58 | 10.64 | 12.45 |
|  |  | 1 | 20 | -3.37 | 9.14 | 10.77 | -1.37 | 11.14 | 12.77 |
|  |  | 1+1 | 20 | -4.51 | 8.58 | 10.55 | -2.51 | 10.58 | 12.55 |
|  |  | 1 | 40 | -2.57 | 9.12 | 10.75 | -0.57 | 11.12 | 12.75 |
|  |  | 1+1 | 40 | -3.99 | 8.50 | 10.37 | -1.99 | 10.5 | 12.37 |
|  |  | 1 | 100 | -1.90 | 9.25 | 11.57 | 0.1 | 11.25 | 13.57 |
|  |  | 1+1 | 100 | -4.53 | 8.49 | 11.24 | -2.53 | 10.49 | 13.24 |
| 1Tx4Rx | 15 | 1 | 5 | -2.88 | 6.02 | 7.40 | -0.88 | 8.02 | 9.4 |
|  |  | 1+1 | 5 | -7.08 | 4.56 | 7.21 | -5.08 | 6.56 | 9.21 |
|  |  | 1 | 10 | -3.05 | 6.57 | 7.47 | -1.05 | 8.57 | 9.47 |
|  |  | 1+1 | 10 | -7.29 | 4.63 | 7.23 | -5.29 | 6.63 | 9.23 |
|  |  | 1 | 20 | N.A | 6.68 | 7.56 | N.A | 8.68 | 9.56 |
|  |  | 1+1 | 20 | -7.08 | 4.52 | 7.09 | -5.08 | 6.52 | 9.09 |
|  | 30 | 1 | 10 | -6.63 | 4.89 | 6.71 | -4.63 | 6.89 | 8.71 |
|  |  | 1+1 | 10 | -6.8 | 4.70 | 6.73 | -4.8 | 6.7 | 8.73 |
|  |  | 1 | 20 | -6.78 | 5.07 | 7.12 | -4.78 | 7.07 | 9.12 |
|  |  | 1+1 | 20 | -7.70 | 4.63 | 6.87 | -5.7 | 6.63 | 8.87 |
|  |  | 1 | 40 | -5.97 | 4.98 | 7.22 | -3.97 | 6.98 | 9.22 |
|  |  | 1+1 | 40 | -7.51 | 4.56 | 6.75 | -5.51 | 6.56 | 8.75 |
|  |  | 1 | 100 | -5.37 | 5.08 | 7.96 | -3.37 | 7.08 | 9.96 |
|  |  | 1+1 | 100 | -7.79 | 4.54 | 7.55 | -5.79 | 6.54 | 9.55 |
| 1Tx8Rx | 15 | 1 | 5 | -6.25 | 2.69 | 4.37 | -4.25 | 4.69 | 6.37 |
|  |  | 1+1 | 5 | -9.88 | 1.37 | 3.92 | -7.88 | 3.37 | 5.92 |
|  |  | 1 | 10 | -6.23 | 3.04 | 4.42 | -4.23 | 5.04 | 6.42 |
|  |  | 1+1 | 10 | -9.68 | 1.59 | 3.91 | -7.68 | 3.59 | 5.91 |
|  |  | 1 | 20 | N.A | 3.00 | 4.51 | N.A | 5 | 6.51 |
|  |  | 1+1 | 20 | -9.47 | 1.33 | 3.77 | -7.47 | 3.33 | 5.77 |
|  | 30 | 1 | 10 | -9.20 | 2.02 | 3.59 | -7.2 | 4.02 | 5.59 |
|  |  | 1+1 | 10 | -9.76 | 1.67 | 3.48 | -7.76 | 3.67 | 5.48 |
|  |  | 1 | 20 | -9.34 | 2.12 | 3.91 | -7.34 | 4.12 | 5.91 |
|  |  | 1+1 | 20 | -10.13 | 1.61 | 3.58 | -8.13 | 3.61 | 5.58 |
|  |  | 1 | 40 | -8.63 | 2.08 | 4.02 | -6.63 | 4.08 | 6.02 |
|  |  | 1+1 | 40 | -10.25 | 1.51 | 3.51 | -8.25 | 3.51 | 5.51 |
|  |  | 1 | 100 | -7.97 | 2.20 | 4.83 | -5.97 | 4.2 | 6.83 |
|  |  | 1+1 | 100 | -10.36 | 1.68 | 4.17 | -8.36 | 3.68 | 6.17 |

As agreed in the last meeting, these cases for PUSCH with 1DMRS configuration under 2Tx2Rx are removed, due to that the throughput ratio to set up requirements cannot be reached.

Table4: The target SNR performance for NR PUSCH for FR1 with 2 Tx with CP-OFDM

| Tx/Rx | SCS (KHz) | Number of DMRS symbol | BW(MHz) | Target SNR for MCS2 (dB)(ideal) | Target SNR for MCS16 (dB)(ideal) | Target SNR for MCS2 (dB)(impairment) | Target SNR for MCS16 (dB)(impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Tx2Rx | 15 | 1+1 | 5 | 0.84 | 16.23 | 2.84 | 18.23 |
|  |  | 1+1 | 10 | 1.01 | 16.60 | 3.01 | 18.6 |
|  |  | 1+1 | 20 | 0.82 | 16.49 | 2.82 | 18.49 |
|  | 30 | 1+1 | 10 | 0.03 | 18.39 | 2.03 | 20.39 |
|  |  | 1+1 | 20 | -0.73 | 18.27 | 1.27 | 20.27 |
|  |  | 1+1 | 40 | -0.35 | 18.09 | 1.65 | 20.09 |
|  |  | 1+1 | 100 | -1.01 | 18.81 | 0.99 | 20.81 |
| 2Tx4Rx | 15 | 1 | 5 | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 5 | -3.26 | 9.19 | -1.26 | 11.19 |
|  |  | 1 | 10 | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 10 | -3.2 | 9.48 | -1.2 | 11.48 |
|  |  | 1 | 20 | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 20 | -3.23 | 9.22 | -1.23 | 11.22 |
|  | 30 | 1 | 10 | -3 | 10.8 | -1 | 12.8 |
|  |  | 1+1 | 10 | -3.35 | 9.86 | -1.35 | 11.86 |
|  |  | 1 | 20 | -2.38 | 10.79 | -0.38 | 12.79 |
|  |  | 1+1 | 20 | -3.83 | 9.81 | -1.83 | 11.81 |
|  |  | 1 | 40 | -1.6 | 10.74 | 0.4 | 12.74 |
|  |  | 1+1 | 40 | -3.84 | 9.75 | -1.84 | 11.75 |
|  |  | 1 | 100 | -1.04 | 11.53 | 0.96 | 13.53 |
|  |  | 1+1 | 100 | -4.53 | 10.03 | -2.53 | 12.03 |
| 2Tx8Rx | 15 | 1 | 5 | -2.8 | 7.74 | -0.8 | 9.74 |
|  |  | 1+1 | 5 | -5.88 | 5.19 | -3.88 | 7.19 |
|  |  | 1 | 10 | N.A | 8.68 | N.A | 10.68 |
|  |  | 1+1 | 10 | -5.9 | 5.23 | -3.9 | 7.23 |
|  |  | 1 | 20 | N.A | 9.04 | N.A | 11.04 |
|  |  | 1+1 | 20 | -5.68 | 5.16 | -3.68 | 7.16 |
|  | 30 | 1 | 10 | -5.53 | 6.25 | -3.53 | 8.25 |
|  |  | 1+1 | 10 | -6.15 | 5.42 | -4.15 | 7.42 |
|  |  | 1 | 20 | -5.16 | 6.17 | -3.16 | 8.17 |
|  |  | 1+1 | 20 | -6.5 | 5.28 | -4.5 | 7.28 |
|  |  | 1 | 40 | -4.65 | 6.15 | -2.65 | 8.15 |
|  |  | 1+1 | 40 | -6.44 | 5.26 | -4.44 | 7.26 |
|  |  | 1 | 100 | -4.08 | 6.53 | -2.08 | 8.53 |
|  |  | 1+1 | 100 | -7.02 | 5.34 | -5.02 | 7.34 |

Table5: The target SNR performance for NR PUSCH for FR1with DFT-OFDM

| Tx/Rx | SCS | Number of DMRS symbol | BW | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- |
| 1T2Rx | 15 | 1 | 5 | 3.67 | 5.67 |
|  | 15 | 1+1 | 5 | -3.82 | -1.82 |
|  | 30 | 1 | 10 | -2.97 | -0.97 |
|  | 30 | 1+1 | 10 | -3.77 | -1.77 |
| 1Tx4Rx | 15 | 1 | 5 | -2.85 | -0.85 |
|  | 15 | 1+1 | 5 | -6.37 | -4.37 |
|  | 30 | 1 | 10 | -5.39 | -3.39 |
|  | 30 | 1+1 | 10 | -6.73 | -4.73 |
| 1Tx8Rx | 15 | 1 | 5 | -6.36 | -4.36 |
|  | 15 | 1+1 | 5 | -9.82 | -7.82 |
|  | 30 | 1 | 10 | -8.97 | -6.79 |
|  | 30 | 1+1 | 10 | -9.56 | -7.56 |

Table6 The target SNR performance for NR PUSCH for FR2 with CP-OFDM (to be updated)

| Tx/Rx | SCS | BW | Number of DMRS symbol | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS16 (dB) (ideal) | Target SNR for MCS20 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) | Target SNR for MCS16 (dB) (impairment) | Target SNR for MCS20 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2Rx | 60 | 50 | 1 |  |  |  |  |  |  |
|  | 60 | 100 | 1 |  |  |  |  |  |  |
|  | 120 | 50 | 1 |  |  |  |  |  |  |
|  | 120 | 100 | 1 |  |  |  |  |  |  |
|  | 120 | 200 | 1 |  |  |  |  |  |  |
| 2T2Rx | 60 | 50 | 1 |  |  |  |  |  |  |
|  | 60 | 100 | 1 |  |  |  |  |  |  |
|  | 120 | 50 | 1 |  |  |  |  |  |  |
|  | 120 | 100 | 1 |  |  |  |  |  |  |
|  | 120 | 200 | 1 |  |  |  |  |  |  |

Table7: The target SNR performance for NR PUSCH for FR2 with DFT-s-OFDM (to be updated)

| Tx/Rx | SCS | Number of DMRS symbol | BW | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- |
| 1T2Rx | 60 | 1 | 50 |  |  |
|  | 120 | 1 | 50 |  |  |

Observation 1: Based on the simulation results, some test cases with 1 only DMRS configuration is unreliable, which has bad performance and cannot achieve the 70% of maximum throughput due to the chosen channel conditions. Removing these cases in current specification should be considered.

# 3	Conclusion

In this contribution, the ideal and impairment simulation results are provided.

Observation 1: Base on the simulation results, some test cases with only 1 DMRS configuration are unreliable, which has bad performance and cannot achieve the 70% of maximum throughput due to the chosen channel condition. Removing these cases in current specification should be considered.

# References

R4-1816346, WF on NR BS demodulation general part, Huawei.

R4-1816347, WF on NR PUSCH demodulation Nokia.