3GPP TSG-RAN WG4 Meeting #90	R4-1902609

# Athens, Greece, 25th February – 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 SS-SINR test case for EN-DC in FR1 (Section A.4.7.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The SS-SINR measurement accuracy test cases for EN-DC in FR1 are not added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Creat SS-SINR measurement accuracy test cases for EN-DC in FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SS-SINR measurement accuracy test cases for EN-DC in FR1 are not added |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section A.4.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change ------------------------------------------------------------

A.4.7.3	SS-SINR

A.4.7.3.1	Intra-frequency case

A.4.7.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.12.1.1.

A.4.7.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.4.7.3.1.2-1. The absolute accuracy of SS-SINR intra-frequency measurement is test by using the parameters in Table A.4.7.3.1.2-2. The configuration of cell 1 (E-UTRA PCell) is specified in section A.3.7.2.1. In all test cases, Cell 2 is the PSCell and Cell 3 is the target cell.

Table A.4.7.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.4.7.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  |  | Config 1,4 |  | FDD |  |  |  |
|  |  |  | Config 2,3,5,6 |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1,4 |  | Not Applicable |  |  |  |
|  |  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |
| TRS configuration |  |  |  |  | TRS.2.1 TDD |  |  |  |
| TCI state |  |  |  |  | TCI.State.0 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  |  | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  | Config 3,6 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  |  |  | Config 2,5 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  | Config 3,6 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  |  | Config 2,5 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  | Config 3,6 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |
| SMTC configruation |  |  |  |  | SMTC.1 |  |  |  |
| SSB configuration |  |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2,4,5 | kHz | 15 |  |  |  |
|  |  |  | Config 3,6 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/15kHz | TBD |  | TBD |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | TBD |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | TBD |  |
| Note2 | Config 1,2,4,5 |  |  | dBm/SCS | TBD |  | Same as Noc for 15kHz |  |
|  | Config 3,6 |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  | TBD |  | TBD |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | TBD |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | TBD |  |
|  |  |  |  | dB | [5] | [-3.19] | [-5.46] | [-5.46] |
|  |  |  |  | dB | TBD | TBD | TBD | TBD |
| SS-RSRPNote3 | Config 1,2,4,5 |  | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/SCS | TBD | TBD | TBD | TBD |
|  |  |  | NR_FDD_FR1_B |  |  |  | TBD | TBD |
|  |  |  | NR_TDD_FR1_C |  |  |  | TBD | TBD |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | TBD | TBD |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | TBD | TBD |
|  |  |  | NR_FDD_FR1_G |  |  |  | TBD | TBD |
|  |  |  | NR_FDD_FR1_H |  |  |  | TBD | TBD |
|  | Config 3,6 |  | NR_FDD_FR1_A, NR_TDD_FR1_A |  | TBD | TBD | TBD | TBD |
|  |  |  | NR_FDD_FR1_B |  |  |  | TBD | TBD |
|  |  |  | NR_TDD_FR1_C |  |  |  | TBD | TBD |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | TBD | TBD |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | TBD | TBD |
|  |  |  | NR_FDD_FR1_G |  |  |  | TBD | TBD |
|  |  |  | NR_FDD_FR1_H |  |  |  | TBD | TBD |
| SS-SINR Note3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A | dB | [5] | [-3.19] | [-5.46] | [-5.46] |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |
| IoNote3 |  | Config 1,2,4,5 | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/ 9.36MHz | TBD |  | TBD |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | TBD |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | TBD |  |
|  |  | Config 3,6 | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/ 38.16MHz | TBD |  | TBD |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | TBD |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | TBD |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | TBD |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

A.4.7.3.1.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in section 10.1.12.1.1.

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

A.4.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.14.1.1 and 10.1.14.1.2 for inter frequency measurement.

A.4.7.3.2.2	Test Parameters

In this test case the two NR cells (i.e., Cell 2 and Cell 3) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.4.7.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table A.4.7.3.2.2-2. In all test cases, Cell 2 is the PSCell and Cell 3 is target cell. Cell 1 is the E-UTRA cell of which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.4.7.3.2.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.4.7.3.2.2-1: SS-SINR Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  |  |  | TCI.State.0 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3,6 |  | SR.2.1 TDD |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2,5 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3,6 |  | CR.2.1 TDD |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2,5 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3,6 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configruation |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
| Note2 | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/15kHz | TBD |  | TBD |  | TBD |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
| Note2 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/SCS | TBD |  | TBD |  | TBD |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A |  | TBD |  | TBD |  | TBD |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
|  |  |  | dB | [-1.75] | [-1.75] | [20] | [20] | [-4.0] | [-4.0] |
|  |  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| SS-RSRPNote3 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/SCS | TBD | TBD | TBD | TBD | TBD | TBD |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | TBD | TBD |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | TBD | TBD |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | TBD | TBD |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | TBD | TBD |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | TBD | TBD |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | TBD | TBD |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A |  | TB D | TBD | TBD | TBD | TBD | TBD |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | TBD | TBD |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | TBD | TBD |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | TBD | TBD |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | TBD | TBD |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | TBD | TBD |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | TBD | TBD |
| SS-SINR Note3 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dB | [-1.75] | [-1.75] | [20] | [20] | [-4.0] | [-4.0] |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/ 9.36MHz | TBD |  | TBD |  | TBD |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/ 38.16MHz | TBD |  | TBD |  | TBD |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | TBD |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | TBD |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | TBD |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

A.4.7.3.2.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in section 10.1.14.1.1 and 10.1.14.1.2.

----------------------------------------------------------- End of Change ----------------------------------------------------------------