3GPP TSG-RAN WG4 Meeting #90	R4-1902648

Athens, Greece, 25 February – 1 March 2019

Agenda Item:	6.10.2.3

Source:	Huawei

Title:	WF on Filler Bits for Test Model

Document for:	Approval

# Background

## From RAN4#89

WF R4-1816736 [“WF on filling remaining RBs of PDCCH symbols” Huawei, HiSilicon] discussed several options

- (option 1) Have PDCCH span entire symbol; satisfying a maximum of  6⌊N_RB⁄6⌋ RBs

- (option 2) The filler region is occupied by PDSCH corresponding to another user (e.g. third user N_RNTI = 3)

- (option 3) The filler region is occupied by PDSCH of user 1 (and user 2). Their PDSCH will span the entire slot (all 14 symbols) except for the PDCCH

- (option 4) Create a second PDCCH to occupy the filler region as much as possible within the first two symbols

- (option 5) Double the number of CCEs so that the number of occupied RBs is 6 across 2 symbols.

- (option 6) Move PDCCH to center of bandwidth

- (option 7) PDCCH 1 CCE spanning only 1 symbol only

- (option 8) keep current design

- Other options not precluded

## From RAN4#90

### Contributions

The contributions regarding the filler bits in this meeting are listed below.

- R4-1900304, “View on PDCCH filler”, Samsung

- R4-1901328, “On filling remaining RBs of PDDCH in test models”, Nokia, Nokia Shanghai Bell

- R4-1900497,” Further discussion on NR TM configuration”, ZTE

- R4-1901337, “Draft CR to 38.141-1 clause 4.9.2 Test models - filling PDCCH”, Nokia, Nokia Shanghai Bell

- R4-1901338, “Draft CR to 38.141-2 clause 4.9.2 Test models - filling PDCCH”, Nokia, Nokia Shanghai Bell

- R4-1901364, “Proposal on filling remaining RBs of PDCCH symbols in Test Model definitions”, Keysight Technologies

- R4-1901371, “NR Test Model PDSCH Arrangement”, Ericsson

- R4-1901493, “Discussion of filling remaining RBs in PDCCH symbols of test models”, Huawei, HiSilicon

- R4-1901494, “TP to TS38.141-1 subclause 4.9.2 filling remaining RBs in PDCCH symbols”, Huawei, HiSilicon

- R4-1901495, TP to TS38.141-2 subclause 4.9.2 filling remaining RBs in PDCCH symbols”, Huawei,

## Ad-hoc session

In the approved minutes of the Ad-hoc session [1], the following was agreed:

The power problem is well accepted and RAN4 needs to solve it.

Further investigate the following options:

- (option 1) Have PDCCH span entire symbol; satisfying a maximum of  6NRB6 RBs

- (option 2) The filler region is occupied by PDSCH corresponding to another user (e.g. third user N_RNTI = 3)

- (option 3) The filler region is occupied by PDSCH of user 1 (and user 2). Their PDSCH will span the entire slot (all 14 symbols) except for the PDCCH.

  - DMRS is always located at the third symbol or beyond

- (option 4) Create a second PDCCH to occupy the filler region as much as possible within the first two symbols

# Discussion

## Previous options

Diagrams of the various options are shown in Figure 1 and Figure 2.

- Option 1 uses PDCCH as much as possible (the number of PDCCH and aggregation levels are functions of bandwidth). The power for any remaining RBs is distributed among the PDCCH.

- Option 2 uses PDSCH based on PDSCH-mapping-type-B.

- Option 3:

  - for 1 user, the PDSCH occupies the entire slot except for a little corner

  - for 2 users, the size of the CORESET is 3 times the RBG size. The CORESET occupies 2 symbols by 3 RBGs. The PDSCH wraps around

- Option 4:

  - There are either 1 or 2 CORESETs. When 2 CORESETs present, the second CORESET spans the first two symbols as much as possible. The power for any remaining RBs is distributed among the PDCCH.

PDSCH

Fill

PDCCH

PDCCH

PDCCH

PDSCH

PDCCH

PDSCH

PDSCH

PDCCH

PDCCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

Option 1

Option 2

Option 3 (non-boosting TMs left, boosting TMs right)

PDSCH

Fill

PDCCH

PDCCH

PDCCH

PDSCH

PDCCH

PDSCH

PDSCH

PDCCH

PDCCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

Option 1

Option 2

Option 3 (non-boosting TMs left, boosting TMs right)

Figure 1. Options 1 – 3 (design not scale)

PDSCH

Fill

PDCCH

PDCCH

PDCCH

Option 4

PDSCH

Fill

PDCCH

PDCCH

PDCCH

Option 4

Figure 2. Option 4

## New options

Another option is shown in Figure 3. In this option (option 9 to avoid confusion),

- TM1.1, TM3.1: the PDCCH is removed. Entire slot is PDSCH for user 1, EVM measurements skip the first two symbols.

- TM2: PDCCH is kept (to help synchronization) EVM measurements skip the first two symbols.

- TM1.2, TM3.2, TM3.3: the PDCCH is removed. Entire slot is PDSCH for user 1 and user 2. EVM measurements skip the first two symbols.

PDSCH

PDSCH

PDCCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

Not used for EVM measurements

PDSCH

PDSCH

PDSCH

PDSCH

PDCCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

Not used for EVM measurements

PDSCH

PDSCH

Figure 3. Option 9 non-boosting TMs left, TM2 middle, boosting TMs right

Why skip 2 symbols:

- Measurements always being on same symbol

- MSR considerations

Another option is shown in Figure 3. In this option,

- TM1.1, TM3.1: the PDCCH is removed. Entire slot is PDSCH for user 1, EVM measurements skip the first two symbols.

- TM2: PDCCH is kept (to help synchronization) EVM measurements skip the first two symbols.

- TM1.2, TM3.2, TM3.3: the PDCCH is removed. Entire slot is PDSCH for user 1 and user 2. EVM measurements skip the first two symbols.

PDSCH

PDSCH

PDCCH

PDS

H

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

Not used for EVM measurements

PDSCH

PDSCH

PDCCH

PDSCH

PDCCH

PDSCH

PDSCH

PDSCH

PDCCH

PDS

H

PDSCH

PDSCH

PDSCH

PDSCH

PDSCH

Not used for EVM measurements

PDSCH

PDSCH

PDCCH

PDSCH

PDCCH

PDSCH

Figure 3. Option 10. non-boosting TMs left, TM2 middle, boosting TMs right (sizes not to scale)

The idea is : the first x RBs are split into PDCCH for symbols 0,1, then for symbols 2-13, it is filled with PDSCH

One possible design is:

- PDSCH has the same modulation as PDSCH. Measurements performed on both

- For TM2, no change is applied

- For TM1.2, TM3.2, TM3.3, some possibilities include

  - PDSCH has the same modulation as PDSCH and boosting as PDSCH. PDCCH would also have the same boosting as PDSCH

- EVM measurements apply to all PDSCH after the first 2 symbols

Other possible designs are not precluded.

# Way forwards

- Companies are encouraged to evaluate the implementation aspects for Options 1, 2, 3, 4, 9, 10.

- In addition, minimization of specification impact is a desired goal.

- Testability considerations must be considered

- If design includes unoccupied REs then impact of power scaling needs consideration from RF impacts

- Other options may be considered

A final design is expected next meeting

# References

- R4-1902262, “Ad-hoc meeting minutes for NR conformance testing”, Huawei, RAN4#90, Feb. 25-Mar. 1, 2019