3GPP TSG RAN WG4 Meeting #90bis	R4-1902804

Xian, China, 8th– 12th April, 2019

3GPP TSG RAN WG1 #96			R1-1903810

Athens, Greece, February 25th – March 1st, 2019

Title:	LS on OTA timing alignment for IAB

Release:	Rel-16

Work Item:	NR_IAB-core

Source:	RAN WG1

To:	RAN WG4

Cc:	-

Contact Person:

#### Name:	Xinghua Song

E-mail Address:	songxinghua@huawei.com

- Overall Description:

Agreements (RAN1 #Ad Hoc1901):

- An IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta

  - T_delta is signalled from the parent node, where the value is intended to account for factors such as the offset between parent DL Tx and UL Rx, if any due to factors such as Tx to Rx switching time, HW impairments, etc.

  - TA is the timing gap between UL Tx timing and DL Rx timing, which is derived based on existing Rel-15 mechanism

Agreements (RAN1 #96):

- T_delta is indicated by a parent to the child node independently from the existing Rel.15 TA indication from the parent node used to set the UL Tx timing of the child IAB node’s MT

  - T_delta is updated on an aperiodic basis determined by the parent node

  - The child IAB node should trigger its DL TX timing adjustment by TA/2 + T_delta after it receives the timing offset T_delta indication from its parent node, if it is using OTA Timing Case 1 to obtain its DL timing.

    - FFS: behavior if TA/2 + T_delta results in an effective negative timing offset

    - FFS: delay between receiving T_delta and application of T_delta at the child node

  - Separate value ranges/granularities may be considered for T_delta in FR1 and T_delta in FR2

RAN1 made the following agreements on the over-the-air (OTA) timing alignment for IAB

RAN1 has agreed on the basic mechanism of over-the-air (OTA) timing alignment. As shown in Figure 1, when the switching gap Tg is equal to the TA_offset as defined in TS 38.133, i.e. UL Rx and DL Tx are well aligned, the child node can set its DL TX timing ahead of its DL Rx timing by TA/2, i.e. T_delta = 0. However, when the switching gap Tg is not equal to the TA_offset, i.e. UL Rx and DL Tx are not well aligned, the child node can set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta, where T_delta = (TA_offset - Tg)/2.

RAN1 has not agreed on the exact values of T_delta in terms of its range and granularity though RAN1 agreed that separate values will be defined for FR1 and FR2. It is RAN1’s understanding that it is RAN4’s expertise to determine the exact values of T_delta, which is intended to account for factors such as the gap between uplink reception and downlink transmission timing at the parent node, hardware impairment, etc. It is also RAN1’s understanding that the granularity of T_delta may have an impact to the DL synchronization accuracy across multiple hops for IAB.

2. Actions:

To RAN WG4.

ACTION 1: RAN1 would like RAN4 to provide input on the range and granularity of T_delta in FR1 and FR2.

ACTION 2: RAN1 would like RAN4 to confirm whether the DL synchronization accuracy requirement defined in the current specification should also be applied for multi-hop scenarios for IAB.

ACTION 3: RAN1 would like RAN4 to provide input on the requirement of OTA timing alignment across multiple hops in order to fulfil the DL synchronization accuracy requirements defined in the current specification.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #96bis  8th – 12th Apr 2019	Xi’an, China

TSG-WG1 Meeting #97    13th – 17th May 2019	Reno, USA

Appendix

Figure 1:  Alignment of DL TX and UL RX timing at the parent node