3GPP TSG-RAN WG4 Meeting #90bis                                                                    R4- 1902888

Xi’AN, China, 08 - 12 April, 2019

Agenda Item:	6.10.8.3

Source:	Intel Corporation

Title:	Discussion about L1-RSRP measurement requirement for beam reporting

Document for:	Discussion

# Introduction

In this contribution, we discuss the L1-RSRP beam reporting related topics:

- Rx beam sweeping factor

- Periodic and aperiodic reporting delay

# Rx beam sweeping factor for periodic CSI-RS

For periodic CSI-RS configured with repetition “OFF”, if it’s QCL-TypeD with SSB or CSI-RS with repetition “ON” for L1-RSRP beam reporting, then N=1 can apply. In this case, UE don’t need to do Rx beam sweeping. Otherwise, N=8 shall apply.

For periodic CSI-RS configured with repetition “ON”, if N=1 condition apply, it needs to make sure that the number of CSI-RS resources in the resource set is large enough for UE to do the Rx beam training. However, it may not be guaranteed, therefore N=8 is preferred.

Proposal 1: For periodic CSI-RS configured with repetition “OFF”, if it’s QCL-TypeD with SSB or CSI-RS with repetition “ON” for L1-RSRP beam reporting, then N=1 can apply. Otherwise, N=8 shall apply.

Proposal 2: For periodic CSI-RS configured with repetition “ON”, N=8.

# Reporting delay

For periodic reporting, network will configure the period and offset of the reporting instances via CSI-ReportPeriodicityAndOffset. There is no need to define extra reporting delay.

For aperiodic reporting, RAN1 has already discuss the CSI-RS computing time in 38.214 section 5.4. The reporting delay is dependent on the UE capability, i.e. beamReportTiming and beamSwitchTiming, which applied to both SSB and CSI-RS.  Therefore, we propose to not define reporting for aperiodic reporting for SSB and CSI-RS.

Proposal 3: No need to define aperiodic and periodic reporting delay requirement.

# Conclusion

In this contribution, the following conclusion can be drawn:

Proposal 1: For periodic CSI-RS configured with repetition “OFF”, if it’s QCL-TypeD with SSB or CSI-RS with repetition “ON” for L1-RSRP beam reporting, then N=1 can apply. Otherwise, N=8 shall apply.

Proposal 2: For periodic CSI-RS configured with repetition “ON”, N=8.

Proposal 3: No need to define aperiodic and periodic reporting delay.

# References