3GPP TSG-WG RAN4 Meeting #90bis	R4-1903005

# Xian, China, 8th – 12th April, 2019

Source: 	Nokia, Nokia Shanghai Bell, [AT&T]

Title: 					TP for TR 37.716-11-11 Introduction of DC_66A-66A_n5A and DC_66A-66A-66A_n5A

Agenda Item: 			9.3.2 [DC_R16_1BLTE_1BNR_2DL2UL-Core]

Document for:	Approval

# Introduction

This TP introduces two-band EN-DC configurations of DC_66A-66A_n5A and DC_66A-66A-66A_n5A to TR 37.716-11-11.

**************************** Start of changes ********************************************

## 6.1.X	DC_66-66_n5 and DC_66-66-66_n5

### 6.1.X.1	Operating bands for DC

Table 6.1.X.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_66_n5 | 66 | n5 | No |

### 6.1.X.2	Configuration for DC

Table 6.1.X.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_66A-66A_n5A | DC_66A_n5A | CA_66A-66A | n5A |
| DC_66A-66A-66A_n5A | DC_66A_n5A | CA_66A-66A-66A | n5A |

### 6.1.X.3	Maximum output power for DC

The maximum output power for the uplink EN-DC configuration, DC_66A_n5A, is already specified in Rel.15 TS 38.101-3 Table 6.2B.1.3-1.

### 6.1.X.4	Spurious emission band UE co-existence for DC

The requirements for the uplink EN-DC configuration, DC_66A_n5A, are already specified in Rel.15 TS 38.101-3 Table 6.5B.3.3.1-1.

### 6.1.X.5	MSD analysis for DC

The co-existence study is already made in TR 37.716-11-11 for the uplink EN-DC configuration, DC_66A-n5A in Rel-15. The 2nd and 5th IMD may fall into the band n5 downlink and Pcell REFSENS excpetions can be represented by the REFSENS expcetion already specified for DC_66A-n5A.

### 6.1.X.6	∆TIB and ∆RIB values

For DC_66-66_n5 and DC_66-66-66_n5, the TIB,c and RIB,c values are derived from DC_66_n5 and are given in the tables below.

Table 6.X.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66-66_n5, DC_66-66-66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |

Table 6.X.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66-66_n5, DC_66-66-66_n5 | 66 | 0 |
|  | n5 | 0 |

**************************** End of changes ********************************************