3GPP TSG-RAN WG4 #90BIS Meeting                                        	R4-1903038

Xi’an, China, 08 – 12 April, 2019

Source: 	LG Electronics

Title: 	Co-existence simulation results of CLI

Agenda item:	8.2.2

Document for:	Discussion

- Introduction

- Based on the WID, RAN4 needs to perform co-existence study to identify conditions of co-existence among different operators in adjacent channels.

  - (Target no or very minimal impact on RF requirement)

- Complete core and performance requirements in time [2]

- Companies are encouraged to provide co-existence simulation results based on agreed simulation scenarios and parameters.

In RAN4 #90 meeting, WF on co-existence study of Cross-Link Interference (CLI) [1] was agreed as follows:

To see the feasibility of operating dynamic TDD without CLI mitigation schemes, NR deployment scenarios including macro-to-macro, Indoor-to-macro, micro-to-micro, and indoor-to-indoor are considered for co-existence evaluation by using agreed simulation scenarios [1] and assumptions [3]. In this contribution, our observations and proposals on the co-existence evaluation in the indoor scenario are presented.

- Simulation scenarios and assumptions

- FR1 (4 GHz) and FR2 (30 GHz)

- Macro (aggressor) → Macro (victim)

- Indoor (aggressor) → Macro (victim)

- Indoor (aggressor) → Indoor (victim)

- FR1 (4 GHz) only

- Macro (aggressor) → Indoor (victim)

- FR2 (30 GHz) only

- Micro (aggressor) → Micro (victim)

In [1], the co-existence evaluation scenarios of CLI were agreed as follows:

For the co-existence evaluation of CLI, both BS-to-BS interference and UE-to-UE interference were considered and traffic mode was used as full buffer. The ACLR, ACS requirements and RF components of FR1 are the same as LTE and they are reused from [4]. For the ACLR requirements of FR2, they are reused from [5]. SINR statistic was chosen to get CDF curves at 5%-tile and 50%-tile with dynamic TDD and no dynamic TDD operations as an evaluation metric. Moreover, system bandwidths of 100 MHz and 200 MHz were used for FR1 and FR2 with 30 kHz subcarrier spacing, respectively. More details of the simulation assumptions and parameters can be found in [3].

Table 1 Simulation assumptions in the indoor scenario for FR1

| Layout | Indoor hotspot (Indoor-to-Indoor) : Total 12 BSs (operator A:6 BSs & operator B:6 BSs) 120m x 50m |
| --- | --- |
| Inter-BS distance | 20m |
| Minimum BS-UE (2D) distance | 0m |
| Minimum UE-UE (2D) distance | 1m~3m |
| Carrier frequency | 4GHz |
| BS TX power | 24 dBm per TRX i.e. local area BS, 4T4R 30dBm total |
| UE TX power | 23 dBm |
| Path-loss model | - Indoor (Aggressor) → Indoor (Victim) - BS-to-BS: InH-office [TR 38.803] - BS-to-UE: InH-office [TR 38.803] - UE-to-UE: InH-office [TR 38.803] |
| BS antenna | FR1 BS antenna element pattern for Indoor scenario from [2] / ceiling |
| BS antenna height: | 3m |
| UE antenna | Omni |
| UE antenna height | 1.5m |
| eNB antenna element gain | 3.5 dBi |
| Antenna gain of UE | 0 dBi |
| Cell selection criteria | Cell selection is based on RSRP |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| UE power control | Power control as defined in Section 3.6 |
| Multi operators layout | uncoordinated operation (100% Grid Shift) |

Table 2 Simulation assumptions in the indoor scenario for FR2

| Network layout | Indoor hotspot (Indoor-to-Indoor) : Total 12 BSs (operator A:6 BSs & operator B:6 BSs) 120m x 50m |
| --- | --- |
| Inter-site distance | 20 m |
| BS antenna height | 3 m (ceiling) |
| Path-loss model | - Indoor (Aggressor) → Indoor (Victim) - BS-to-BS: InH-office [TR 38.803] - BS-to-UE: InH-office [TR 38.803] - UE-to-UE: InH-office [TR 38.803] |
| Shadowing correlation | NA |
| Multi operators layout for indoor | Uncoordinated operation (100%) |

- Initial simulation results

In this section, we summarize our initial simulation results of the indoor scenario. As shown in Figure 1 and Figure 2, the same transmission direction of DL was applied to the victim cell and DL/UL transmission direction was applied to the aggressor cell. Here, we have made an assumption that misalignment of transmission direction between the victim cell and the aggressor cell can be quite a large (100%).  From the result of CDF in Figure 1, there was no performance degradation on the victim DL whether the transmission direction of the aggressor was DL or UL. On the other hand, some performance degradation was found on the victim DL when transmission direction of the aggressor was UL in Figure 2.

Figure 1 Indoor-to-Indoor (victim: DL) SINR result at 4 GHz

Figure 2 Indoor-to-Indoor (victim: DL) SINR result at 30 GHz

Observation 1: There was no performance degradation on the victim DL when the transmission direction was UL 100% at 4 GHz.

Observation 2: At 30 GHz, there was some performance degradation when the transmission direction of the aggressor was UL 100%.

As shown in Figure 3 and Figure 4, the same transmission direction of UL was applied to the victim cell and DL/UL transmission direction was applied to the aggressor cell. Like Indoor-to-Indoor (victim: DL) case, the same assumption has been made that misalignment of transmission direction between the victim cell and the aggressor cell can be quite a large (100%). Compared to the case where the transmission direction of the victim cell was DL at 4 GHz, there was slight performance degradation when the transmission direction of the aggressor was DL at 30 GHz.

Figure 3 Indoor-to-Indoor (victim: UL) at 4 GHz

Figure 4 Indoor-to-Indoor (victim: UL) at 30 GHz

Observation 3: When the transmission direction of the aggressor cell was DL 100% at 4 GHz, there was no performance degradation.

Observation 4: At 30 GHz, there was slight performance degradation when the transmission direction of the aggressor was DL 100%.

Table 3 shows the throughput results of the indoor scenario. For 4 GHz, when the transmission direction of the victim was DL, there was no throughput loss on DL aggressor at 5%-tile and 50%-tile. In case the victim’s transmission direction was UL, the throughput results were decreased by about 0.96% and 1.88% at 5%-tile and 50%-tile, respectively.

Observation 5: At 4 GHz, in case the victim’s transmission direction was UL, the throughput results were decreased by about 0.96% and 1.88% at 5%-tile and 50%-tile, respectively.

At 30 GHz, there was some performance degradation when the transmission direction of the victim cell was DL and UL. The throughput results on DL victim were decreased by about 16.65% and 8.81% at 5%-tile and 50%-tile, respectively. In case the transmission direction of the victim cell was UL, the throughputs were also decreased by 7.32% and 1.32% at 5%-tile and 50%-tile, respectively.

Observation 6: At 30 GHz, in case the victim’s transmission direction was DL with the dynamic TDD operation (Aggressor: DL → UL), the throughput results were decreased by about 16.65% and 8.81% at 5%-tile and 50%-tile, respectively.

Observation 7: At 30 GHz, in case the victim’s transmission direction was UL with the dynamic TDD operation (Aggressor: UL → DL), the throughput results were decreased by about 7.32% and 1.32% at 5%-tile and 50%-tile, respectively.

Table 3 Throughput results for Indoor-to-Indoor scenario

|  | Throughput (Mbps) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 GHz |  |  |  | 30 GHz |  |  |  |
|  | Victim: DL |  | Victim: UL |  | Victim: DL |  | Victim: UL |  |
|  | Aggressor: DL | Aggressor: UL | Aggressor: UL | Aggressor: DL | Aggressor: DL | Aggressor: UL | Aggressor: UL | Aggressor: DL |
| 5%-title | 12.971 | 13.098 | 15.161 | 15.015 | 44.078 | 36.740 | 39.641 | 36.740 |
| Loss (%) | 0 |  | 0.96 |  | 16.65 |  | 7.32 |  |
| 50%-tile | 41.449 | 41.901 | 38.072 | 37.355 | 135.807 | 123.839 | 125.492 | 123.839 |
| Loss (%) | 0 |  | 1.88 |  | 8.81 |  | 1.32 |  |

Based on our evaluation results, there was no performance degradation when operating the dynamic TDD in the indoor scenario at 4 GHz. However, some performance degradation was found in the indoor scenario at 30 GHz due to the fact that the ACIR requirement of FR2 is more tighten than FR1. Although, we might have some throughput loss at 30 GHz, still it is feasible to operate the dynamic TDD operation in the indoor scenario and it would result in less impact of cross-link interference.

- Conclusion

Based on the simulation results above, the following observations have been made:

- Observation 1: There was no performance degradation on the victim DL when the transmission direction was UL 100% at 4 GHz.

- Observation 2: At 30 GHz, there was some performance degradation when the transmission direction of the aggressor was UL 100%.

- Observation 3: When the transmission direction of the aggressor cell was DL 100% at 4 GHz, there was no performance degradation.

- Observation 4: At 30 GHz, there was slight performance degradation when the transmission direction of the aggressor was DL 100%.

- Observation 5: At 4 GHz, in case the victim’s transmission direction was UL, the throughput results were decreased by about 0.96% and 1.88% at 5%-tile and 50%-tile, respectively.

- Observation 6: At 30 GHz, in case the victim’s transmission direction was DL with the dynamic TDD operation (Aggressor: DL → UL), the throughput results were decreased by about 16.65% and 8.81% at 5%-tile and 50%-tile, respectively.

- Observation 7: At 30 GHz, in case the victim’s transmission direction was UL with the dynamic TDD operation (Aggressor: UL → DL), the throughput results were decreased by about 7.32% and 1.32% at 5%-tile and 50%-tile, respectively.

According to the observations, it is feasible to operate dynamic TDD in the indoor scenario for FR1 without changing the current RF requirements and CLI mitigation schemes. In contrast, FR2 may need CLI mitigation schemes.

References

- R4-1902506, WF on co-existence study of CLI

- R4-1900222, Work plan of Cross Link Interference (CLI) and Remote Interference Management (RIM) for NR

- R4-1902507, WF on simulation assumptions of co-existence study for CLI

- TR 36.828, Further enhancements to LTE Time Division Duplex (TDD) for Downlink-Uplink (DL-UL) interference management and traffic adaptation

- TR 38.803, Radio Frequency (RF) and co-existence aspects (Release 14)