3GPP TSG-WG RAN4 Meeting #90bis	R4-1903078

# Xian, China, 8th – 12th April, 2019

Source: 	Nokia, Nokia Shanghai Bell, [AT&T]

Title:	TP for TR 37.716-21-11 DC_30A-66A_n5A, DC_30A-66A-66A_n5A and DC_30A-66A-66A-66A_n5A

Agenda Item: 			9.4.2 [DC_R16_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# Introduction

This TP introduces two-band EN-DC configurations of DC_30A-66A_n5A, DC_30A-66A-66A_n5A and DC_30A-66A-66A-66A_n5A to TR 37.716-21-11.

**************************** Start of changes ********************************************

## 5.1.X	DC_30-66_n5, DC_30-66-66_n5 and DC_30-66-66-66_n5

### 5.1.X.1	Operating bands for DC

Table 5.1.X.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_30-66_n5 | CA_30-66 | n5 | No |

### 5.1.X.2	Configuration for DC

Table 6.1.X.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_30A-66A_n5A | DC_30A_n5A DC_66A_n5A | CA_30A-66A | n5A |
| DC_30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A | CA_30A-66A-66A | n5A |
| DC_30A-66A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A | CA_30A-66A-66A-66A | n5A |

### 5.1.X.3	Coexistence studies

Based on co-existence studies of DC_66A_n5A in TR 37.863-01-01, the 2nd and 5th IMD of 66+n5 may fall into the band n5 downlink and Pcell REFSENS excpetions can be represented by the REFSENS expcetion already specified for DC_66A-n5A.

Based on co-existence studies of DC_30A_n5A in TR 37.863-01-01, 5th IMD product of 30+n5 may fall into band 66 downlink.

### 5.1.X.4	∆TIB and ∆RIB values

For DC_30-66_n5, DC_30-66-66_n5 and DC_30-66-66-66_n5, the TIB,c and RIB,c values are derived from CA_5-30-66 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30-66_n5, DC_30-66-66_n5, DC_30-66-66-66_n5 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_30-66_n5, DC_30-66-66_n5, DC_30-66-66-66_n5 | 30 | 0 |
|  | 66 | 0.4 |
|  | n5 | 0.5 |

### 5.1.X.5	Refsens requirements

Pcell REFSENS exceptions for band n5 due to 2 uplink of band 66 and band n5 can be represented by the REFSENS expcetion already specified for DC_66A-n5A in TS 38.101-3.

Scell REFSENS exceptions for band 66 due to 2 uplink of band 30 and band n5, MSD is specified in Table 5.1.X.5-1

Table 5.1.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_30A-66A_n5A | 30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | 66 | 2133 | 5 | 25 | 2133 | [2.5 dB] | FDD | IMD5 |
|  | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |

**************************** End of changes ********************************************