3GPP TSG-RAN4 Meeting #90bis	R4-1903215

# Xi’an, China, 8th Apr – 12th Apr, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RRC-based BWP switch delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-3-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specific requirements for RRC-based BWP switch delay are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete RRC-based BWP switch delay requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRC-based BWP switch delay requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

8.6.3	RRC based BWP switch delay

For RRC-based BWP switch, after the UE receives BWP switching request, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot right after the beginning of DLslot n+ TRRCprocessingDelay + TBWPswitchDelayRRC, where DL slot n is the last slot containing the RRC command, and TRRCprocessingDelay is the length of the RRC procedure delay in slots defined in clause 12 in TS 38.331 [2], and TBWPswitchDelayRRC is the BWP switching delay for RRC based BWP switch.

When a BWP-related capability of bwp-WithoutRestriction or bwp-SameNumerology or bwp-DiffNumerology is present, UE shall complete BWP switch within the time duration TBWPswitchDelayRRC defined in Table 8.6.3-1.

Table 8.6.3-1: BWP switch delay TBWPswitchDelayRRC

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelayRRC (slots) |
| --- | --- | --- |
| 0 | 1 | [3] |
| 1 | 0.5 | [5] |
| 2 | 0.25 | [9] |
| 3 | 0.125 | [17] |
| Note 1: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |

When BWP-related capabilities of bwp-WithoutRestriction and bwp-SameNumerology and bwp-DiffNumerology are absent, if new BWP configurations are added, UE shall complete BWP switch within the time duration TBWPswitchDelayRRC defined in Table 8.6.3-1. If no new BWP configurations are added, UE shall complete BWP switch within the time duration TBWPswitchDelayRRC defined in Table 8.6.3-2.

Table 8.6.3-2: BWP switch delay TBWPswitchDelayRRC

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelayRRC (slots) |
| --- | --- | --- |
| 0 | 1 | [2] |
| 1 | 0.5 | [4] |
| 2 | 0.25 | [8] |
| 3 | 0.125 | [15] |
| Note 1: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |

<< End of Change >>