3GPP TSG-RAN WG4 Meeting #90b              	R4-1903365

Xi’an, China, 8th – 12th April, 2019

Agenda item:	6.12.2.3.1

Source:	Samsung

Title:	Remaining issues on performance requirements for NR PUCCH in Rel-15

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remained issues for NR BS demodulation performance requirements are further discussed. The performance requirement of Multi-slot PUCCH are agreed and captured in the WF [1] as follows:

- PUCCH format

- PUCCH format1

- PUCCH format 3

- Number of repetition slots

- 2

- Frequency Hopping

- Option1: inter-slot frequency hopping enabled and intra-slot frequency hopping disabled

- Option2: both intra and inter slot frequency hopping are disabled.

- Test applicability

- PUCCH format1

- The agreements defined for multi-slot PUCCH only apply to the BS supporting multi-slot PUCCH

In this contribution, the view of remained issue for PUCCH is provided.

# 2	Discussion

## 2.1	Multi-slot PUCCH

In order to improve UL coverage, Multi-slot PUCCH transmission method is supported. Based on RAN1 UE feature list, PUCCH format 1/3/4 can support multi slot transmission, the number of slot can be supported with 2/4/8. For PUCCH repetition, the starting OFDM symbol and during within a slot is always same.

The performance gain achieved by multi-slots transmission, depends on many factors, such as practical channel estimation, frequency hopping type, as well as practical LLR combination method. For example, in case of multi-slot transmission, whether cross-slot channel estimation will be applied. In case of frequency hopping, both frequency hopping across different slots and within a slot can be supported, the hopping pattern is different for two kinds of hopping type. As agreed in the last meeting, the requirement of multi-slot PUCCH cannot add a unified value compared with single –slot PUCCH requirement simply.

For PUCCH format 1/3/4, RAN4 has already specified the performance requirement with following test case.

Table 1:  Simulation assumption for long short PUCCH formats

| Parameters |  | PUCCH format 1 | PUCCH format 3 | PUCCH format 4 |
| --- | --- | --- | --- | --- |
| BW |  | FR1: 15KHz 5MHz, 10MHz, 20MHz 30KHz, 10MHz, 20MHz, 40MHz, 100MHz FR2: 60KHz, 50MHz, 100MHz 120KHZ, 50MHz, 100MHz, 200MHz |  |  |
| Antenna configuration |  | 1T2R, 1T4R and 1T8R for FR1, 1T2R for FR2 |  |  |
| Propagation Condition |  | FR1 : TDLC300-100: FR2: TDLA30-300 |  |  |
| Carrier frequency (GH) |  | FR1: 4GHz: FR2:30GHz |  |  |
| (#Bit, # Symbols, #PRB) |  | (2,14,1) | (16,14,1) (16,4,3) | (22,14,1) |
| DMRS pattern |  |  | FR1: Without additional DMRS for all cases With additional DMRS for cases with the number of OFDM symbols more than 9 FR2: Without and with additional DMRS |  |
| Modulation |  |  | QPSK |  |
| Starting Symbol Index |  | 0 |  |  |
| Index of orthogonal |  | 0 |  |  |
| Occ-Length |  |  | n2 |  |
| Occ-Index |  |  |  |  |
| Test metric | DTX to ACK probability < 1% | √ | x |  |
|  | Missed ACK probability < 1% | √ | x |  |
|  | NACK2ACK <0.1% | √ | x |  |
|  | BLER < 1% | x | √ |  |

Currently, RAN4 has specified many test cases for single-slot PUCCH transmission, heavy workload to align and finalize the performance requirement are still needed.

Obviously, defining the requirements for Format 1/3/4 with multi-slot transmission under all the supported number of slots, is impractical, considering it need to cover the defined SCS and BW combination.  In order to reduce the effort, we need to down select the test cases for multi-slot PUCCH.

As agreed, two options are listed for multi-slot PUCCH as follows:

- PUCCH format1

- PUCCH format 3

For format1, the UCI bit is less than 2 bits, mainly transmit HARQ-ACK or SR information.  The structure is based on the CGS sequence and block -wise spread with an orthogonal cover code, there is no coding gain achieved compared with format 3. Meanwhile, HARQ-ACK or SR has more high priority than CSI information. The HARQ-ACK or SR transmission should be guaranteed, especially in the cell edge scenario.

For Format 3, the number of RB can be configured to support 16 RB; the number of allocated symbols can be configured 4 to 14. The transmission structure is based on DFT-s-OFDM, which has low PAPR characteristic compared with CP-OFDM and can have good coverage. In order to match the coverage, the low coding rate can be chosen aiming to achieve coding gain, which can be controlled by the number of allocated RBs (and the number of allocated symbols) for the PUCCH format 3 transmission. Meanwhile, additional DMRS can be supported; with good channel estimation results can achieve better performance.

In the previous meeting, one company submitted the initial simulation results with comparison of LTE single slot and NR PUCCH with multi-slot transmission. Although the simulation assumption is different with RAN4 agreed for NR PUCCH requirements, it can provide some insight of the performance gain with multi-slot PUCCH. Also, based on the simulation result in this contribution, format 1 is suffered with the coverage issue more seriously compared with format 3. Hence, we prefer to specify the multi-slot PUCCH format 1with high priority.

Proposal 1: Priority to specify the performance requirement with Format 1 for multi-slot PUCCH transmission

As for the frequency hopping, UE can be configured. The UE transmits the PUCCH starting a first PRB, provided by the higher layer parameter starting PRB in the slots with even number and starting from the second PRB, provided by higher layer parameter secondHop PRB in slots with odd number. While for the frequency hopping within a slot, the hopping pattern between the first PRB and the second PRB is same with each slot. For single-slot PUCCH transmission, the frequency hopping is enabled. Compared with intra-slot frequency hopping, the frequency diversity and time diversity again are expected to achieve more in the inter-slot frequency hopping pattern, considering the channel during one slot is

Thus, we prefer to define performance requirement for inter slot frequency hopping enabled for multi-slot PUCCH transmission.

For the hopping pattern,

- Inter slot frequency hopping: enable

- StartingPRB =0 for slot with even number

- SecondHop = the largest PRB index - nrofPRBs for slot with odd number

Proposal 2: Inter slot frequency hopping is enable across different slots for multi-slot PUCCH transmission, frequency hopping within a slot is not configured.

- Intra slot frequency hopping: disable

- Inter slot frequency hopping: enable

- StartingPRB =0 for slot with even number

- SecondHop = the largest PRB index - nrofPRBs  for slot with odd number

As for the number of slot for transmission, NR can support 2/4/8 slots, as agreed in last meeting, 2 number of slot repetition is agreed.

As for the test cases for multi-slot PUCCH, in order to, we prefer to reuse the test cases defined for single slot PUCCH. For format 1, 2 slot repetitions with (2 bit, 14 symbol, and 1RB) are chosen as the requirement of multi-slot PUCCH.

Proposal 3: Reuse single PUCCH format 1 with 2 slot repetitions as the test case of multi-slot PUCCH.

Since RAN4 group has already defined the test applicability with SCS &combination, so, we prefer to down select the case from the agreed SCS & BW combination to guarantee the requirements can be tested, aiming to reduce the test effort and simulation time effort. Thus, the chosen SCS& BW combination is preferred as follows:

- FR1: 15KHz, 5MHz, 30KHz, 10MHz,

- FR2: 50KHz, 50MHz, 120KHz, 50MHz

Proposal 4: Down select SCS&BW from the agreed test cases,   prefer specified the multi-slot PUCCH requirements with the following SCS &BW:

- FR1: 15KHz, 5MHz, 30KHz, 10MHz,

- FR2: 50KHz, 50MHz, 120KHz, 50MHz

The detail simulation assumption can be referred as the follows table

Table 2:  Initials simulation assumption for format 1 with multi-slot transmission

| Parameters |  | PUCCH format 1 |
| --- | --- | --- |
| BW |  | FR1: 15KHz 5MHz, 30KHz, 10MHz, FR2: 60KHz, 50MHz, 120KHz, 50MHz |
| Antenna configuration |  | 1T2R, 1T4R and 1T8R for FR1, 1T2R for FR2 |
| Propagation Condition |  | FR1 : TDLC300-100: FR2: TDLA30-300 |
| Carrier frequency (GH) |  | FR1: 4GHz: FR2:30GHz |
| (#Bit, # Symbols, #PRB) |  | (2,14,1) |
| DMRS pattern |  |  |
| Modulation |  | QPSK |
| Starting Symbol Index |  | 0 |
| Index of orthogonal |  | 0 |
| Occ-Length |  |  |
| Occ-Index |  |  |
| Number of repetition slots |  | 2 |
| Inter-slot frequency hopping |  | enable |
| Intra-slot frequency hopping |  | disable |
| Hopping pattern |  | StartingPRB =0 for slot with even number SecondHop = the largest PRB index - nrofPRBs for slot with odd number |
| Test metric | DTX to ACK probability < 1% | √ |
|  | Missed ACK probability < 1% | √ |
|  | NACK2ACK <0.1% | √ |
|  | BLER < 1% | x |

# 3	Conclusion

In this contribution, we provided our view about the remained issue of NR PUCCH demodulation requirement.

Proposal 1: Priority to specify the performance requirement with Format 1 for multi-slot PUCCH transmission

Proposal 2: Inter slot frequency hopping is enable across different slots for multi-slot PUCCH transmission, frequency hopping within a slot is not configured.

- Intra slot frequency hopping: disable

- Inter slot frequency hopping: enable

- StartingPRB =0 for slot with even number

- SecondHop = the largest PRB index - nrofPRBs  for slot with odd number

Proposal 3: Reuse single PUCCH format 1 with 2 slot repetitions as the test case of multi-slot PUCCH

Proposal 4: Down select SCS&BW from the agreed test cases, prefer to specify the multi-slot PUCCH requirements with the following SCS &BW:

- FR1: 15KHz, 5MHz, 30KHz, 10MHz,

- FR2: 50KHz, 50MHz, 120KHz, 50MHz

# References

R4-1902440, WF on NR PUCCH demodulation requirements in Rel-15, Huawei