3GPP TSG RAN WG4 Meeting #90bis		R4-1903412

Xian, China, 8th– 12th April, 2019

Source: 	NTT DOCOMO, INC.

Title: 	On scaling factor N

Agenda item:	6.10.7.1.2

Document for:	Discussion

- Introduction

RAN4 has discussed about requirements for RLM, but there has been still remaining issues including PDCCH transmission parameters for OOS/IS and N=1 condition. In this contribution, we provide our views on remaining issues regarding scaling factor N.

- Discussion

At the last RAN4 meeting, there was discussion on conditions to apply N=1 for RLM. Followings are the current specification based on the agreements [1].

8.1.2 Requirements for SSB based radio link monitoring 8.1.2.2 Minimum requirement UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period. UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] period becomes better than the threshold Qin_SSB within TEvaluate_in_SSB [ms] evaluation period. TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-1 for FR1. TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-2 for FR2 with - N=1, if the SSB configured for RLM is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting, and the QCL association is known to UE; - N=8, otherwise. ### 8.1.3 Requirements for CSI-RS based radio link monitoring 8.1.3.2 Minimum requirement UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS [ms] period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS [ms] evaluation period. UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS [ms] period becomes better than the threshold Qin_CSI-RS within TEvaluate_in_CSI-RS [ms] evaluation period. - TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-1 for FR1. - TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-2 for FR2, where - N=1, if the CSI-RS resource configured for RLM is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting or SSBs configured for L1-RSRP reporting, all CSI-RS resources configured for RLM are mutually TDMed, and the QCL association is known to UE; - N=8, otherwise.

There still was argument whether to keep remaining condition as highlighted in light blue. Regarding SSB-based RLM, if SSB configured for RLM is QCL-Type D with CSI-RS for L1-RSRP reporting, UE can train Rx beams using CSI-RS for beam reporting. In the current situation, RAN4 has already agreed that UE performs Rx beam sweeping on CSI-RS configured for L1-RSRP reporting, and current description should be kept in TS 38.133. Otherwise, time duration for Rx beam sweeping is reserved for both of SSB and CSI-RS, which seems not necessary. Based on above discussion, we made following proposal.

Proposal 1: Following condition of N=1 for SSB based RLM should be kept in TS 38.133.

- if the SSB configured for RLM is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting, and the QCL association is known to UE

For CSI-RS based RLM, the condition needs to be kept, since Rx beam information is available based on SSB or CSI-RS that is used for beam management. Following proposal is made based on the discussion.

Proposal 2: Following condition of N=1 for CSI-RS based RLM should be kept in TS 38.133.

- if the CSI-RS resource configured for RLM is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting or SSBs configured for L1-RSRP reporting, all CSI-RS resources configured for RLM are mutually TDMed, and the QCL association is known to UE

- Conclusion

In this contribution, we provided our views on requirements of radio link monitoring, and we made following proposals.

Proposal 1: Following condition of N=1 for SSB based RLM should be kept in TS 38.133.

- if the SSB configured for RLM is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting, and the QCL association is known to UE

Proposal 2: Following condition of N=1 for CSI-RS based RLM should be kept in TS 38.133.

- if the CSI-RS resource configured for RLM is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting or SSBs configured for L1-RSRP reporting, all CSI-RS resources configured for RLM are mutually TDMed, and the QCL association is known to UE

# Reference

- 3GPP, TS38.133 v15.4.0, Dec. 2018.