3GPP TSG-RAN WG4 Meeting #90bis	R4-1903416

Xian, China, 8th– 12th April, 2019

Agenda item:	6.12.1.4.3

Source: 		NTT DOCOMO, INC.

Title: 	Simulation results for PMI test in FR2

Document for:	Discussion

1. Introduction

This contribution presents our simulation results for following PMI test in FR2 (DDDSU) [1].

2. Simulation results

We follow simulation assumption in Table. A1, which is identical to the simulation assumption of Test 4.6. Figure 1 and 2 show simulation results for follow PMI tests for 2Tx2Rx ULA Low and relative throughput ratio of Figure 1, respectively.

Figure 1: Test 4.6 (ULA Low)

Figure 2: Relative Throughput ratio (Following / Random)

PMI requirements are specified in terms of the ratio as follows.

In the definition of γ, 70 and 90 % of the maximum throughput obtained at SNRUE using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRUE with random precoding.

Table I: Summary of link performance for Test 4.6

| SCS | CBW | Antenna | Test point | Reference SNR point | Test requirement | Difference between each test requirement |
| --- | --- | --- | --- | --- | --- | --- |
| 120 kHz | 100 MHz | 2Tx2Rx (ULA Low) | 90% | 5.5 dB | 1.12 | - |
|  |  |  | 70% | 3.1 dB | 1.20 | +0.08 |

From the link simulation results, we have following proposals.

Proposal:  For FR2 test 4.6 (ULA Low), test requirement shall be 1.12, if test point is designed with 90% of the maximum throughput.

3. Conclusion

This contribution presents our simulation results for following PMI test in FR2 (DDDSU).

Proposals are presented as follows.

Proposal:  For FR2 test 4.6 (ULA Low), test requirement shall be 1.12, if test point is designed with 90% of the maximum throughput.

References

[1] 3GPP, Samsung, R4-1902404 Way forward on NR UE CSI requirements, Mar. 2019.

Appendix

Table A1: FR2 2Tx PMI test cases (test 4.6)

| Parameter | Unit | Test 4.6 FR2 TDD with TDD DL-UL pattern DDDSU |
| --- | --- | --- |
| System parameter |  |  |
| Bandwidth/SCS | MHz/kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 |
| Duplex Mode |  | TDD |
| DLUL configuration |  | DDDSU (FR2.120-1) |
| Propagation channel |  | TDL_A 30ns, 35Hz |
| Correlation and antenna configuration |  | 2*2 Option 1: ULA Low Option 2: ULA Medium |
| CSI-RS resource setting (aperiodic) |  |  |
| resourceConfigType |  | aperiodic |
| NZP CSI-RS ports |  | 2 port {3000,3001} |
| CSI report setting (aperiodic) |  |  |
| reportConfigType |  | aperiodic |
| Reporting interval and slot offset |  | 5/1 |
| (N1,N2) |  | N.A |
| (O1,O2) |  | N.A |
| CodebookMode |  | N.A |
| Codebook Restriction |  | 001111 |
| Reporting granularity |  | Wide-band |
| MCS Table |  | 64QAM |
| Rank |  | 1 |
| FRC |  | MCS13 |
| Physical channel for reporting |  | PUSCH |
| CQI table |  | Table 1 without 256QAM |
| Minimum CSI delay |  | 1.75 ms |