3GPP TSG-RAN WG4 Meeting #90Bis	R4-1903563

Xi’an, China, 8th Apr – 12th Apr, 2019

Source: 	ZTE

Title:	Further consideration of EMC test wanted signal level

Agenda Item:	6.9.3

Document for:	Approval

Introduction

The EMC test wanted signal level has been discussed in [1] and [2]. However, the wanted signal level is not decided as the test method of EMC radiated immunity test has not been settle down and there are strong dependency between this two topics. In RAN4#90, the corresponding EMC RI test method and configuration has been agreed in[3]. Base on the latest version, we would like to provide some further consideration of wanted signal level of EMC test.

Discussion

As the introduction of active antenna system(AAS), the radiated immunity test shared similar test set-up as OTA out-of-band blocking test but with much higher interference level. The comparison is shown below in figure 1-a and 1-b:

Figure 1-a: EMC RI test 														Figure 1-b: OTA out-of-band blocking

The comparison of these two test has been gone over many times during previous discussions. A conclusion table is shown below as table 1:

Table 1 Comparison between RI test and OTA OOB test

|  | Radiated Immunity test | OTA out-of-band blocking test |
| --- | --- | --- |
| Test frequency | 80MHz---6GHz | 30MHz---12.75GHz |
| wanted signal level | EISminSENS + XdB | EISminSENS + 6dB |
| interference signal level | 10V/m | 0.36V/m |
| interference signal type | 1kHz sine wave 80% AM modulated CW signal | CW signal |

It is clearly shown in the table 1 that the wanted signal level of RI test is set to EISminSENS + XdB, where X is an undefined number. However, to ensure the repeat-ability and standardization of specifications, a fixed number should be used instead of an X which leaves the test lab or BS vendor to decide. This is also under discussion in ETSI ERM WGEMC and CCSA TC9 WG1.

When comparing the 0.36V/m to 10V/m. a 29dB more stringent BS RF filter requirement is added. So far, to avoid this, two mechanisms have been proposed as “spatial exclusion” and “extension of exclusion band of RI test”. In current TS 38.113, each of these two can be chosen to achieve compliance. However, there has been one LS[4] sent out to ETSI ERM WGEMC and the reply LS has confirmed that the spatial exclusion method will not be accepted by European regulatory. So the exclusion band extension is proposed and we would like to discuss the wanted signal level based on this mechanism.

In [3], the exclusion band has been extended to 200MHz which ensures the received power after the RF filter at 10V/m interfere is at least 29dB less than that at 0.36/m interfere(the OTA out-of-band blocking requirement). In this case, a similar wanted signal level should be defined for 10V/m case to the OTA OOB blocking case. So far, the  EISminSENS + 6dB is used for OTA OOB blocking. However, the EMC RI test is not considering the RX requirement of the base station. The principle of setting the wanted signal level for a BS within an EMC test is to grantee that the performance criteria will not be limited or influenced by the RF performance, e.g the throughput being considered in EMC test. Hence, a higher wanted signal level comparing to EISminSENS + 6dB should be considered.

In previous 3GPP EMC specs as TS 36.113 and TS 37.113, reference sensitivity + 15dB is provided as an example for conducted case comparing reference sensitivity + 6dB of OOB conducted test.

Observation: There is a 9dB difference between wanted signal level for EMC test and OOB conducted test.

Hence we believe this 9dB difference can be kept for OTA case as based on a calibration before each test, there should be no difference between conducted case and OTA case. So a 9dB is proposed to be added based on EISminSENS + 6dB which means EISminSENS + 15 dB is proposed as wanted signal level for BS type 1-O in EMC test.

Proposal: EISminSENS + 15 dB is proposed as wanted signal level for BS type 1-O in EMC test.

Conclusion

In this contribution, we give analysis of EMC test wanted signal level and proposals are:

Observation: There is a 9dB difference between wanted signal level for EMC test and OOB conducted test.

Proposal: EISminSENS + 15 dB is proposed as wanted signal level for BS type 1-O in EMC test.

- References

R4-1810317 on wanted signal power and spatial exclusion, ZTE

R4-1814760 on wanted signal power for RI test and the comparison with out of band blocking test, ZTE

R4-1902334 - Draft CR to TS 38.113 Exclusion Bands for Radiated Immunity Test, Ericsson

R4-1816739 LS to ETSI ERM WGEMC on 3GPP RAN4 progress on the EMC specification for OTA AAS BS, ZTE, Ericsson