3GPP TSG-RAN WG4 Meeting #90bis	R4-1903572

Xi'an, China, 8th – 12th April 2019

Agenda item:	6.12.2.4	PRACH 	[NR_newRAT-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	NR PRACH for high speed in Rel.15

Document for:	Approval

- Introduction

In the past meetings, the following agreements were reached for BS performance under HST scenarios [1, 2]. In this contribution, we provide simulation assumptions for NR PRACH restricted set A and B.

RAN#81 [1] The UE and BS demodulation performance requirements under HST scenarios are treated in TEI15 after December 2018. RAN4#90 [2] - Companies are encourage to evaluate for RAN4#90bis when they could provide evaluation results for the following scenarios : - PUSCH requirements for high speed scenarios - Option 1 HST (up to 300km/h or 350km/h at Band 7 (2.7GHz) [and Band n77 (3.6GHz)]) - Option 2: Use the same band as LTE to reduce the workload for BS side (i.e., maximum Doppler shift is 1340Hz). - PRACH requirements for high speed scenarios - Preamble format 0 with restricted set A and B NOTE: For above scenarios, 1 additional DMRS is baseline. Other assumption is not precluded. - Operators are encourage to provide the motivation to introduce HST related tests as well as their priority on the test to be introduced in Rel-15 in RAN4 #90bis

2.	Discussion

In BS demodulation, PRACH requirement for burst format 0 with normal mode is specified [3].

TS 38.104 Table 8.4.2.2-1: PRACH missed detection requirements for Normal Mode, 1.25 kHz SCS Number of TX antennas; Number of RX antennas; Propagation conditions and correlation matrix (Annex [TBD]); Frequency offset; SNR (dB). Burst format 0. 1; 2; AWGN; 0; [-15.1]. [TDLC300-100]; 400 Hz; [-6.7]. 4; AWGN; 0; [-17.3]. [TDLC300-100]; 400 Hz; [-12.2]. 8; AWGN; 0; [-19.6]. [TDLC300-100]; 400 Hz; [-16.3]. TS 38.211 Table 6.3.3.1-1: PRACH preamble formats for and kHz. Format; Support for restricted sets. 0; 839; 1.25 kHz; Type A, Type B. 1; 839; 1.25 kHz; Type A, Type B. 2; 839; 1.25 kHz; Type A, Type B. 3; 839; 5 kHz; Type A, Type B.

In the current RAN4 specification [3], PRACH requirements for only normal mode are defined. PRACH requirements for normal mode supports frequency offset of up to 400Hz, which is not enough for HST scenario, since very larger Doppler frequency offset should be considered. In LTE, PRACH requirements for high-speed mode with restricted set A and B are specified to ensure the performance under HST scenarios. Similarly, in NR, taking into account HST scenarios, test cases for restricted set A and B under larger frequency offset corresponding to the assumed maximum Doppler shift should be defined.

Observation 1: For HST supports, PRACH requirements for restricted set A and B are needed.

According to TS 38.211 [4], restricted set A and B can be configured to all PRACH burst formants of long sequence. When we consider macro scenario, not only burst format 0 but also other formats should be considered, however, at this moment, it is compromised to consider only burst format 0 in Rel.15 in order to reduce the work and support HST. When requirements for other burst formats in the future are requested, it should be discussed to add corresponding requirements to RAN4 specification in TEI-15.

Proposal 1: For HST support, define PRACH requirements for restricted set A and B for burst format 0 in Rel.15. PRACH requirements for burst format 1, 2 and 3 will be added to RAN4 specification in TEI-15.

For restricted set A, it is enough to consider parameters equivalent to LTE. On the other hand, for restricted set B, at least maximum Doppler shift (i.e., 2334Hz) based on UE speed 350km/h at carrier frequency 3.6GHz should be considered as frequency offset conditions. As parameters for PRACH restricted set A and B tests, Table 4 is proposed.

Proposal 2: Consider parameters for PRACH restricted set type A and B tests as follow:

Table 1: Proposed parameters for NR high speed train test

| Parameter | value |  |
| --- | --- | --- |
|  | Restricted set type A | Restricted set type B |
| Number of Tx | 1 | 1 |
| Number of Rx | 2, 4, 8 | 2, 4, 8 |
| Preamble format | Format 0 | Format 0 |
| Frequency offset / Propagation condition | 0Hz: AWGN 400Hz: TDLC300-100 800Hz: AWGN 1340Hz: AWGN | 0Hz: AWGN 400Hz: TDLC300-100 800Hz: AWGN 2334Hz or 2500Hz: AWGN |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Testing metric | False alarm probability: 0.1% missed detection: 99% | False alarm probability: 0.1% missed detection: 99% |

3.	Conclusion

In this contribution, we provide our views on PRACH for HST. The following observation and proposals are obtained.

Observation 1: For HST supports, PRACH requirements for restricted set A and B are needed.

Proposal 1: For HST support, define PRACH requirements for restricted set A and B for burst format 0 in Rel.15. PRACH requirements for burst format 1, 2 and 3 will be added to RAN4 specification in TEI-15.

Proposal 2: Consider parameters for PRACH restricted set type A and B tests as follow:

Table 1: Proposed parameters for NR high speed train test

| Parameter | value |  |
| --- | --- | --- |
|  | Restricted set type A | Restricted set type B |
| Number of Tx | 1 | 1 |
| Number of Rx | 2, 4, 8 | 2, 4, 8 |
| Preamble format | Format 0 | Format 0 |
| Frequency offset / Propagation condition | 0Hz: AWGN 400Hz: TDLC300-100 800Hz: AWGN 1340Hz: AWGN | 0Hz: AWGN 400Hz: TDLC300-100 800Hz: AWGN 2334Hz or 2500Hz: AWGN |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Testing metric | False alarm probability: 0.1% missed detection: 99% | False alarm probability: 0.1% missed detection: 99% |

References

- RP-182149, “Way forward on RAN4 work for Core and Perf. part for NR”, RAN4 Chairman, RAN #81, September 2018

- R4-1902442, “WF on high speed related requirements for NR BS demodulation in Rel.15”, NTT DOCOMO, INC., China Telecom, RAN4 #90, February 2019

- “3GPP TS 38.104: NR; Base Station (BS) radio transmission and reception”, 3GPP

- “3GPP TS 38.211: NR; Physical channels and modulation”, 3GPP