3GPP TSG-RAN4 Meeting #90bis	R4-1903788

# Xi’An, China, 8th – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of L1-RSRP delay test in FR2 (section A.5.6, A.7.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | L1-RSRP delay test cases for FR2 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test cases for L1-RSRP measurement delay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L1-RSRP measurement delay requirements are not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.6, A.7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.5.6.3	L1-RSRP measurement for beam reporting

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used

A.5.6.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.5.6.3.1.1-1.

The AoA setup for this test is Setup 1 as defined in section A.3.15

Table A.5.6.3.1.1-1: Applicable NR configurations for FR2 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

A.5.6.3.1.2	Test parameters

There are two cells in the test, E-UTRAN PCell (Cell 1) and FR1 PSCell (Cell 2). The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 are given in Table A.5.6.3.1.2-1 and Table A.5.6.3.1.2-2 below.

In CSI measurement configuratation, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1, and T2 respectively.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.5.6.3.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |
| Duplex mode | 1~4 |  | TDD |
| TDD Configuration | 1~4 |  | TDDConf.3.1 |
| BWchannel | 1~4 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~4 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~4 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~4 |  | CCR.3.1 TDD |
| SSB configuration | 1,2 |  | SSB.1 FR2 |
|  | 3,4 |  | SSB.2 FR2 |
| OCNG Patterns | 1~4 |  | OP.1 |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~4 |  | SMTC.1 |
| TRS Configuration | 1~4 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~4 |  | TCI.State.2 |
| DRX configuration | 1~4 |  | Off |
| reportConfigType | 1~4 |  | periodic |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |
| Number of reported RS | 1~4 |  | 2 |
| L1-RSRP reporting period | 1~4 | slot | 640 |
| T1 | 1~4 | s | 5 |
| T2 | 1~4 | s | 1 |
| Propagation condition | 1~4 |  | AWGN |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~4 |  | AWGN |

Table A.5.6.3.1.2-1: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~4 | dBm/15kHz | TBD |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | TBD |  |  |  |
|  | 3,4 |  | TBD |  |  |  |
|  | 1~4 | dB | TBD | TBD | -Infinity | TBD |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | TBD | TBD | -Infinity | TBD |
|  | 3,4 |  | TBD | TBD | -Infinity | TBD |
| Io Note3 | 1,2 | dBm/95.04MHz | TBD | TBD | TBD | TBD |
|  | 3,4 |  | TBD | TBD | TBD | TBD |
|  | 1~4 | dB | TBD | TBD | -Infinity | TBD |

A.5.6.3.1.3	Test Requirements

The UE shall send L1-RSRP report every 640 slots. After 480ms plus 640 slots from the beginning of time period T2, UE shall send L1-RSRP report including the results for both SSB#0 and SSB#1 while meeting the accuracy requirements defined in 10.1.20.1. The reported L1-RSRP value shall include the Rx antenna gain in the range of [0-17]dB.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used

Editor’s Note: to be added based on A.5.6.3.1.

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used

A.5.6.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in Table A.5.6.3.3.1-1.

Table A.5.6.3.3.1-1: Applicable NR configurations for FR2 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz CSI-RS SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz CSI-RS SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

A.5.6.3.1.2	Test parameters

There are two cells in the test, E-UTRAN PCell (Cell 1) and FR1 PSCell (Cell 2). The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 are given in Table A.5.6.3.3.2-1 and Table A.5.6.3.3.2-2 below.

In CSI measurement configuratation, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of two successive time periods, with time duration of T1, and T2 respectively. At 20ms from the beginning of T2, UE is triggered to measure on the aperiodic CSI-RS resource set containing two resources, and to report based on the reporting configuration in Table A.5.6.3.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.5.6.3.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~2 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~2 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~2 |  | CCR.3.1 TDD |
| SSB configuration | 1~2 |  | SSB.1 FR2 |
| CSI-RS configuration | 1~2 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | Off |
| reportConfigType | 1~2 |  | aperiodic |
| reportQuantity | 1~2 |  | cri-RSRP |
| Number of reported RS | 1~2 |  | 2 |
| qcl-Info | 1~2 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~2 |  | TBD |
| Propagation condition | 1~2 |  | AWGN |
| T1 | 1~2 | s | 5 |
| T2 | 1~2 | s | 1 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |

Table A.5.6.3.1.2-1: CSI-RS specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~2 | dBm/15kHz | TBD |  |  |  |
| Note2 | 1~2 | dBm/SSB SCS | TBD |  |  |  |
|  | 1~2 | dB | TBD | TBD | -Infinity | TBD |
| CSI-RS RSRP Note3 | 1~2 | dBm/SSB SCS | TBD | TBD | -Infinity | TBD |
| Io Note3 | 1~2 | dBm/95.04MHz | TBD | TBD | TBD | TBD |
|  | 1~2 | dB | TBD | TBD | -Infinity | TBD |

A.5.6.3.1.3	Test Requirements

The UE shall send L1-RSRP report at slot [TBD] from the beginning of T2. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the accuracy requirements defined in 10.1.20.1. The reported L1-RSRP value shall include the Rx antenna gain in the range of [0-17]dB.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used

Editor’s Note: to be added based on A.5.6.3.3.

<End of Change 1>

<Start of Change 2>

A.7.6.3	L1-RSRP measurement for beam reporting

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used

Editor’s Note: to be added based on A.5.6.3.1.

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used

Editor’s Note: to be added based on A.7.6.3.1.

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used

Editor’s Note: to be added based on A.5.6.3.3.

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used

Editor’s Note: to be added based on A.7.6.3.3.

<End of Change 2>