3GPP TSG-RAN WG4 Meeting #90bis	R4-1903892

Xi’an, China, 8 – 12 April 2019

Agenda Item:	6.11.2.2.1

Source:	Ericsson

Title:	Discussion on L1-RSRP accuracy requirements

Document for:	Discussion

# 1	Introduction

RAN4#90 agreed with the way forward on L1-RSRP measurement accuracy [1]:

- For measurement accuracy, - The accuracy is defined based on the single sample. - RAN4 is to decide the minimum BW used for deriving the CSI-RS based L1-RSRP measurement accuacy based on the simulation results. - Interested companies are encouraged to provide L1-RSRP accuracy simulation results based on the simulation assumptions in this WF.

This contribution provides our L1-RSRP measurement simulation result and discusses L1-RSRP measurement accuracy requirements.

# 2	Simulation results

## 2.1	SSB based L1-RSRP measurement

Table 1 shows the parameters for the SSB based L1-RSRP measurement agreed in [1]. Figure 1 shows the simulation results of SSB based L1-RSRP measurement error, and Table 2 is the summary of results.

Table 1	Simulation parameters for the SSB based L1-RSRP measurement.

| Parameters | Value |
| --- | --- |
| SCS | 15kHz, 30kHz, 120kHz |
| Side condition (SNR) | -3dB |
| Number of samples (N) | 1 (single sample measurement) |
| Propagation channel | AWGN |

Figure 1	SSB based L1-RSRP measurement error.

Table 2	Summary of SSB based L1-RSRP measurement error

|  | 5%-ile | 95%-ile |
| --- | --- | --- |
| SCS=15kHz | -1.2 | 0.99 |
| SCS=30kHz | -0.8 | 0.72 |
| SCS=120kHz | -0.35 | 0.34 |

It is observed from the simulation result that the estimation error is within +/-1.2dB with 90% probability (from 5%-ile to 95%-ile) for both FR1/FR2. If we assume the L1-RSRP measurement error distribution follows the normal distribution (i.e., σ =1.0), +/-1.2dB corresponds to +/-1.64σ (90%). If we consider +/-3.29σ as specified in TS38.133 A.2.1.2 (99.9%), the relative L1-RSRP accuracy is within +/-2.5dB for 99.9%. Considering the implementation margin as 0.5dB, we propose to set the relative L1-RSRP accuracy requirements for FR1 to +/-3.0dB.

Proposal 1: Set SSB based relative L1-RSRP accuracy to +/-3.0dB under normal condition for FR1.

For the SSB based absolute L1-RSRP accuracy, we think we can reuse the same approach as SS-RSRP even with the side condition is -6dB, that is,

- Normal condition with Io=-70dBm/BW is derived by adding 1.5dB to the relative L1-RSRP accuracy in normal condition.

- Extreme condition with Io=-70dBm/BW is derived by adding 4.5dB to the absolute L1-RSRP accuracy in normal condition.

- Normal condition with Io=-50dBm/BW is derived by adding 3.5dB to the absolute L1-RSRP accuracy in normal condition with Io=-70dB/BW.

- Extreme condition with Io=-50dB/BW is derived by adding 2dB to the absolute L1-RSRP accuracy in extreme condition with Io=-70dB/BW.

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 dB | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | -6 dB | ANR_FDD_FR1_A, NR_TDD_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_G, NR_FDD_FR1_H, | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

Table 10.1.2.1.2-1: SS-RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 2 | 3 | -3 dB | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3 | 3 | -6 dB | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

With this methodology the SSB based absolute L1-RSRP accuracy requirements for FR1 are proposes as follows.

Proposal 2: SSB based absolute L1-RSRP accuracy requirements for FR1 are proposes as follows:

|  | Normal condition | Extreme condition |
| --- | --- | --- |
| Maximum Io = -50dB/BW | +/-4.5dB | +/-9.0dB |
| Maximum Io = -70dB/BW | +/-8.0dB | +/-11.0dB |

## 2.2	CSI-RS based L1-RSRP measurement

Table 3 shows the parameters for the CSI-RS based L1-RSRP measurement based on [1]. Figure 2 shows the CSI-RS based L1-RSRP measurement error based on the number of PRBs, and Table 4 summarizes the results.

Table 3	Simulation parameters for the CSI-RS based L1-RSRP measurement.

| Parameters | Value |
| --- | --- |
| SCS | 15kHz, 30kHz, 120kHz |
| Side condition (SNR) | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1 (single sample measurement) |
| Number of PRB | 24, 48, 96 |
| Propagation condition | AWGN |

Figure 2	CSI-RS based L1-RSRP measurement error.

Table 4	Summary of CSI-RS based L1-RSRP measurement error

|  | PRB | 5%-ile | 95%-ile |
| --- | --- | --- | --- |
| SCS=15kHz | 24 | -1.25 | 1.12 |
|  | 48 | -0.77 | 0.73 |
|  | 96 | -0.57 | 0.54 |
| SCS=30kHz | 24 | -0.79 | 0.78 |
|  | 48 | -0.54 | 0.55 |
|  | 96 | -0.36 | 0.41 |
| SCS=120kHz | 24 | -0.37 | 0.44 |
|  | 48 | -0.24 | 0.28 |
|  | 96 | -0.15 | 0.23 |

It is observed from the simulation result that the estimation error is within +/-1.25dB for 24PRB with 90% probability for FR1. Similar to SSB based L1-RSRP measurement accuracy, if we assume the L1-RSRP measurement error distribution follows the normal distribution, the relative L1-RSRP accuracy is within +/-2.5dB for 99.9%. Considering the implementation margin as 0.5dB, we propose to set the relative L1-RSRP accuracy requirements with 24PRB for FR1 to +/-3.0dB.

Proposal 3: Set the CSI-RS based relative L1-RSRP accuracy requirements under normal condition to +/-3.0dB for FR1 (24PRB).

Regarding the CSI-RS based absolute L1-RSRP accuracy, we propose to reuse the same methodology as SSB based L1-RSRP requirements above.

Proposal 4: CSI-RS based absolute L1-RSRP accuracy requirements for FR1 are proposes as follows:

|  | Normal condition | Extreme condition |
| --- | --- | --- |
| Maximum Io = -50dB/BW | +/-4.5dB | +/-9.0dB |
| Maximum Io = -70dB/BW | +/-8.0dB | +/-11.0dB |

If we look 96PRB case, it is observed that the estimation error is within +/-0.6dB with 90% probability. Based on that we can set the relative L1-RSRP accuracy requirements with 96PRB for FR1 to +/-2.0dB.

Observation: With 96PRB, the relative L1-RSRP accuracy is improved from +/-3.0dB to +/-2.0dB.

Proposal 5: RAN4 discuss whether CSI-RS based L1-RSPR accuracy requirements with 96 PRB is introduced or not.

# 3	Summary

Proposal 1: Set SSB based relative L1-RSRP accuracy to +/-3.0dB under normal condition for FR1.

Proposal 2: SSB based absolute L1-RSRP accuracy requirements for FR1 are proposes as follows:

|  | Normal condition | Extreme condition |
| --- | --- | --- |
| Maximum Io = -50dB/BW | +/-4.5dB | +/-9.0dB |
| Maximum Io = -70dB/BW | +/-8.0dB | +/-11.0dB |

Proposal 3: Set the CSI-RS based relative L1-RSRP accuracy requirements under normal condition to +/-3.0dB for FR1 (24PRB).

Proposal 4: CSI-RS based absolute L1-RSRP accuracy requirements for FR1 are proposes as follows:

|  | Normal condition | Extreme condition |
| --- | --- | --- |
| Maximum Io = -50dB/BW | +/-4.5dB | +/-9.0dB |
| Maximum Io = -70dB/BW | +/-8.0dB | +/-11.0dB |

Observation: With 96PRB, the relative L1-RSRP accuracy is improved from +/-3.0dB to +/-2.0dB.

Proposal 5: RAN4 discuss whether CSI-RS based L1-RSPR accuracy requirements with 96 PRB is introduced or not.

# References

- R4-1902565, “Way forward on L1 RSRP accuracy”, Ericsson, MediaTek, Nokia.