3GPP TSG-RAN WG4 Meeting #90b                 	 R4-1903907

Xi’an, China, April 8th– 12th, 2019

Agenda item:            6.10.7.5.2

Source:	MediaTek Inc.

Title:	Remaining Issues on BWP Switch Interruption

Document for:          Discussion

1	Introduction

In this paper, we discuss the remaining issues for RRC-based BWP switch interruption, including

2	RRC-based BWP switch

In our understanding, the interruptions for BWP switch happens during the RRC procedure delay, which is TRRC_delay + TBWPswitchDelayRRC according to [1].

According to TS 38.331, the overall RRC procedure delay is extended due to BWP switch. This means UE has to be ready for the reception of uplink grant on the new BWP. In other words, UE’s behavior remains undefined during the whole period of TRRC_delay + TBWPswitchDelayRRC.

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation). In case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], FFS section. Figure 12.1-1: Illustration of RRC procedure delay Table 12.1-1: UE performance requirements for RRC procedures for UEs Procedure title:; Network -> UE; UE -> Network; Value [ms]; Notes. RRC Connection Control Procedures. RRC reconfiguration; RRCReconfiguration; RRCReconfigurationComplete; 10. RRC reconfiguration (scell addition/release); RRCReconfiguration; RRCReconfigurationComplete; 16. RRC reconfiguration (SCG establishment/ modification/ release); RRCReconfiguration; RRCReconfigurationComplete; 16. RRC setup; RRCSetup; RRCSetupComplete; 10. RRC Release; RRCRelease; NA. RRC re-establishment; RRCReestablishment; RRCReestablishmentComplete; FFS. RRC resume; RRCResume; RRCResumeComplete; 10. RRC resume (scell addition/release); RRCResume; RRCResumeComplete; 16. Initial security activation; SecurityModeCommand; SecurityModeComplete/SecurityModeFailure; 5. Initial security activation + RRC reconfiguration; SecurityModeCommand, RRCReconfiguration; RRCReconfigurationComplete; FFS; The two DL messages are transmitted in the same TTI. Other procedures. UE assistance information; UEAssistanceInformation; NA. UE capability transfer; UECapabilityEnquiry; UECapabilityInformation; FFS. Counter check; CounterCheck; CounterCheckResponse; 5.

Since UE behavior during RRC processing delay is undefined, there is no need to further specify interruption requirements for RRC-based BWP switch.

Proposal 1: Do not specify interruption requirements for RRC-based BWP switch.

In case not defining any interruption requirement may be easily to be mis-understood that RRC-based BWP switch on 1 CC will never interrupt other CC. We think it may be also fine to capture some high-level description on the spec like “During the whole RRC procedure delay TRRC_delay + TBWPswitchDelayRRC, UE is allowed to cause interruption to other active serving cells due to switching its active BWP of a serving cell. There is no requirement for the starting time and length for this interruption.”

Proposal 2: Add a sentence in the spec: “During the whole RRC procedure delay TRRC_delay + TBWPswitchDelayRRC, UE is allowed to cause interruption to other active serving cells due to switching its active BWP of a serving cell. There is no requirement for the starting time and length for this interruption.”

3	Conclusions

In the contribution, we provide our view on the remaining issue RRC-based BWP switching interruption requirements. We have the following proposals:

Proposal 1: Do not specify interruption requirements for RRC-based BWP switch.

Proposal 2: Add a sentence in the spec: “During the whole RRC procedure delay TRRC_delay + TBWPswitchDelayRRC, UE is allowed to cause interruption to other active serving cells due to switching its active BWP of a serving cell. There is no requirement for the starting time and length for this interruption.”

4	Reference

[1] R4-1816689,	“WF on RRC and MAC based BWP switching”, Nokia, Nokia Shanghai Bell, Mediatek Inc.