3GPP TSG-RAN4#90                                                                                     		 R4-1903981

Xi’an, China, 8th– 12th April, 2019

Agenda Item:	6.12.1.2.1

Source: 	Ericsson

Title: 	Simulation results for NR UE PDSCH demodulation tests

Document for:	Discussion

# Introduction

In this contribution we provide the simulation results for alignment purposes for both FR1 and FR2, based on the agreed simulation assumption from last meeting as listed in the following tables.

Table 1 List of simulation cases for FR1 FDD

| Case Number | CHBW/ SCS | MIMO | PDSCH type and MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK, MCS 4 | 1 Layer | 52 | TDL-B 100ns, 400 Hz | MMSE-IRC | 4 |
| 2 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK, MCS 4 | 1 Layer | 6 (in the middle of CBW) | TDL-C 300ns, 100 Hz | MMSE-IRC | 4 |
| 3 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 256QAM, MCS 24 | 1 Layer | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| 4 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 16QAM, MCS 13 | 2 Layers | 52 | TDL-C 300ns, 100 Hz | MMSE-IRC | 4 |
| 5 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM, MCS 19 | 2 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| 6 | 10MHz/15kHz | 4Tx 4Rx ULA Low | Type A, 16QAM, MCS 13 | 3 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| 7 | 10MHz/15kHz | 4Tx 4Rx ULA Low | Type A, 16QAM, MCS 13 | 4 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| 8 | 10MHz/15kHz | 2Tx 2Rx ULA Med | Type A, 16QAM, MCS 13 | 2 Layers | 52 | TDL-A 30ns, 10Hz | R-ML | 4 |
| 9 | 10MHz/15kHz | 4Tx 4RX ULA Med A | Type A, 16QAM, MCS 13 | 3 Layers | 52 | TDL-A 30ns, 10Hz | R-ML | 4 |
| 10 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 16QAM, MCS 13 | 1 Layer | 52 | TDL-C 300ns, 100 Hz | MMSE-IRC | 8 |
| 11 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type B, QPSK, MCS 4 | 1 Layer | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| 12 (LTE-NR test #1) | 10MHz/15kHz | 4Tx 2Rx ULA Low 4Tx 4Rx ULA Low | Type A, QPSK, MCS 4 | 1 Layer | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| 13 (LTE-NR test #2) | 10MHz/15kHz | 4Tx 2Rx ULA Low 4Tx 4Rx ULA Low | Type A, QPSK, MCS 4 | 1 Layer | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| 14 | 20MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM, MCS 19 | 2 Layers | 51 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |

Table 2 List of simulation cases for FR1 TDD

| Case Number | CHBW/ SCS | MIMO | PDSCH type and MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | TDD Configuration | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 106 | TDL-B 100ns, 400 Hz | MMSE-IRC | 1 | 8 |
| 2 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 6 (in the middle of CBW) | TDL-C 300ns, 100 Hz | MMSE-IRC | 1 | 8 |
| 3 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 256QAM MCS 24 | 1 Layer | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| 4 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 106 | TDL-C 300ns, 100 Hz | MMSE-IRC | 1 | 8 |
| 5a | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM MCS 19 | 2 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| 5b | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM MCS 19 | 2 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 16 |
| 6 | 40MHz/30kHz | 4Tx 4Rx ULA Low | Type A, 16QAM MCS 13 | 3 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| 7 | 40MHz/30kHz | 4Tx 4Rx ULA Low | Type A, 16QAM MCS 13 | 4 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| 8 | 40MHz/30kHz | 2Tx 2Rx ULA Med | Type A, 16QAM MCS 13 | 2 Layers | 106 | TDL-A 30ns, 10Hz | R-ML | 1 | 8 |
| 9 | 40MHz/30kHz | 4Tx 4RX ULA Med A | Type A, 16QAM MCS 13 | 3 Layers | 106 | TDL-A 30ns, 10Hz | R-ML | 1 | 8 |
| 10 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 16QAM MCS 13 | 1 Layer | 106 | TDL-C 300ns, 100 Hz | MMSE-IRC | 1 | 16 |
| 11 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 2 | 8 |
| 12 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 3 | 8 |
| 13 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type B, QPSK, MCS 4 | 1 Layer | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| 14 | 20MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM, MCS 19 | 2 Layers | 51 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |

Table 3 List of simulation cases for FR2

| Case Number | CHBW/ SCS | MIMO | PDSCH type and MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | TDD Configuration | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 66 | TDL-C 60ns, 300Hz | MMSE-IRC | 1 | 8 |
| 1a (TRS 1 slot, DMRS 1 add) | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 66 | TDL-C 60ns, 300Hz | MMSE-IRC | 1 | 8 |
| 1b (TRS 1 slot, DMRS 2 add) | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 66 | TDL-C 60ns, 300Hz | MMSE-IRC | 1 | 8 |
| 2 | 100MHz/120kHz | 2Tx 2Rx XPL Med A | Type A, 64QAM MCS 18 | 1 Layer | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| 3 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, QPSK MCS 4 | 2 Layers | 6 (in the middle of CBW) | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |
| 4 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| 5 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 64QAM MCS 17 | 2 Layers | 66 | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |
| 6 | 100MHz/120kHz | 2Tx 2Rx ULA Med | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 75Hz | R-ML | 2 | 10 |
| 7 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 1 Layer | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 16 |
| 8 | 50MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 32 | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |
| 9 | 200MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 132 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| 10 | 50MHz/60kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |

Simulation results

Figure 1 Simulation results for FR1 FDD tests

Figure 2 Simulation results for FR1 TDD tests

Figure 3 Simulation results for FR2 tests

Also the results are attached below.

# Conclusions

In this contribution, we provide results based on the agreements made in previous meeting. We propose to include our results for further alignment to adjust the existing PDSCH UE demodulation tests requirements.

# References

[1] R4-1902403, “Way forward on NR UE PDSCH demodulation requirements and General aspects”, RAN4#90, Intel Corporation

[2] R4-19xxxxx, “Simulation assumptions for NR PDSCH demodulation performance requirements”, RAN4#90bis, Intel Corporation