3GPP TSG-RAN4  #90bis	R4-1904090

# Xi’an, China, 8 - 12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.810 | CR | - | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 38.810 on FR2 RRM test methods |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The details of how to control the SINR for 2AoA case is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The details of how to control the SINR for 2AoA case are proposed to include in TR38.810: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The FR2 RRM test methods procedures for 2AoA case are incomplete and cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.4.2, Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

=================================Start of change=================================

##### 6.2.1.4.2	Radiated requirements Reference point and Testing directions

For RRM baseline measurement setup based on DFF and IFF, the Reference Point is located at the centre of the QZ. From the UE perspective the reference point is the input of UE antenna array.

Calibration of power level and relative power level test parameters and RRM metrics (at the required AoAs) is required and shall be provided by the test system.

Appropriate timing and relative timing test parameters and RRM metrics (at the required AoAs) shall be provided by the test system within declared uncertainties.

The Test cases in core specification TS 38.133 [17] will be specified at the Reference point, according to the following principles:

-	Specify absolute Noc level at the Reference point per angle of arrival (AoA)

-	Noc level may have different value according to operating band and UE power class

-	Specify SNR at the Reference point per angle of arrival (AoA)

-	SNR is a test-specific value

-	The angle(s) of arrival (AoA(s)) will be specified in each test case.

Figure 6.2.1.4.2-1: DL SNR reference point for RRM testing methodology

The following Scenarios for RRM requirements have been identified and can be supported by the NR RRM Test Methods:

Scenario 1: RRM requirement with single Angle of Arrival (1 AoA) with signal coming from RX beam peak direction.

Scenario 2: RRM requirement with single Angle of Arrival (1 AoA) with signal coming from RX non-beam peak direction.

-	The test can be performed in any single direction which is covered by Nth percentile EIS spherical coverage of the DUT

-	Value of N according to power class, as defined in TS 38.101-2 [16] clause 7.3.4.

Scenario 3: RRM requirement with two Angle of Arrivals (2 AoAs).

-	Test directions:

-	Both signals come from the directions covered by the N% percentile EIS spherical coverage of the DUT

-	The angle between two probes should match the relative probe spacing of 30, 60, 90, 120, 150 deg and UE is in the directions in which the UE RRM test cases can be performed.

-	Value of N according to power class, as defined in TS 38.101-2 [16] clause 7.3.4.

-	Case 1: TDM transmissions from 2 probes (i.e. each probe transmits both signal and/or artificial noise in TDM manner)

-	Case 2: Simultaneous transmission of signals from 2 probes

-	The detailed on how to control the SINR for Case 1 and Case 2 is provided in Annex H.

The following Types of RRM requirements have been identified and can be supported by the NR RRM Test Methods:

-	Type 1:

-	RRM requirements defined under the assumption that the UE is using “Fine” UE RX beams.

-	“Fine” UE RX beams are the beams used by the UE to perform PDSCH reception and used to define UE RF requirements (e.g. EIS, EIS spherical coverage)

-	Type 2:

-	Requirements defined under the assumption that the UE is using “Rough” UE RX beams.

-	“Rough” UE RX beams are the beams which the UE is using for RRM measurements (e.g. for SSB measurements)

The following Modes for useful signals (S) and noise signals (N) Configuration have been identified and can be supported by the NR RRM Test Methods:

-	Mode 1:

-	Test system transmits useful signals (S) and noise signals (N) to emulate target SNR condition

-	Mode 2:

-	Test system transmits only useful signals (S).

For selecting the testing direction (AoA to test the requirement) fulfilling certain preconditions, two methods are feasible from the perspective of RRM baseline measurement setup, as follows:

-	Method 1: Run a pretest in the RRM baseline measurement system to identify all the directions (with a given spatial granularity) at which the UE fulfils a given precondition (e.g. spherical coverage EIS). The testing directions are then chosen out of the valid directions, following a given rule. The precondition to be fulfilled, and the rule how to select the testing direction out of the valid directions, are specified in the test description.

-	Method 2: For each given potential direction, test first a given precondition (e.g. minimum TP for a given power), which validates the direction as valid for testing or not. If the direction is valid, test the requirement, if not, jump to the next potential direction following a given rule. The rule how to select the potential directions and the precondition to validate them as testing direction, are specified in the test description.

=================================End of change=================================

=================================Start of change=================================

Annex H:
SINR control for Scenario 3: RRM requirement with two Angle of Arrivals (2 AoAs)

# H.1	Case 1: TDM transmissions from 2 probes

For Scenario 3 with 2AoA for both Mode 1 and Mode 2, the methods are same as the Scenario 1 and Scenario 2 with 1AoA.

# H.2	Case 2: Simultaneous transmission of signals from 2 probes

# H.2.1	Mode 1:

For Mode 1, the SINR equation at baseband is given as follows:

SINR1BB=S1*G1*LossS2+NG2*Loss+N*G1*Loss+Noise_floor

Where S1 and S2 are signal level for probe 1 and probe 2 respectively. G1 and G2 are the antenna gain for probe 1 and probe 2 respectively. Loss is the pathloss between probe 1&2 and DUT. Noise_floor is the total noise at DUT baseband receiver. N is the artificial noise at the reference point for probe 1&2 (Artificial noise level is identical for probe 1 and probe 2).

Since the wanted noise is set 6dB above UE thermal noise, therefore the total noise of baseband Noise_floor can be ignored when calculating the SINR. And the SINR can be rewritten as:

SINR1BB≈S1*G1*LossS2+NG2*Loss+N*G1*Loss=1S2S1+1SNR1*G2G1+1SNR1

Where SNR1 is the SNR from probe 1, and we can observer that the key parameters to control SINR at baseband is G2/G1.

However, it is difficult to obtain the exact G2/G1 value since it depends on UE implementation. But we can still calculate the lower bound of maximum feasible SINR once the assumptions on maximum antenna gains difference G2/G1 is determined.

The maximum gain difference is the case in which interference comes from peak direction meanwhile the signal comes from the direction at the Nth percentile EIS values. Therefore, the following assumptions on maximum gain difference for fine/rough beam types are given as:

- Fine beam: the maximum G2/G1 is X, where X is derived based on EIS spherical coverage requirement (i.e. difference between the peak EIS and Nth percentile EIS values) based on TS 38.101-2 [16]. Value of N according to power class, as defined in TS 38.101-2 [16] clause 7.3.4.

- Rough beam: the maximum G2/G1 is X+Z-Y, where value Z characterizes the antenna gain difference between “fine” and “rough” RX beams within Nth percentile EIS directions, and value Y characterizes the antenna gain difference between the fine and rough beams in the ‘fine’ RX beam peak direction.

## H.2.1	Mode 2:

For Mode 2, the SINR at baseband is given as follows:

SINR1BB=S1*G1*LossS2*G2*Loss+Noise_floor

If we set the signal level from interfering probe at reference point is 6dB higher than UE thermal noise, then the approximate SINR can be rewrite as:

SINR1≈S1S2*G2/G1

Same as Mode 1, the following assumptions on maximum gain difference for fine/rough beam types are given as:

- Fine beam: the maximum G2/G1 is X, where X is derived based on EIS spherical coverage requirement (i.e. difference between the peak EIS and Nth percentile EIS values) based on TS 38.101-2 [16]. Value of N according to power class, as defined in TS 38.101-2 [16] clause 7.3.4.

- Rough beam: the maximum G2/G1 is X+Z-Y, where value Z characterizes the antenna gain difference between “fine” and “rough” RX beams within Nth percentile EIS directions, and value Y characterizes the antenna gain difference between the fine and rough beams in the ‘fine’ RX beam peak direction.

Annex I:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-04 | R4#82bis | R4-1702939 |  |  |  | Skeleton of TR38.xyz on test methods for NR | 0.0.1 |
| 2017-04 | R4#82bis | R4-1704361 |  |  |  | Introducing NR SI outcome on testability | 0.0.2 |
| 2017-08 | R4#84 | R4-1708997 |  |  |  | Corrected title, enabling measurements of NSA UEs, OTA requirement to test mapping, and RRM baseline setup | 0.0.3 |
| 2017-09 | NR AH#3 | R4-1709525 |  |  |  | Corrected RRM baseline setup, introduced MU budget for RF baseline setup | 0.0.4 |
| 2017-10 | R4#84bis | R4-1710455 |  |  |  | Added DUT positioning guidelines | 0.0.5 |
| 2017-11 | R4#85 | R4-1713810 |  |  |  | Introduced description of the test interface, quality of quiet zone characterization procedures, and descriptions of measurement uncertainty elements; corrected the RRM baseline setup | 0.0.6 |
| 2017-11 | R4#85 | R4-1713812 |  |  |  | Preliminary measurement uncertainty values | 0.1.0 |
| 2017-12 | RP#78 | RP-172403 |  |  |  | Submitted for information to RAN | 1.0.0 |
| 2018-01 | R4 AH #1801 | R4-1800341 |  |  |  | R4-1800779 preliminary MU assessment for EIS R4-1801261 applicability criteria for the RF baseline and the MU assessment | 1.0.1 |
| 2018-02 | R4#86 | R4-1892187 |  |  |  | R4-1803554 TP to TR on TR structure for test method applicability R4-1802909 TP to TR 38.810 - Adding Note to EIRP MU assessment for baseline setup R4-1803414 TP to TR 38.810 - Adding AUT reference antenna masks to Annex D D.2.1 R4-1803537 TP to TR 38.810 - Characterization of Quiet Zone at mm-wave NR frequencies for Combined axis Positioner R4-1803419 TP to TR 38.810 on testing and calibration aspects for baseline system R4-1803535 TP to TR38.810 v1.0.1 on applicability of baseline setup R4-1803532 TP for TR 38.810 v1.0.0 on RRM baseline setup R4-1801808 TP to 38.810 on UE measurements for demodulation test setup in FR2 R4-1803531 TP for TR 38.810 v1.0.1 on Demodulation Baseline setup R4-1803416 Proposals for Equivalence criteria revision Removed instances of “TRS” from the text, applied stylistic corrections to clause headings, renamed the three “Summary of measurement uncertainty and test tolerances” clauses in the RF, RRM, and demodulation setups to “Summary of initial uncertainty assessment” | 1.1.0 |
| 2018-03 | RP#79 | RP-180181 |  |  |  | R4-1803572 TP to TR 38.810 on Measurement Uncertainty of Mismatch R4-1803571 TP to TR 38.810 - Adding MU format and MU assessment for CATR R4-1803574 TP to 38.810: Rationale behind CATR as NR RF baseline R4-1803575 TP to 38.810 addition of CATR for arbitrary device size Updated visual quality of the embedded equations Aligned clause headings with terminology agreed for permitted methods | 2.0.0 |
| 2018-04 | R4#86bis | R4-1804135 |  |  |  | R4-1805062 TP to TR 38.810 Adding Appendix F - NFTF Rationale R4-1805439 Minor corrections to 38.810 Combined axes system sections R4-1805893 TP to TR 38.810 on editorial aspects R4-1805894 TP to TR 38.810 – Full package for Near Field Test Range (NFTF) Corrected clause numbering on section 5.2.3.3 | 2.1.0 |
| 2018-07 | R4 AH1807 | R4-1808774 |  |  |  | R4-1808409 TP to TR38.810 to reflect RAN4#86bis agreements and editorial changes R4-1808412 TP to TR on OTA testing with battery R4-1806381 TP to TR 38.810 on procedure to decide applicable test methods for mmWave UE R4-1806731 TP to 38.810: Clarifications related to DUT Antenna Configurations R4-1808413 TP on RF test procedures for EVM and blocking R4-1808402 TP to TR 38.810 on open items for RRM R4-1808403 TP for TR 38.810 v2.1.0 on Demodulation and CSI Baseline setup R4-1808405 TP to TR 38.810 on Propagation model definition R4-1808518 TP to TR 38.810 on channel model generation methodology R4-1808407 TP on path delay grid for channel models Editorial corrections Updated “Definitions, symbols and abbreviations” | 2.2.0 |
| 2018-08 | R4#88 | R4-1809982 |  |  |  | R4-1809508 MU factors contributing to RRM parameters and metrics R4-1809509 TP for TR 38.810 on definition and feasibility of parameters and metrics for UE RRM BLS R4-1809510 TP to TR 38.810 on open items for UE Demodulation testing methodology R4-1809511 MU factors contributing to DL SNR accuracy and range R4-1809512 TP to TR 38.810 on demod measurement set-up applicability R4-1809222 TP to TR 38.810 on missing aspects for channel model option 1 R4-1809513 TP to TR 38.810 on Propagation model definition R4-1809515 TP to TR 38.810 on Channel model naming R4-1809586 TP to TR 38.810 on option 2 channel model generation methodology for FR2: parameters and procedure updates | 2.3.0 |
| 2018-08 | R4#88 | R4-1811893 |  |  |  | R4-1811096 TP to TR38.810 on DUT Repositioning R4-1811777 MU factors contributing to DL SNR for demodulation and CSI R4-1811778 MU factors contributing to DL SNR in RRM Test cases R4-1811779 TP to TR 38.810 on editorial corrections R4-1811780 TP to TR 38.810 on the remaining details of channel modelling methodology R4-1811782 TP to TR 38.810 on the remaining details of NR RRM testing methodology R4-1811833 Introduction of other methods for RRM baseline measurement system R4-1811835 On multiple antenna port capability for RRM baseline measurement system R4-1811836 TP to TR 38.810 on the remaining details of NR FR2 UE Demodulation and CSI testing methodology R4-1811891 TP to 38.810 on measurement grids of EIRP/EIS, TRP, Spherical Coverage R4-1810870 TP to 38.810: Update of blocking measurement procedure Editorial corrections | 2.4.0 |
| 2018-09 | RAN#81 | RP-181710 |  |  |  | Editorial corrections | 2.5.0 |
| 2018-09 | RAN#81 | RP-182064 |  |  |  | Editorial corrections - Fixed broken references in Clause 8.2.1.1 - Updated Tables 5.3-1 and 5.3-2 to align the terms | 2.6.0 |
| 2018-09 | RAN#81 | RP-182134 |  |  |  | Editorial corrections - Changed Release 15 to Release 16 - Changed TR title to “NR; Study on test methods” | 2.6.1 |
| 2018-09 | RAN#81 |  |  |  |  | Approved by plenary – Rel-16 spec under change control | 16.0.0 |
| 2018-12 | RAN#82 | RP-182363 | 0003 | 1 | F | Comibned CR to TR 38.810 after RAN4 #88bis and RAN4 #89 | 16.1.0 |

=================================End of change=================================