3GPP TSG-RAN WG4 Meeting #90bis	R4-1904300

# Xi’an, China, 8th – 12th Apr, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add test cases for scheduling restriction during SSB based Beam Failure Detection and Link recovery procedures in FR1 in SA (A.6.5.5.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To Add test cases for scheduling restriction due to SSB based Beam Failure Detection and Link recovery procedures in FR1 in SA (A.6.5.5.5) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new test cases for scheduling restriction due to SSB based Beam Failure Detection and Link recovery procedures in FR1 in SA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for scheduling restriction due to SSB based Beam Failure Detection and Link recovery procedures in FR1 are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

#### A.6.5.5.5	scheduling availability restriction during Beam Failure Detection and Link Recovery for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode

##### A.6.5.5.5.1	Test Purpose and Environment

The purpose is to test scheduling availability restrictions when the UE is performing beam failure detection or when the UE is performing L1-RSRP measurement for candidate beam detection, when no DRX is used. This test will verify the scheduling availability restriction requirements in clause 8.5.7 and 8.5.8.

The test parameters are given in Tables A.6.5.5.5.1-1, A.6.5.5.5.1-2 and A.6.5.5.5.1-3 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.5.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.6.5.5.5.1-1 additionally shows the variation of the downlink SNR of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. This test will focus on the scheduling availability during T1 and T2 (during beam failure detection) and during T4 and T5 (during candidate beam detection). In the test, DRX configuration is not enabled. Test 1 is to test the scheduling availability restriction of UE performing beam failure detection and candidate beam detection when SSB has the same SCS with PDSCH/PDCCH, Test 2 is to test the scheduling availability restriction of UE performing beam failure detection and candidate beam detection when SSB has different SCS than PDSCH/PDCCH. During the test the UE is scheduled to transmit continuously in UL.

Table A.6.5.5.5.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 3 | TDD duplex mode, 30kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.5.1-2: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 | Test 2 |  |
| Active PCell |  |  |  |  | Cell 1 | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 | 1 |  |
| Duplex mode |  | Config 1 |  |  | FDD | FDD |  |
|  |  | Config 2, 3 |  |  | TDD | TDD |  |
| TDD Configuration |  | Config 1 |  |  | Not Applicable | Not Applicable |  |
|  |  | Config 2 |  |  | TDDConf.1.1 | TDDConf.1.1 |  |
|  |  | Config 3 |  |  | TDDConf.1.2 | TDDConf.1.2 |  |
| CORESET Reference Channel |  | Config 1 |  |  | CR. 1.1 FDD | CR. 1.1 FDD |  |
|  |  | Config 2 |  |  | CR. 1.1 TDD | CR. 1.1 TDD |  |
|  |  | Config 3 |  |  | CR. 2.1 TDD | CR. 2.1 TDD |  |
| SSB Configuration |  | Config 1 |  |  | SSB.1 FR1 | SSB.2 FR1 |  |
|  |  | Config 2 |  |  | SSB.1 FR1 | SSB.2 FR1 |  |
|  |  | Config 3 |  |  | SSB.2 FR1 | SSB.1 FR1 |  |
| SMTC Configuration |  | Config 1, 2 |  |  | SMTC.1 | SMTC.1 |  |
|  |  | Config 3 |  |  | SMTC.1 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  |  | 15 KHz | 15 KHz |  |
|  |  | Config 3 |  |  | 30 KHz | 30 KHz |  |
| csi-RS-Index assigned as RLM RS |  |  |  |  | [0] | [0] |  |
| OCNG parameters |  |  |  |  | OP.1 | OP.1 |  |
| CP length |  |  |  |  | Normal | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 | 2 |  |
|  | Aggregation level |  |  | CCE | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 | 6 |  |
| DRX |  |  |  |  | OFF | OFF | DRX is not in use |
| Gap pattern ID |  |  |  |  | [N.A.] | [N.A.] | No measurement gap pattern is configured |
| ssb-Index |  |  |  |  | 2 | 2 | Number of SSB indexes used for beam failure detection |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | absent | When the field is absent, the UE applies the 10% |
| rsrp-ThresholdSSB |  |  |  |  | TBD | TBD | Threshold used for Qin_LR |
| powerControlOffsetSS |  |  |  |  | NA | NA | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | [n2] | [n2] | see TS 38.321 [7], section 5.17 |
| beamFailureDetectionTimer |  |  |  |  | [pbfd4] | [pbfd4] | see TS 38.321 [7], section 5.17 |
| ZP CSI-RS configuation |  |  |  |  | TBD | TBD |  |
| CSI-IM configuration |  |  |  |  | TBD | TBD |  |
| Periodic CSI reporting |  |  |  |  | PUCCH | PUCCH |  |
| CSI reporting periodicity |  |  | Config 1, 2 | slot | [5] | [5] |  |
|  |  |  | Config 3 |  | [10] | [10] |  |
| T1 |  |  |  | s | 1 | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 0.4 | 0.4 |  |
| T3 |  |  |  | s | [0.6] | [0.6] |  |
| D1 |  |  |  | s | [0.24] | [0.44] |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table A.6.5.5.5.1-3: Cell specific test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 and Test 2 |  |  |  |  | Test 1 and Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SSB of set q0 |  |  |  |  | SSB of set q1 |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta |  | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | Config 1 | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | Config 3 |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | Config 1 | dBm/15KHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 2 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 3 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.6.5.5.5.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.6.5.5.5.1-1: SNR variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.6.5.5.5.2	Test Requirements

The UE behaviour during time durations T1, T2, T4 and T5 shall be as follows:

There are no UE scheduling restrictions in Test 1 and the UE will transmit continuously in UL.

There are no scheduling restrictions in Test 2 when the UE supports simultaneousRxDataSSB-DiffNumerology [14] and the UE will transmit continuously in UL.

In Test 2 when the UE does not support simultaneousRxDataSSB-DiffNumerology [14],

- During the time duration T1 and T2, the UE should not transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured for beam failure detection.

- During T4 and T5, The UE should not transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured for L1-RSRP.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<< End of change 1>>