3GPP TSG-RAN WG4 Meeting #90-bis	R4-1904446

Xi’an, China, 8th to 12th April 2019

Source:	Ericsson, Telstra

Title:	TP for TR 37.716-21-11: 3A-7A_n5A, 3C-7A_n5A, 3A-7C_n5A and 3C-7C_n5A

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 to add 3A-7A_n5A, 3C-7A_n5A, 3A-7C_n5A and 3C-7C_n5A

# 2. Text Proposal

# ---Start of changes---

## 5.1.x	DC_3A-7A_n5A
DC_3C-7A_n5A 
DC_3A-7C_n5A 
DC_3C-7C_n5A

### 5.1.x.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n5 | CA_3-7 | n5 | DC_3_n5 DC_7_n5 |

### 5.1.x.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n5A DC_3C-7A_n5A DC_3A-7C_n5A DC_3C-7C_n5A | DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A | CA_3A-7A CA_3C-7A CA_3A-7C CA_3C-7C | n5 |

#### 5.1.x.3	Co-existence Studies

Co-existence analysis from DC_3_n5 shows that there is IMD2 and IMD3 impact from DC_3_n5 UL to Band 7 DL.

Co-existence analysis from DC_7_n5 shows that there is no impact from DC_7_n5 UL to Band 3 DL.

### 5.1.x.4		∆TIB and ∆RIB values

For DC_3-7_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_3-5-7, and are given in the tables below.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n5 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n5 | 0.3 |

Table 5.1.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n5 | 3 | 0 |
|  | 7 | 0 |
|  | n5 | 0 |

### 5.1.x.5	REFSENS requirements

MSD need to be defined in TS 38.101-3, see below.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | FDD | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 |  | IMD21 |
|  | n5 | 845 | 5 | 25 | 890 | N/A |  | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1]	RP-190379, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei